3GPP TSG-RAN WG2 Meeting #125-bis	R2-2403345

# Changsa, China, 15th– 19th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4719 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC CR to resolve issues on NES mode indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, NEC, CEWiT, Continental Automotive, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2024-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issues were first depicted in R2-2401361. In short: Reception of DCI 2_9 (NES mode = enabled) and the entry condition fulfillment for an associated cell are two independent events that may occur likely at different points in time. The present specification uses “and” to specify UE behaviour when both these events are fulfilled. However, it is not specified what happens only when one of these occur but not the other: - DCI 2_9 (NES mode = enabled) is received but no triggering cell is available. A UE may mistake this for immediate cell switch off and declare RLF; or it may continue as if DCI 2_9 was not received. - Triggering cell is available but DCI 2_9 (NES mode = enabled) is not yet received. By the time the DCI 2_9 would be received (if at all), the triggering cell may not remain valid as we do not have any requirement on the UE to ensure the validity of entry condition at the point of reception of DCI 2_9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced changes: - In case A: UE continues measurement/ evaluation until a triggering cell becomes available (or a RLF occurs). - In case B: It is clarified that entry condition should remain fulfilled for an associated cell at the time of receiving NES mode indication from lower layers to initiate handover execution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specified Release-18 Network energy savings for NR affects user experience adversely due to increased RLF or incorrect mobility execution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR 0811 TS 38.304 CR 0390 TS 38.306 CR 1056 TS 38.321 CR 1780 rev 1 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS 38.331 CR 4638 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

3>	if the associated condExecutionCondPSCell is configured:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; and

4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the nr-SCG within the received condRRCReconfig to be applicable cell;

3>	else:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

2>	else if the RRCReconfiguration within condRRCReconfig includes the secondaryCellGroup including the reconfigurationWithSync:

3>	if the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the received condRRCReconfig is not the PSCell; and

4>	if the condReconfigToAddMod for the condReconfigId does not include subsequentCondReconfig; or

4>	if there is no condReconfigToAddMod which includes subsequentCondReconfig for the PSCell; or

4>	if the condReconfigToAddMod for the condReconfigId includes subsequentCondReconfig and there is a condReconfigToAddMod which includes subsequentCondReconfig with a matching condReconfigId value in condExecutionCondToAddModList for the PSCell:

5>	consider the cell to be applicable cell;

2>	if condExecutionCondSCG is configured and condExecutionCond is not configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondSCG as a measId in the VarMeasConfig associated with the SCG measConfig;

2>	if the condExecutionCondPSCell is configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondPSCell as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	if condExecutionCond is configured and condExecutionCondSCG is not configured:

3>	if it is configured via SRB3 or configured within nr-SCG or within nr-SecondaryCellGroupConfig (specified in TS 36.331[10]) via SRB1:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the SCG measConfig;

3>	else:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	if subsequentCondReconfig is included in the entry in MCG VarConditionalReconfig associated with this condReconfigId:

3>	if the UE is in NR-DC:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondSCG as a measId in the VarMeasConfig associated with the SCG measConfig;

3>	else:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell associated to condReconfigId:

3>	if the condTriggerConfig is not configured with nesEvent:

4>	if the condEventId is associated with condEventT1, and if the entry condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4> if the condEventId is associated with condEventD1 or condEventD2, and if the entry conditions applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be fulfilled;

4>	if the measId for this event associated with the condReconfigId has been modified; or

4>	if the condEventId is associated with condEventT1, and if the leaving condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4>if the condEventId is associated with condEventD1 or condEventD2, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be not fulfilled;

3>	else:

4>	if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled post reception of NES mode indication or remains fulfilled at the time of receiving NES mode indication from lower layers for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	if NES mode indication is received from lower layers, latest one indicating that the NES-specific CHO execution condition of the PCell is enabled:

5>	consider the event associated to that measId to be fulfilled;

4>	else (i.e., either the NES mode indication is not yet received from lower layers or the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is not fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig):

5>	consider the event associated to that measId to be not fulfilled; and

5>	continue to perform measurement according to 5.5.3;

4>	if the measId for this event associated with the condReconfigId has been modified; or

4>	if NES mode indication is received from lower layers, latest one indicating that the NES-specific CHO execution condition of the PCell is disabled; or

4>	if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be not fulfilled;

2>	if condExecutionCondPSCell is not configured:

3>	if event(s) associated to all measId(s) within condTriggerConfig for the applicable cell are fulfilled:

4>	consider the applicable cell, associated to that condReconfigId, as a triggered cell;

4>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

2>	else:

3>	if event(s) associated to all measId(s), as indicated in the condExecutionCond and condExecutionCondPSCell, within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

4>	consider the target candidate PCell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered PCell;

4>	consider the target candidate PSCell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered PSCell;

4>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5.

2>	if one of the events associated to the measIds within condTriggerConfig for the applicable cell within the stored condRRCReconfig is not configured with nesEvent, and the other event associated to the measIds within condTriggerConfig for the applicable cell within the stored condRRCReconfig is configured with nesEvent, and at least one of them is fulfilled:

3>	consider the applicable cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

NOTE 1:	Up to 2 MeasId can be configured for each condReconfigId, if condExecutionCondPSCell is not configured. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

NOTE 2:	Void.

NOTE 3:	For CHO with candidate SCG(s), up to 2 MeasId can be configured for condExecutionCond and up to 2 MeasId can be configured for condExecutionCondPSCell for each condReconfigId.