3GPP TSG-RAN WG2 #125	R2-2403369

Changsha, China, 15th April – 19th April 2024

Agenda Item:	7.9.3

Source:	Ericsson

Title:	Discussion on Open Issues in 38.331

Document for:	Discussion, Decision

# 1 Introduction

As per the chairlady’s guidance on providing a list of open issues, the rapporteur has provided a list as shown below. We address a subset of the listed issues and provide our views.

| Open issue | Rapporteur view |
| --- | --- |
| 3. U2U common part |  |
| Issue 3.1: Editor’s Note: FFS whether a new L3 U2U Relay-specific indication is needed in SIB12. | This is covered by [Post125][417]. |
| Issue 3.3: Editor’s Note: Whether to differentiate U2U discovery and U2N discovery can be checked in maintenance. | This is covered by [Post125][417]. |

# 2 Discussion

## Issue 3.1

Editor’s Note: FFS whether a new L3 U2U Relay-specific indication is needed in SIB12.

This issue was discussed in the last meeting, and it was decided that an explicit indication would be added for L2 U2U relay in SIB12. For L3 U2U relays, we believe such an indication is not necessary because L3 U2U relays are based on higher layer mechanisms with very less impact on AS-procedures. However, given that in the post meeting email discussion 417, there is bit of support to introduce an indication, we would prefer an implicit indication where the presence of the U2U discovery configuration means that L3 U2U discovery is supported.

Use an implicit indication in SIB12 for L3 U2U discovery support using the presence/absence of the discovery configuration.

## Issue 3.3

Editor's Note: Whether to differentiate U2U discovery and U2N discovery can be checked in maintenance.

From the network’s perspective there is no difference, as the configuration provided in both cases is only to enable peer UE communication at least over the first hop for U2N relays and over both hops for U2U relays depending on the coverage scenario. In addition, based on the agreement below, the gNB will not be involved in any additional procedures beyond Rel-16 operation.

There are no additional procedures at the gNB beyond Rel-16 operation in the ID reporting/resource allocation procedures for an RRC_CONNECTED U2U relay/remote UE.  Some Rel-16 functionality may not be applicable to U2U (to be determined on a case by case basis).  FFS stage 3 impact to message formats (e.g., additional fields).

As a result, there will be no such differentiated handling at the network for the U2U relays as opposed to U2N relays or for that matter for Rel-16/17 SL operation. Hence, need not differentiate between the U2U and U2N discovery.

Need not differentiate in the SUI between U2U discovery and U2N discovery.

# 3 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use an implicit indication in SIB12 for L3 U2U discovery support using the presence/absence of the discovery configuration.

Proposal 2	Need not differentiate in the SUI between U2U discovery and U2N discovery.

# 4 References