3GPP TSG-RAN WG2 Meeting #125bis	R2-2403455

Changsha, China,  April 15th – 19th, 2024

Agenda item:	8.10.4

Source:	Samsung

Title:	SON/MDT enhancements for leftover topics from R18

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the leftover issues from R18.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

In this contribution, we discuss the UE reporting for these leftover topics from R18.

# Discussion

RACH optimization for SDT

In R18, UE reports the following information for the optimisation of SDT:

- Whether SDT is part of triggered feature combinations.

- Whether SDT is part of used feature combinations.

- Whether the SDT is failed.

Network normally would be able to perform most of the random access optimisations with this information. One information that may be additionally useful is the data volume at the time of triggering SDT.

Proposal 1: RAN2 to discuss reporting of actual data volume triggering SDT for RACH optimisation.

MHI Enhancement for SCG Deactivation/Activation

UE may report whether SCG deactivation was there for a PSCell in MHI. It could be interesting to know whether the SCG deactivation had any performance impacts. For e.g. the UE may log the duration when there was UL data in SCG Bearer, but the SCG was deactivated.

Proposal 2: RAN2 to discuss reporting of the duration when there was UL data in SCG Bearer, but the SCG was deactivated in MHI.

MRO for MR-DC SCG failure

In EN-DC and NGEN-DC, the SCGFailureInformationNR is sent to the MN (eNB or ng-eNB) in case of SCG failure. To support MRO, the following information should be added to the SCGFailureInformationNR similar like NR-NR DC:

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

Proposal 3: Include the following information in the SCGFailureInformationNR.

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

Similarly, in NE-DC, the SCGFailureInformationEUTRA is sent to the MN in case of SCG failure. To support MRO, the following information should be added to the SCGFailureInformationEUTRA similar like NR-NR DC:

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

Proposal 4: Include the following information in the SCGFailureInformationEUTRA.

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 to discuss reporting of actual data volume triggering SDT for RACH optimisation.

Proposal 2: RAN2 to discuss reporting of the duration when there was UL data in SCG Bearer, but the SCG was deactivated.

Proposal 3: Include the following information in the SCGFailureInformationNR.

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

Proposal 4: Include the following information in the SCGFailureInformationEUTRA.

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4