3GPP TSG-RAN WG2 Meeting #125bis                                                           R2-2403478

Changsha, China, 15th – 19th April, 2024

Title: 	Discussion on SRAP corrections for U2U relay

Source: 	ZTE, Sanechips

Agenda item:	7.9.4

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss SRAP corrections for U2U relay.

Discussion

- Egress link determination is missing in the title of 5.3a.1.2.

- For BEARER ID field determination, it should be determined in a UE pair. UE pair info is missing for bearer ID determination.

Proposal 1: For BEARER ID field determination, it should be determined in a UE pair. Add UE pair info for BEARER ID determination.

- There is no “sl-SRAP-ConfigPC5” in RRC spec. Instead, the Local ID pair is configured by “sl-LocalID-PairToAddModList” as shown below. UE checks the UE ID fields in SRAP header with all the entries of the configured Local ID pair list.

Proposal 2: The Local ID pair info is configured by “sl-LocalID-PairToAddModList” in RRC spec. UE checks the UE ID fields in SRAP header of the received SRAP PDU with all the entries of the configured Local ID pair list.

RRCReconfigurationSidelink-v1800-IEs ::= SEQUENCE { sl-SFN-DFN-Offset-r18 SetupRelease { SL-SFN-DFN-Offset-r18 } OPTIONAL, -- Need M sl-CarrierToAddModList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierConfig-r18 OPTIONAL, -- Need N sl-CarrierToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierId-r18 OPTIONAL, -- Need N sl-RLC-BearerToAddModList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfig-r18 OPTIONAL, -- Need N sl-RLC-BearerToReleaseList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfigIndex-r18 OPTIONAL, -- Need N sl-LocalID-PairToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N sl-LocalID-PairToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SRAP-ConfigPC5-r18 OPTIONAL, -- Need N nonCriticalExtension SEQUENCE {} OPTIONAL }

- For U2U relay UE case in 5.4, it is not necessary to say the local ID pair configured by the U2U Relay UE. Suggest to remove “configured by the U2U Relay UE”.

- There is no “DST UE ID field” and “SRC UE ID field” defined in the spec. Instead, it is “UE ID (for DST)” field and “UE ID (for SRC)” field. It should keep align in the spec.

Proposal 3: Agree all the changes in the TP.

Conclusion

In this contribution, we discussed SRAP corrections for U2U relay.

- Proposal 1: For BEARER ID field determination, it should be determined in a UE pair. Add UE pair info for BEARER ID determination.

- Proposal 2: The Local ID pair info is configured by “sl-LocalID-PairToAddModList” in RRC spec. UE checks the UE ID fields in SRAP header of the received SRAP PDU with all the entries of the configured Local ID pair list.

Proposal 3: Agree all the changes in the TP.

3GPP TSG-RAN2 Meeting #125bis	R2-24xxxx

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR |  | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on L2 U2U relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2024-4-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Egress link determination is missing in the title of 5.3a.1.2. - For BEARER ID field determination, it should be determined in a UE pair. UE pair info is missing for bearer ID determination. - There is no “sl-SRAP-ConfigPC5” in RRC spec. Instead, the Local ID pair is configured by “sl-LocalID-PairToAddModList” as shown below. UE checks the UE ID fields in SRAP header with all the entries of the configured Local ID pair list. RRCReconfigurationSidelink-v1800-IEs ::= SEQUENCE { sl-SFN-DFN-Offset-r18 SetupRelease { SL-SFN-DFN-Offset-r18 } OPTIONAL, -- Need M sl-CarrierToAddModList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierConfig-r18 OPTIONAL, -- Need N sl-CarrierToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierId-r18 OPTIONAL, -- Need N sl-RLC-BearerToAddModList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfig-r18 OPTIONAL, -- Need N sl-RLC-BearerToReleaseList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfigIndex-r18 OPTIONAL, -- Need N sl-LocalID-PairToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N sl-LocalID-PairToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SRAP-ConfigPC5-r18 OPTIONAL, -- Need N nonCriticalExtension SEQUENCE {} OPTIONAL }. - For U2U relay UE case in 5.4, it is not necessary to say the local ID pair configured by the U2U Relay UE. Suggest to remove “configured by the U2U Relay UE”. - There is no “DST UE ID field” and “SRC UE ID field” defined in the spec. Instead, it is “UE ID (for DST)” field and “UE ID (for SRC)” field. It should keep align in the spec. - Change “SRAP PDU” to “SRAP Data PDU”. |
| Summary of change: |  | - In 5.3a.1.2, add egress link in the title. - In 5.3a.1.2, add UE pair info for bearer ID determination. - In 5.4, remove “sl-SRAP-ConfigPC5”. Add “any one of the entries in sl-LocalID-PairToAddModList”. - In 5.4, remove “configured by the U2U Relay UE” in the sentence. - Replace “DST UE ID field” with “UE ID (for DST)” field and “SRC UE ID field” with “UE ID (for SRC)” field. - Change “SRAP PDU” to “SRAP Data PDU”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mistakes remaining in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3a.1.2, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.3a.1.2	UE ID fields, egress link and BEARER ID field determination

For an U2U SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID (for SRC) field corresponding to sl-RemoteUE-LocalIdentity configured for the concerned sl-RemoteUE-L2Identity and UE ID (for DST) field corresponding to sl-PeerRemoteUE-LocalIdentity configured for the concerned sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3];

-	Determine the egress link on PC5 interface towards the U2U relay UE based on the concerned sl-RemoteUE-L2Identity and sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3];

-	Determine the BEARER ID field for SL-SRBs as the fixed value (i.e., set 0/1/2/3 for SL-SRB0/1/2/3 respectively) or for SL-DRBs as the 5 LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure of the concerned sl-RemoteUE-L2Identity and sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3].

NEXT CHANGE

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Remote UE, if sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity are both configured, when a SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList or a SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or the 5 bits LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID (for DST) field which does not match sl-PeerRemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairToAddModList or a SRAP Data PDU with SRAP header contains BEARER ID field which does not match 0/1/2/3 or the 5 bits LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure is received, or when a SRAP Data PDU that contains a UE ID (for SRC) field which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link is received by U2U Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives a SRAP Data PDU with invalid or reserved values, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

END OF CHNAGE