3GPP TSG-RAN WG2 Meeting #125bis	R2-2403570

Changsha, China,  April 15th – 19th, 2024

Agenda item:	8.1.2.1

Source:	LG Electronics Inc.

Title:	LCM for NW-sided model

Document for:	Discussion and Decision

# 1. Introduction

During R18 AIML SI for NR air interface, there were three use cases: CSI feedback enhancement, Beam management, and Positioning accuracy enhancement. Each use case includes several sub use-cases as following:

- CSI feedback enhancement:

  - Spatial- frequency domain CSI compression using two-sided AIM model

  - Time domain CSI prediction using UE-side model.

- Beam management:

  - BM-Case1(Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams)

  - BM-Case2(Temporal Downlink beam prediction for Set A of beams based on historic measurement results of Set B of beams)

- Positioning accuracy enhancements:

  - Direct AI/ML positioning

  - AI/ML assisted positioning

The life cycle management (LCM) of AI/ML functionality is related to AI/ML operation, e.g., training, inference, monitoring, update, etc. To manage AI/ML operation, the network can indicate activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI).

In this contribution, we discuss the functionality-based LCM for NW-sided model focusing on the beam management use case.

# 2. Discussion

In the case of NW-sided model, we assume that the model is trained at the network. Also, in this case, there is no motivation to perform the monitoring and management of functionality/model in the UE. As the network performs model training, management, and inference, monitoring/management-related signalling, performance feedback and management instruction are internally available within the network.

Observation 1. The NW-sided model is trained and managed in NW side, i.e., LCM related signalling is internally available within the network

<Figure 1. Functional framework for AI/ML for NR Air Interface [1]>

The RAN2 impact currently visible is delivering data related to model training, inference, and monitoring. Since data collection is handled in NW side data collection agenda, this sub-agenda can be de-prioritized.

Proposal 1. To de-prioritize discussion for LCM for NW-sided model. RAN2 impact is delivering data, and it is separately handled in data collection agenda.

# 3. Conclusion

Observation 1. The NW-sided model is trained and managed in NW side, i.e., LCM related signalling is internally available within the network

Proposal 1. To de-prioritize discussion for LCM for NW-sided model. RAN2 impact is delivering data, and it is separately handled in data collection agenda.

# 4. References

[1] 38843-201