3GPP TSG-RAN2 Meeting #125-bis	R2-2403584

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4757 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on tx profile for SL DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2024-4-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 38.300, tx profile indicates whether sidelink DRX is supported or not supported. In 38.331, the value of tx profile for SL CA is drx-Compatible or drx-Incompatible as shown below: SL-TxProfile-r17 ::= ENUMERATED {drx-Compatible, drx-Incompatible, spare6, spare5, spare4, spare3,spare2, spare1} sl-TxProfileList List of one or multiple Tx profiles, indicating the compatibility of supporting SL DRX as specified in TS 38.321 [3]. It is up to the UE implementation whether/how to apply this field.. The meaning of drx-Compatible/Incompatible is not clear. |
| Summary of change: |  | 1. Clarify that the drx-Compatible/Incompatible means DRX is supported/not supported respectively. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR Sidelink Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE does not know the meaning of tx profile value for SL DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

9.3	Sidelink pre-configured parameters

[text omitted]

–	SL-PreconfigurationNR

The IE SL-PreconfigurationNR includes the sidelink pre-configured parameters used for NR sidelink communication. Need codes or conditions specified for subfields in SL-PreconfigurationNR do not apply.

SL-PreconfigurationNR information elements

-- ASN1START

-- TAG-SL-PRECONFIGURATIONNR-START

SL-PreconfigurationNR-r16 ::=             SEQUENCE {

sidelinkPreconfigNR-r16                   SidelinkPreconfigNR-r16,

...

}

SidelinkPreconfigNR-r16 ::=                 SEQUENCE {

sl-PreconfigFreqInfoList-r16                SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16     OPTIONAL,

sl-PreconfigNR-AnchorCarrierFreqList-r16    SL-NR-AnchorCarrierFreqList-r16                                       OPTIONAL,

sl-PreconfigEUTRA-AnchorCarrierFreqList-r16 SL-EUTRA-AnchorCarrierFreqList-r16                                    OPTIONAL,

sl-RadioBearerPreConfigList-r16             SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16      OPTIONAL,

sl-RLC-BearerPreConfigList-r16              SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16        OPTIONAL,

sl-MeasPreConfig-r16                        SL-MeasConfigCommon-r16                                               OPTIONAL,

sl-OffsetDFN-r16                            INTEGER (1..1000)                                                     OPTIONAL,

t400-r16                                    ENUMERATED{ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL,

sl-MaxNumConsecutiveDTX-r16                 ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32}                         OPTIONAL,

sl-SSB-PriorityNR-r16                       INTEGER (1..8)                                                        OPTIONAL,

sl-PreconfigGeneral-r16                     SL-PreconfigGeneral-r16                                               OPTIONAL,

sl-UE-SelectedPreConfig-r16                 SL-UE-SelectedConfig-r16                                              OPTIONAL,

sl-CSI-Acquisition-r16                      ENUMERATED {enabled}                                                  OPTIONAL,

sl-RoHC-Profiles-r16                        SL-RoHC-Profiles-r16                                                  OPTIONAL,

sl-MaxCID-r16                               INTEGER (1..16383)                                                    DEFAULT 15,

...,

[[

sl-DRX-PreConfigGC-BC-r17                   SL-DRX-ConfigGC-BC-r17                                               OPTIONAL,

sl-TxProfileList-r17                        SL-TxProfileList-r17                                                  OPTIONAL,

sl-PreconfigDiscConfig-r17                  SL-RemoteUE-Config-r17                                                OPTIONAL

]]

}

SL-TxProfileList-r17 ::=                    SEQUENCE (SIZE (1..256)) OF SL-TxProfile-r17

SL-TxProfile-r17 ::=                        ENUMERATED {drx-Compatible, drx-Incompatible, spare6, spare5, spare4, spare3,spare2, spare1}

SL-PreconfigGeneral-r16 ::=                 SEQUENCE {

sl-TDD-Configuration-r16                    TDD-UL-DL-ConfigCommon                                                OPTIONAL,

reservedBits-r16                            BIT STRING (SIZE (2))                                                 OPTIONAL,

...

}

SL-RoHC-Profiles-r16 ::=              SEQUENCE {

profile0x0001-r16                     BOOLEAN,

profile0x0002-r16                     BOOLEAN,

profile0x0003-r16                     BOOLEAN,

profile0x0004-r16                     BOOLEAN,

profile0x0006-r16                     BOOLEAN,

profile0x0101-r16                     BOOLEAN,

profile0x0102-r16                     BOOLEAN,

profile0x0103-r16                     BOOLEAN,

profile0x0104-r16                     BOOLEAN

}

-- TAG-SL-PRECONFIGURATIONNR-STOP

-- ASN1STOP

SL-PreconfigurationNR field descriptions

sl-DRX-PreConfig-GC-BC This field indicates the sidelink DRX configuration for groupcast and broadcast communication, as specified in TS 38.321 [3].

sl-OffsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on. If the field is absent, no offset is applied.

sl-PreconfigDiscConfig This field indicates the configuration for discovery message transmission used by NR sidelink U2N Remote UE.

sl-PreconfigEUTRA-AnchorCarrierFreqList This field indicates the EUTRA anchor carrier frequency list, which can provide the NR sidelink communication configuration.

sl-PreconfigFreqInfoList This field indicates the NR sidelink communication and/ or NR sidelink discovery configuration some carrier frequency(ies). In this release, only one SL-FreqConfig can be configured in the list.

sl-PreconfigNR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink communication configuration.

sl-RadioBearerPreConfigList This field indicates one or multiple sidelink radio bearer configurations.

sl-RLC-BearerPreConfigList This field indicates one or multiple sidelink RLC bearer configurations.

sl-RoHC-Profiles This field indicates the supported RoHC profiles for NR sidelink communications.

sl-SSB-PriorityNR This field indicates the priority of NR sidelink SSB transmission and reception.

sl-TxProfileList List of one or multiple Tx profiles, indicating the compatibility of supporting SL DRX as specified in TS 38.321 [3]. drx-Compatible means SL DRX is supported, drx-Incompatible means SL DRX is not supported. It is up to the UE implementation whether/how to apply this field.

[text omitted]

End of the change