3GPP TSG-RAN WG2 Meeting #125bis	R2-2403620

# Changsha, China, April 15th – 19th, 2024

Agenda Item	: 8.2.5 (FS_Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access for Ambient IoT

Document for	: Discussion and Decision

Introduction

A SID for A-IOT was approved as follows.

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Random access from A-IOT UEs

In TR 38.848, the new requirements are captured as follows.

### 5.2.6 Potential New Requirements needed to support the use case [PR 5.2.6-001] The 5G system shall be able to communicate with an Ambient-IoT device. [PR 5.2.6-002] The 5G system shall be able to provide group communication for a group of Ambient-IoT devices. ..

For the potential new requirement, we need to discuss how to support random access from a group of A-IOT UEs first. RAN1 discusses how the A-IOT UE accesses to the network, and made the following agreements.

Agreement From RAN1 perspective, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used. Agreement For A-IoT contention-based access procedure, at least slotted-ALOHA based access is studied.

RAN1 had already considered the random access from a group of A-IOT UEs and use the slotted-ALOHA based access as a baseline. Then RAN2 should follow the RAN1 decision.

Proposal 1. For random access from a group of A-IOT UEs, the slotted-ALOHA based access is used as a baseline.

Then, we need to discuss how to support random access from a designated UE.

The random access from a designated UE would not need contention-based access and designing the contention-free access procedure would be simple and appropriate because the network requests the data transmission only for a designated UE using the dedicated UE identifier.

Proposal 2. For random access from a designated A-IOT UE, the contention-free access is used as a baseline.

  - Contention-based access procedure

The following is an example for the slotted-ALOHA. If the reader requests the data transmission to the multiple UEs, the UE selects a slot and transmits the data. If at least two UEs transmit the data on a same slot, the collision happens. Then, the collided UEs re-selects another slot to transmit data.

In the slotted-ALOHA, the important point is how the A-IOT UE knows the beginning and end of the slot, called contention-based access occasion hereinafter. The A-IOT UE must know the exact timing of contention-based access occasion to transmit or receive data.

In our view, there are two options to indicate the contention-based access occasion.

The first option is that when the network indicates the group-query message to the multiple A-IOT UEs, the network also indicates the contention-based access occasion to the multiple A-IOT UEs. Then, the multiple A-IOT UEs perform the contention-based access procedure.

The second option is that the timing of the contention-based access occasion is pre-defined. If the group-query message is indicated to the multiple A-IOT UEs, the multiple A-IOT UEs performs the contention-based access procedure based on the pre-defined contention-based access occasion.

In our view, both options are feasible. Thus, we propose that RAN2 study both options.

Proposal 3. RAN2 study following options for A-IOT UEs to acquire the timing of the contention-based access occasion.

- Option 1. The network indicates the contention-based access occasion to the A-IOT UEs in group-query message

- Option 2. The timing of the contention-based access occasion is pre-defined.

Even if the UE knows the timing of the contention-based access occasion, there may be collision if multiple UEs choose the same occasion. Thus, a slot reservation mechanism also needs to be considered.

The simple way is that the A-IOT UE considers that the contention-based access occasion is reserved when receiving its UE identifier. For example, at contention-based access occasion#1, the A-IOT UE transmits the A-IOT UE identifier#1. But, if the A-IOT UE receives the A-IOT UE identifier#2, the A-IOT UE considers that the contention-based access occasion is not reserved. Later on, at contention-based access occasion#2, the A-IOT UE transmits the A-IOT UE identifier#1 and the A-IOT UE receives the A-IOT UE identifier#1. In this case, the A-IOT UE considers that the contention-based access occasion is reserved and the A-IOT UE starts the data transmission and reception procedure.

Proposal 4. At the contention-based access occasion, the A-IOT UE transmits the A-IOT UE identifier. If the A-IOT UE receives its UE identifier, the A-IOT UE considers that the contention-based access occasion is reserved.

  - Collision avoidance during contention-based access procedure

Even if contention-based access procedure is used, if the network properly distributes the multiple A-IOT UEs in time domain, collision can be avoided.

For example, the network (pre-) configures the index to each A-IOT UE to indicate when to initiate the contention-based access procedure. In this example, the first A-IOT UE is configured with index 1 and the second A-IOT UE is configured with index 2. If the A-IOT UEs receive the group-query message, each A-IOT UE performs the access procedure based on the time index, and there would not be any collision.

Observation 1. If the network properly distributes the multiple A-IOT UEs in time domain, collision can be avoided even if the A-IOT UEs perform the contention-based access procedure.

Proposal 5. RAN2 study a mechanism to avoid collision while using contention-based access procedure.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. For random access from a group of A-IOT UEs, the slotted-ALOHA based access is used as a baseline.

Proposal 2. For random access from a designated A-IOT UE, the contention-free access is used as a baseline.

Proposal 3. RAN2 study following options for A-IOT UEs to acquire the timing of the contention-based access occasion.

- Option 1. The network indicates the contention-based access occasion to the A-IOT UEs in group-query message

- Option 2. The timing of the contention-based access occasion is pre-defined.

Proposal 4. At the contention-based access occasion, the A-IOT UE transmits the A-IOT UE identifier. If the A-IOT UE receives its UE identifier, the A-IOT UE considers that the contention-based access occasion is reserved.

Observation 1. If the network properly distributes the multiple A-IOT UEs in time domain, collision can be avoided even if the A-IOT UEs perform the contention-based access procedure.

Proposal 5. RAN2 study a mechanism to avoid collision while using contention-based access procedure.