3GPP TSG RAN WG2 #125-bis		R2-2403656

Changsha, China, April 15th – April 19th, 2024

Agenda Item:	8.1.2.2

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on LCM for UE side models

Document for:	Discussion

# Introduction

RAN #102 finalized the new WID for AI/ML for NR air interface [1]. RAN #103 further revised the WID [2]. The WID describes the objectives of general framework for one-sided AI/ML models as follows:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signaling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signaling is part of the above objective - Necessary signaling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signaling mechanism of applicable functionalities/models

This document includes our discussions, observations and proposals on functionality based LCM for AI/ML-based beam management and AI/ML-based positioning accuracy enhancement. Specifically, we focus on the alignment of NW-side additional conditions. We expand upon the agreements and observations in [3]

# Discussion

  - Alignment of NW-side additional conditions for AI/ML based beam management

We propose the following alternatives for the alignment of NW-side additional conditions for AI/ML based beam management:

- NW provides an ID to the UE during training data collection.

  - This ID is later used during inference by the UE-sided model to achieve consistency on NW-side additional conditions. In this scenario, if the UE does not have a model supporting the NW-side additional conditions, and if the NW has a trained model for the additional conditions, then the NW could transfer the trained model to the UE.  Otherwise the NW configures the UE to fallback to a conventional method.

  - If the UE has a model (at least partially) supporting the NW-side additional conditions, the NW can trigger data collection (through configuration) for fine tuning of the UE-sided model.

- In case an ID is not available:

  - the NW configures a UE-side measurement and performs monitoring to achieve consistency of additional conditions. Here, the NW provides a configuration on performance metrics. If the performance criteria is met by the UE-side model(s), then consistency is achieved. Otherwise the NW configures the UE to fallback to a conventional method.

  - Alignment of NW-side additional conditions for AI/ML based positioning

We propose the following alternatives for the alignment of NW-side additional conditions for AI/ML based positioning:

- NW provides an ID to the UE during training data collection.

  - This ID is later used during inference by UE-side model to achieve consistency on NW-side additional conditions. In this scenario, if the UE does not have a model supporting the NW-side additional conditions, and if the NW has a trained model for the additional conditions, then the NW could transfer the trained model to the UE.  Otherwise the NW configures the UE to fallback to a conventional method.

  - If the UE has a model (at least partially) supporting the NW-side additional conditions, the NW can trigger data collection (through configuration) for fine tuning of the UE-side model.

- In case an ID is not available:

  - The NW configures measurements and performs monitoring to achieve consistency of additional conditions. Here, the NW provides a configuration on performance metrics. If the performance criteria is met by the UE-side model(s), then consistency is achieved. Otherwise the NW configures the UE to fallback to a conventional method.

# Conclusion

In this contribution, we have provided a discussion on the alignment of NW-side additional conditions for AI/ML-based beam management and AI/ML-based positioning accuracy enhancement.

Proposal 1: RAN 2 to study the signaling needed to achieve alignment of NW-side additional conditions.

Observation 1: For both beam management and positioning, alignment of NW-side additional conditions could be through an ID assigned by the NW during training data collection. In the absence of an ID, alignment can be achieved by model monitoring.

Proposal 2: For both beam management and positioning, though alignment of NW-side additional conditions could also be achieved via model transfer, to be in line with progress in RAN 1, RAN 2 to prioritize alignment via data collection and model monitoring.

# References

RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0 (2023-12)