3GPP TSG RAN WG2 #125-bis		R2-2403657

Changsha, China, April 15th – April 19th, 2024

Agenda Item:	8.1.3

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on NW side data collection

Document for:	Discussion

# Introduction

RAN #102 finalized the new WID for AI/ML for NR air interface [1]. RAN #103 further revised the WID [2]. The WID describes the objectives of general framework for one-sided AI/ML models as follows:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signaling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signaling is part of the above objective - Necessary signaling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signaling mechanism of applicable functionalities/models

This document includes our discussions, observations and proposals on the NW-side data collection for AI/ML-based beam management. We expand upon agreements and observations captured in [3]

# Discussion

  - Training data collection for AI/ML based beam management

- Set B is a subset of Set A

  - NW configures measurements of Set A

  - UE makes the measurements of Set A

  - Measurements are transferred to the training entity (offline). Choice of Set B is up to implementation.

- Set B is different from Set A

  - NW configures 2 different measurements. One for Set A and one for Set B.

  - UE makes the measurements of Set A and Set B

  - Measurements are transferred to the training entity (offline).

  - Inference data collection for AI/ML based beam management

- NW configures measurements and corresponding report based on the NW-side model input characteristics. These measurements include: Beams (or beam pairs) within Set B.

- UE reports the measurements to the NW.

- NW performs inference

  - BM-Case1 – beam related information of top K beams (beam index or L1-RSRP) in Set A

  - BM-Case2 – beam related information of top K beams (beam index or L1-RSRP) in Set A at future time instance(s)

  - Monitoring data collection for AI/ML based beam management

- Set B is a subset of Set A

  - NW configures measurements of Set A

  - NW configures report of Set A and Set B. Set B is for model inference and Set A is for evaluating the inference.

  - UE makes the measurements of Set A

  - UE reports top K beams of Set B and Set A

- Set B is different from Set A

  - NW configures 2 different measurements and reports. One for Set A and one for Set B.

  - UE makes the measurements of Set A and Set B

  - UE reports top K beams of Set A and Set B

In both cases, the NW computes the model output using measurements from both sets and computes the KPIs. The NW then makes a decision regarding model activation/deactivation/switching/fallback

# Conclusion

In this contribution, we have provided a discussion on NW-side data collection for AI/ML-based beam management.

Proposal 1: Specification of signaling for beam management to enable NW-side training data collection should depend on whether Set B is a subset of Set A or Set B is different from Set A.

Proposal 2: For NW-side beam management training data collection, NW transmits a measurement configuration to the UE. NW trains the model using UE measurements

Proposal 3: For NW-side beam management inference data collection, the NW configures measurements and corresponding report based on the NW-side model input characteristics. The UE reports the measurements to the NW followed by inference at the NW.

Proposal 4: Specification of signaling for beam management to enable NW-side monitoring data collection should depend on whether Set B is a subset of Set A or Set B is different from Set A.

Proposal 5: For NW-side beam management monitoring data collection, the NW configures measurements for model monitoring. The UE then collects the monitoring data. NW computes the model output, computes the KPIs and makes a decision regarding model activation/deactivation/switching/fallback

# References

RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0 (2023-12)