3GPP TSG-RAN WG2 Meeting #125bis					     R2-2403689

Changsha, China,  April 15th – 19th, 2024

Agenda item:	8.9.2

Source:	Sateliot

Title:	RAN aspects of S&F operation for IoT NTN

Document for:	Discussion and Decision

# Introduction

The support of S&F operation for delay-tolerant services is one of the objectives of Rel-19 IoT NTN work item [1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In parallel, support of S&F is being addressed at SA2 under FS_ FS_5GSAT_ARCH_Ph3 [2]. The study item is expected to be finalized by TSG #104 (June 2024).

In this contribution, we share our view on the possible impacts that S&F operation can have on the radio interface.

# Discussion

Some discussion on potential RAN impact is provided on our discussion paper submitted to SA2#160 (S2-2313155, [3]). Such contribution outlines a general framework in terms of architecture and potential solution elements for the realization of S&F operation in multi-satellite IoT NTN deployments.

Assuming an architecture with full eNB on board the satellite, it’s our view that support for S&F can be introduced with a minimum impact on the RAN functions excepting the broadcasting of system information (SI) intended to advertise UEs whether the network is operating in S&F mode or in normal (default) mode.

Observation#1. Assuming an architecture with full eNB on board the satellite, it’s our view that support for S&F can be introduced with a minimum impact on the RAN functions excepting the broadcasting of system information (SI) intended to advertise UEs whether the network is operating in S&F mode or in normal (default) mode.

In this respect, it is important to consider that a given satellite cell could be alternating between S&F and normal operation mode during an orbit. For instance, consider the case of an Earth-moving satellite cell in a NGSO constellation such as the one depicted in Figure 1. Such a satellite cell will have to be operated necessarily in S&F mode when flying over service areas in which the satellite is or cannot be connected to the ground network. But the same satellite cell may be operated in normal/default mode in other service areas if a feeder link is available.

Figure 1. Illustration of a Earth-moving satellite cell operating switching operation between “normal/default” and “S&F” operation modes.

Proposal#1. The information indicating whether/when a satellite cell is operating in S&F mode or normal/default mode should be broadcasted via system information.

More details on the above proposal can be found in our previous contribution R2-2301886 [4], submitted to RAN2#121 in March 2023.

# Conclusion

Observation#1. Assuming an architecture with full eNB on board the satellite, it’s our view that support for S&F can be introduced with a minimum impact on the RAN functions excepting the broadcasting of system information (SI) intended to advertise UEs whether the network is operating in S&F mode or normal (default) mode.

Proposal#1. The information indicating whether/when a satellite cell is operating in S&F mode or normal/default mode should be broadcasted via system information.

# References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

[2] SP-231199, “New SID on Study on Integration of satellite components in the 5G architecture Phase III”, Rapporteur (Thales)

[3] S2-2313155, “Discussion on a reference framework for S&F for IoT NTN”

[4] R2-2301886, “Complementing discontinuous coverage with minimum support for discontinuous feeder link operation”, Sateliot, GateHouse, Novamint, Intelsat, Airbus, Hispasat, ESA, TNO, 3GPP TSG-RAN WG2 Meeting #121-e, February 27th – March 3rd, 2023