3GPP TSG-RAN WG2 Meeting #125bis	                                    R2-2403739

Changsha, China, April 15th – 19th, 2024

Source:             ZTE Corporation, Sanechips

Title:                  [Z116][Z102][S863]  Consideration on the MUSIM UAI Reporting

Agenda item:    7.17.2

Document for:   Discussion and Decision

- Introduction

In the new round ASN.1 review, [RIL-Z116] was proposed with the description as below

In the current structure all the r18 MUSIM UAI are packaged together, which means once one trigger condition (e.g. Gap priority, Cap Restriction or needforGap) was satisfied, the UE has to include all of the R18 MUSIM UAI. Now there is no prohibit timer for the needforGap, the prohibit timer for the Proactive Cap restriction/Gap priority reporting would be disabled (for that even the UAI was triggered by needforGap, the UE would still include other R18 MUSIM UAI with current ASN.11 structure and procedure description)

Furthermore, [Z102][S863] was proposed to solve the musim-MaxCC reporting related issue as below:

Problem description in the Z102: The UE is unable to trigger UAI to indicate the maximum CC restriction for the case that the UE has only the maximum CC restriction and the UE did not transmit a UEAssistanceInformation with musim-MaxCC since it was configured to provide MUSIM. We think the musim-MaxCC shall also be restricted on the musim-CandidateBandList, which means if the UE only has musim-MaxCC restriction on the bands outside of the musim-CandidateBandList, then there is no need to report. For example, the candidate bands are all FR1 bands. But the UE has no CC number restriction on FR1 bands (may have restriction on the FR2 bands), then there is no need to report musim-MaxCC Problem description in the S863: Consider that UE has initiated UAI for reporting max-cc information and started prohibit timer. UE need not initiate UAI report to report max-cc even for the first time, while T346n is running. Further prohibit timer may be “started or restarted”, rather than “started”

In this paper, we’d like to give detail analysis on these issues

- Discussion

2.1  [Z116] on UAI Setting

With the current MUSIM UAI structure and UAI setting procedure as below, once the MUSIM UAI was triggered, the UE has to include all of the R18 MUSIM UAI information (i.e. musim-GapPriorityPreferenceList-r18/ musim-GapKeepPreference-r18/ musim-CapRestriction-r18/ musim-NeedForGapsInfoNR-r18)

UEAssistanceInformation-v1800-IEs ::= SEQUENCE { /********************omit the unrelated part********************************************/ musim-Assistance-v1800 MUSIM-Assistance-v1800 OPTIONAL, } MUSIM-Assistance-v1800 ::= SEQUENCE { musim-GapPriorityPreferenceList-r18 MUSIM-GapPriorityPreferenceList-r18 OPTIONAL, musim-GapKeepPreference-r18 ENUMERATED {true} OPTIONAL, musim-CapRestriction-r18 MUSIM-CapRestriction-r18 OPTIONAL, musim-NeedForGapsInfoNR-r18 NeedForGapsInfoNR-r16 OPTIONAL }

1> if transmission of the UEAssistanceInformation message is initiated to provide MUSIM assistance information according to 5.7.4.2 or 5.3.5.3: 2> if the UE has a preference for MUSIM periodic gap(s): 3> include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured; … 2> if the UE has a preference for MUSIM aperiodic gap: 3> include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured; … 2> if UE has a preference for temporary capability restriction: ... 3> if UE has a preference for measurement gap requirement: … 2> if the UE has no longer preference for temporary capability restriction: 3> do not include the corresponding temporary capability restriction preference in the musim-CapRestriction in the musim-Assistance IE;

In the last meeting, it has also been agreed that

=> No need to define a timer if initiates transmission of the UAI message to provide measurement gap requirement information.

Then we consider the case that the UE has reported the proactive capability restriction to the network and start the prohibit timer, then the UAI was triggered to provide measurement gap requirement information, the UE has to include the proactive capability restriction in the UAI again even the prohibit timer is running.

Observation 1: With the current MUSIM UAI setting procedure description and the Asn.1 structure, the proactive capability restriction/Gap priority assistance information may be reported again even the corresponding prohibit timer is running.

Furthermore, there is no prohibit timer for the case to provide need for Gap info in the UAI, which means that the reporting of the proactive capability restriction/Gap priority assistance information may also be out of the prohibit timer control.

To solve this issue, the MUSIM UAI setting and MUSIM UAI ASN.1 structure shall be decoupled. From the ASN.1 aspect, the MUSIM UAI structure shall be changed as below:

UEAssistanceInformation message /******************************Omit the unchanged part**********************************/ UEAssistanceInformation-v1800-IEs ::= SEQUENCE { idc-FDM-Assistance-r18 IDC-FDM-Assistance-r18 OPTIONAL, idc-TDM-Assistance-r18 IDC-TDM-Assistance-r18 OPTIONAL, multiRx-PreferenceFR2-r18 ENUMERATED {single, multiple } OPTIONAL, musim-Assistance-v1800 MUSIM-Assistance-v1800 OPTIONAL, musim-GapPriorityPreferenceList-r18 MUSIM-GapPriorityPreferenceList-r18 OPTIONAL, musim-GapKeepPreference-r18 ENUMERATED {true} OPTIONAL, musim-CapRestriction-r18 MUSIM-CapRestriction-r18 OPTIONAL, musim-NeedForGapsInfoNR-r18 NeedForGapsInfoNR-r16 OPTIONAL /********************Omit the uanchanged part************************************************/ MUSIM-Assistance-v1800 ::= SEQUENCE { musim-GapPriorityPreferenceList-r18 MUSIM-GapPriorityPreferenceList-r18 OPTIONAL, musim-GapKeepPreference-r18 ENUMERATED {true} OPTIONAL, musim-CapRestriction-r18 MUSIM-CapRestriction-r18 OPTIONAL, musim-NeedForGapsInfoNR-r18 NeedForGapsInfoNR-r16 OPTIONAL }

This modification is also included in the TP1 of Annex.

Proposal 1: Agree with the modification to the R18 MUSIM UAI structure as in the TP1.

For the procedure description part, in the current spec, once the MUSIM UAI was triggered, all of the MUSIM UAI items would be set. To solve this issue, when setting the MUSIM UAI, the detail trigger condition shall be considered for the corresponding MUSI UAI items. For example only when the UAI was triggered by the proactive UAI reporting, the proactive assistance information would be included. Based on this the TP2 was proposed as in the Annex

Proposal 2: Agree with the modification to the R18 MUSIM UAI setting procedure as in the TP2.

2.2  [Z102][S863] on Max-CC Reporting

For the proactive UE capability reporting, the related description is as below:

1> if configured to provide MUSIM assistance information for temporary capability restriction: 2> if the UE has temporary capability restriction on the current configuration and timer T348 is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList; 3> start the timer T348 with the timer value set to the musim-WaitTimer. 2> if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList since it was configured to provide MUSIM assistance information for temporary capability restriction; or 2> if the current musim-AffectedBandsList and/or musim-AvoidedBandsList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList; 3> start the timer T346n with the timer value set to the musim-ProhibitTimer. 2> if the current musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-MaxCC; 3> start the timer T346n with the timer value set to the musim-ProhibitTimer.

With this description, when the UE has only the maximum CC restriction and the UE did not transmit a UEAssistanceInformation with musim-MaxCC since it was configured to provide MUSIM, the UE are unable to trigger the UAI to indicate the maximum CC restriction.

Observation 2: The UE is unable to trigger UAI to indicate the maximum CC restriction for the case that the UE has only the maximum CC restriction and the UE did not transmit a UEAssistanceInformation with musim-MaxCC since it was configured to provide MUSIM.

To solve this issue, we need to merge the description on the musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC together for that they are all regulated by the prohibit timer and are all belong to the proactive restriction.

Before providing the detail solution, we need to confirm whether the musim-MaxCC shall also be restricted on the musim-CandidateBandList first. Obviously, there are 2 options:

- Option 1: The musim-MaxCC was also restricted on the musim-CandidateBandList.

- Option 2: The musim-MaxCC was not restricted on the musim-CandidateBandList

For the option 1, if the UE only has musim-MaxCC restriction on the bands outside of the musim-CandidateBandList, then there is no need to report. For example, the candidate bands are all FR1 bands, but the UE has no CC number restriction on FR1 bands (may have restriction on the FR2 bands), then there is no need to report musim-MaxCC. However if go to the option 2, the UE has to report the maximum CC restriction for this case, and this restriction would have impact to all of the bands/bands combinations.

Proposal 3: Ran2 to discuss which option is correct for the restriction on the musim-MaxCC:

- Option 1: The musim-MaxCC was also restricted on the musim-CandidateBandList;

- Option 2: The musim-MaxCC was not restricted on the musim-CandidateBandList.

Based on the above observation and proposal, the TP was also provided for each option.

TP for the Option 1: For the understanding that the musim-MaxCC was also restricted on the musim-CandidateBandList,

2> if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction; or 2> if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC; 3> start the timer T346n with the timer value set to the musim-ProhibitTimer. 2> if the current musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-MaxCC; 3> start the timer T346n with the timer value set to the musim-ProhibitTimer.

TP for the Option 2: For the understanding that the musim-MaxCC was not restricted on the musim-CandidateBandList (compared with solution 1, the only difference is highlighted in yellow)

2> if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList or if the UE has temporary capability restriction on the maximum CC number, and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction; or 2> if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC; 3> start the timer T346n with the timer value set to the musim-ProhibitTimer. 2> if the current musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-MaxCC; 3> start the timer T346n with the timer value set to the musim-ProhibitTimer.

The above TP was also included in the Annex.

Proposal 4: Agree the TP3 or TP4 as in the Annex based on the conclusion of the Proposal 3.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: With the current MUSIM UAI setting procedure description and the Asn.1 structure, the proactive capability restriction/Gap priority assistance information may be reported again even the corresponding prohibit timer is running.

Proposal 1: Agree with the modification to the R18 MUSIM UAI structure as in the TP1.

Proposal 2: Agree with the modification to the R18 MUSIM UAI setting procedure as in the TP2.

Observation 2: The UE is unable to trigger UAI to indicate the maximum CC restriction for the case that the UE has only the maximum CC restriction and the UE did not transmit a UEAssistanceInformation with musim-MaxCC since it was configured to provide MUSIM.

Proposal 3: Ran2 to discuss which option is correct for the restriction on the musim-MaxCC:

- Option 1: The musim-MaxCC was also restricted on the musim-CandidateBandList;

- Option 2: The musim-MaxCC was not restricted on the musim-CandidateBandList.

Proposal 4: Agree the TP3 or TP4 as in the Annex based on the conclusion of the Proposal 3.

- Annex

TP 1

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

/******************************Omit the unchanged part**********************************/

UEAssistanceInformation-v1800-IEs ::= SEQUENCE {

idc-FDM-Assistance-r18                IDC-FDM-Assistance-r18                          OPTIONAL,

idc-TDM-Assistance-r18                IDC-TDM-Assistance-r18                          OPTIONAL,

multiRx-PreferenceFR2-r18             ENUMERATED {single, multiple }                  OPTIONAL,

musim-Assistance-v1800                MUSIM-Assistance-v1800                          OPTIONAL,

musim-GapPriorityPreferenceList-r18     MUSIM-GapPriorityPreferenceList-r18  OPTIONAL,

musim-GapKeepPreference-r18             ENUMERATED {true}                    OPTIONAL,

musim-CapRestriction-r18                MUSIM-CapRestriction-r18             OPTIONAL,

musim-NeedForGapsInfoNR-r18             NeedForGapsInfoNR-r16                OPTIONAL

/********************Omit the uanchanged part************************************************/

MUSIM-Assistance-v1800 ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18     MUSIM-GapPriorityPreferenceList-r18           OPTIONAL,

musim-GapKeepPreference-r18             ENUMERATED {true}                             OPTIONAL,

musim-CapRestriction-r18                MUSIM-CapRestriction-r18                      OPTIONAL,

musim-NeedForGapsInfoNR-r18             NeedForGapsInfoNR-r16                         OPTIONAL

}

TP 2

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

1>	if transmission of the UEAssistanceInformation message is initiated to provide MUSIM Gap assistance information or provide MUSIM assistance information for leaving RRC_CONNECTED according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

4>	if UE has a preference for MUSIM gap priority;

5>	include the musim-GapPriorityPreferenceList the UE prefers to be configured;

5>	if the UE has preference to keep all colliding MUSIM gaps for periodic MUSIM gap(s):

6>	include the musim-GapKeepPreference;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-CapRestriction-r18according to 5.7.4.2 or 5.3.5.3:

2>	if UE has a preference for temporary capability restriction:

3>	if UE has a preference for serving cell(s) and/or SCG to be released:

4>	include the musim-Cell-SCG-ToRelease;

5>	set musim-CellToRelease to include the serving cell(s) the UE prefers to be released;

5>	set scg-ReleasePreference to scgReleasePreferred if the UE prefers the SCG to be released;

3>	if UE has a preference to indicate the serving cells with restricted capabilities:

4>	include the musim-CellToAffectList the UE prefers to be configured;

5>	include the musim-ServCellIndex and the musim-MIMO-Layers-DL/ musim-MIMO-Layers-UL/ musim-SupportedBandwidth-DL/ musim-SupportedBandwidth-UL for the corresponding serving cell;

3>	if UE has a preference to indicate the maximum number of CCs:

4>	include the musim-caRestriction for the corresponding musim-MaxCC the UE prefers to be configured;

5>	include the the musim-MaxCC-DL/ musim-MaxCC-UL for the corresponding maximum number of CCs;

3>	if UE has a preference to indicate band(s) and/or combination(s) of bands with capabilities restricted which comprise of the band(s) that is/are indicated in musim-CandidateBandList:

4>	include the musim-AffectededBandsList the UE prefer to be configured with capabilities restricted;

5>	include the musim-bandEntryIndex for each band or each band of the combination(s) for which capabilities are restricted;

5>	include the musim-CapabilityRestricted for the corresponding band;

3>	if UE has a preference to indicate band(s) and/or combination(s) of bands to be avoided which comprise of band(s) that is indicated in musim-CandidateBandList:

4>	include the musim-AvoidedBandsList the UE prefers not to be configured;

5>	include the musim-bandEntryIndex for each band or each band of the combination(s) to be avoided;

2>	if the UE has no longer preference for temporary capability restriction:

3>	do not include the corresponding temporary capability restriction preference in the musim-CapRestriction IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-NeedForGapsInfoNR-r18 according to 5.7.4.2 or 5.3.5.3:

2 3>if UE has a preference for measurement gap requirement:

3 4>if the TargetBandFilterNR-r16 of NeedForGapsConfigNR is configured:

4 5>include the musim-NeedForGapsInfoNR to provide the measurement gap requirement information from the requestedTargetBandFilterNR-r16 of NeedForGapsConfigNR configuration in RRCResume message or RRCReconfiguration message of NR target bands the UE prefer to be configured;

3 4>else:

4 5>include the musim-NeedForGapsInfoNR to provide the measurement gap requirement information   for all the supported bands;

3 4>include the gap requirement information of intra-frequency measurement for each supported NR serving cell.

2>	if the UE has no longer preference for measurement gap requirement:

3>do not include the musim-NeedForGapsInfoNR;

TP 3 (based on option 1 in proposal 3)

#### 5.7.4.2	Initiation

/*************************************omit the unrelated part************************************/

2>	if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction; or

2>	if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC;

3>	start the timer T346n with the timer value set to the musim-ProhibitTimer.

2>	if the current musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-MaxCC;

3>	start the timer T346n with the timer value set to the musim-ProhibitTimer.

TP 4 (based on option 2 in proposal 4)

#### 5.7.4.2	Initiation

/*************************************omit the unrelated part************************************/

2>	if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList or if the UE has temporary capability restriction on the maximum CC number, and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction; or

2>	if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC;

3>	start the timer T346n with the timer value set to the musim-ProhibitTimer.

2>	if the current musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-MaxCC;