3GPP TSG-RAN2 Meeting #125-bis	R2-2403864

Changsha, China, 15 – 19 April 2024

Title:	DRAFT LS on RLM and BFD relaxation when short DRX is configured

Response to:	-

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	Ericsson [TSG RAN WG2]

To:	3GPP RAN WG4

Cc:

Contact Person:

#### Name:		Martin van der Zee

E-mail Address:	martin.van.der.zee@ericsson.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R2-2403862 (CR 38.331 Rel-17) and R2-2403863 (CR 38.331 Rel-18)

1. Overall Description:

RAN4 captured in TS 38.133 that the UE is not UE is not allowed to perform RLM and/or BFD relaxation when the configured DRX cycle is larger than 80 ms. Furthermore TS 38.133 is agnostic about the configuration of both a short and long DRX cycle.

For the case when short DRX cycle is configured, i.e. both a short and long DRX cycle are configured, RAN2 captured the following in TS 38.331:

When the short DRX cycle is smaller or equal to 80 ms and the long DRX cycle is larger than 80 ms then the UE is allowed to perform RLM and/or BFD relaxation while drx-ShortCycleTimer is running and the relaxed measurement criteria are met.

The CRs to TS 38.331 are attached for information.

2. Actions:

To RAN4

ACTION: 	RAN2 kindly asks RAN4 to take note of the RAN2 agreement and reply if needed.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #126		20 - 24 May 2024		Fukuoka, Japan

TSG RAN2 Meeting #127		19 - 23 August 2024		Maastricht, The Netherlands