3GPP TSG-RAN WG2 Meeting #125bis		R2-2403986

Changsha, China, 15th Apr. – 19th Apr. 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1827 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RACH resource set availability check |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Guangdong Genius |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core, NR_cov_enh-Core, NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2024-4-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#125 meeting, the resource set in MAC and RRC was discussed and agreed: - The understanding and intention in RAN2 is that one set of Random Access resource could include both 2-step and 4-step RA type. This is already in specification. But in the description in Clause 5.1.1C, it is stated as: The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type: The current wording causes confusion that each set of RACH resource is either for 4-step RA type or for 2-step RA type. It is better to update the corresponding description to avoid such confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the decription in Clause 5.1.1C to avoid any confusion. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality RACH resource set selection for RedCap Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no impact. - If the UE is implemented according to the CR and the network is not, there is no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not clear the the RACH resource set could include either 2-step RA type, or 4-step RA type, or both 2-step and 4-step RA type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.1.1c	Availability of the set of Random Access resources

The MAC entity shall for each set of configured Random Access resources:

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT.

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

End of change