3GPP TSG RAN WG2#125bis	R2-2402133

Changsha, China, 15th - 19th April 2024

3GPP TSG-RAN WG4 Meeting # 110	R4-2403654

Athens, Greece, Feb 26 – March 1, 2024

Title:	LS on SRS and PRS bandwidth aggregation feature for positioning

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 agreed that the UL CA capability shall be the pre-requisite for a UE support RAN1 feature 41-4-6 and parameter defined in feature 41-4-6 needs to be in line with parameters reported in UL CA capability. For example, the first two components defined in feature 41-4-6 are quoted as below

- The number of supported aggregated carriers in intra band contiguous carriers

- Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE

For the first component parameter, it shall be less than or equal to the maximum number of the component carrier associated with IE ca-BandwidthClassUL-NR.

For the second component parameter, it shall be less than or equal to the maximum aggregated transmission bandwidth associated with IE ca-BandwidthClassUL-NR. Additionally, it shall be less than or equal to the maximum aggregated bandwidth for the supported CA configuration in Table 5.5A.1-1 in TS 38.101-1 for FR1 bands or Table 5.5A.1-1 in TS 38.101-2  for FR2 bands  for the band where aggregated SRS CCs is configured.

IE ca-BandwidthClassUL-NR in 38.306 is quoted as below.

| ca-BandwidthClassUL-NR Defines for UL, the class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by the UE, as specified in TS 38.101-1 [2] and TS 38.101-2 [3]. When all FeatureSetUplinkId:s in the corresponding FeatureSetsPerBand are zero, this field is absent. For FR1, the value 'F' shall not be used as it is invalidated in TS 38.101-1 [2]. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Based on above RAN4 agreement, For the second component parameter, the following modifications for the values in component 2 in feature 41-4-6 are suggested,

For 2 in Component 1:

FR1 bands: {20, 40, 50, 80, 100, 160, 180, 190,200}

FR2 bands: {50, 100,200, 400, 600, 800}

For 3 in Component 1:

FR1 bands: {80, 100, 160,200, 240, 300}

FR2 bands: {50, 100, 200,300,400, 600,800, 1000, 1200}

For RAN1 feature 41-4-7, RAN4 agreed that the capability on UE power class to transmit the SRS aggregated CCs needs to be reported to the network. The reported power class can be PC2 or PC3 and applicable to FR1 only. For the case when UE can support aggregation of 2 and 3 SRS CCs, and if the power class for 2 aggregated CCs is different with power class for 3 aggregated CCs, both power class should be indicated to the network.

2. Actions:

To RAN WG1/WG2 group.

ACTION: 	RAN4 would like RAN1/RAN2 take the information from this LS into consideration.

RAN4 kindly asks RAN1 to update the description of feature 41-4-7 to include the sub-component on the UE power class and other additional modification of the values in maximum aggregated UL SRS bandwidth component

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #110		Feb 26 – March 01, 2024					Athens

TSG RAN WG4 Meeting #110bis		April 15 – April 19, 2024					China