3GPP TSG-RAN WG2 Meeting #126	R2-2404236

Fukuoka, Japan May 20th – 24th, 2024

Agenda Item:	7.2.5

Source: 	CATT

Title: 	Activation of SRS When Configured with Validity Area

Document for: 	Discussion and Decision

1	Introduction

In this contribution, the activation of srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig is discussed.

2	Discussion

Issue#1: activation of srs-PosRRC-InactiveValidityAreaPreConfigList

In RAN2#125bis, the following RIL is proposed.

| H906 | For this case, the trigger is not from the RRC layer, So the AC should not be determined in the RRC layer. | AC shouled be determined in the upper layer rather thant he RRC layer for activation request. | The clause does not say that it is triggered by RRC layer. It only says if triggered but who triggers is not stated. we can discuss this in coming meeting. Anyhow AC is needed for this case too. |
| --- | --- | --- | --- |

Based on the RIL H906, the following modification is implemented, and the reason is “For this case, the trigger is not from the RRC layer, So the AC should not be determined in the RRC layer.”. How the access category is determined is still FFS.

1> else if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 2> if the resumption of the RRC connection is triggered due to cell reselection as specified in clause 5.3.13.6; or 2> if the resumption of the RRC connection is triggered due to the need for SRS for positioning configuration and no stored srs-PosRRC-InactiveValidityAreaPreConfigList for the camped cell exists: 3> if an emergency service is ongoing: 4> select '2' as the Access Category; 4> set the resumeCause to emergency; 3> else: 4> select '8' as the Access Category; 4> set the resumeCause to srs-PosConfigOrActivationReq;

However, for this issue, we would like to first discuss whether the comment by RIL H906 is valid, i.e., whether the trigger for activation of srs-PosRRC-InactiveValidityAreaPreConfigList is from upper. After the srs-PosRRC-InactiveValidityAreaPreConfigList is configured for the UE via RRCRelease message, the RRC layer does not inform the configuration to the upper layer. That is, the upper layer does not know whether the UE has valid SRS configuration, and whether the SRS type is srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig or R17 SRS configuration. Hence, the activation request cannot be triggered by upper layer.

Observation 1: The upper layer does not know whether the UE has valid SRS and whether the SRS type is srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig or R17 SRS configuration, so the SRS activation request cannot be triggered by upper layer.

The upper layer (application layer) can only decide when to trigger the positioning event, and trigger RRC layer to perform positioning. Then the RRC layer will initiate positioning procedure, including request SRS configuration and request SRS activation to the NW. This issue has been discussed online in RAN2#125, and reached consensus. So the access category and resume cause should be set by RRC layer.

Proposal 1: The activation of srs-PosRRC-InactiveValidityAreaPreConfigList should be triggered by RRC layer rather than upper layer, and the access category should also be determined by RRC layer because upper layer does not know whether the UE has valid SRS and whether the SRS type is srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig or R17 SRS configuration. Adopt the TP in Annex with name P1.

Issue#2: activation of SP SRS

In RAN2#123, we achieved the following agreement that SP SRS is supported to be configured with validity area and reuse legacy mechanism to deactivate the SP SRS.

Agreement:

Semi-persistent SRS is supported to be configured with validity area, and RAN2 agree to reuse legacy mechanism to deactivate the SP SRS

However, we have never discussed how to activate the configured SP SRS. From our perspective, for both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig, the transmission of SP SRS should be controlled by the activate MAC CE as legacy.

Proposal 2: For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig, the transmission of SP SRS should be activated by MAC CE as legacy. There is no protocol impact.

In legacy mechanism, when the UE has positioning requirement, it will send event report to LMF. Then the LMF could know when to activate the SP SRS. But for the mechanism of SRS configured with validity area, it is not clear how the LMF know when to activate the SP SRS and which cell the activation command is sent to. Possible solutions are given below,

- For srs-PosRRC-InactiveValidityAreaPreConfigList, when the UE sends the resume cause srs-PosConfigOrActivationReq to indicate the change or activations of SRS configuration, if the preconfigured SRS is SP SRS, the network can activate the SP SRS with the activation MAC CE as legacy. This activation request of preconfigured SP SRS can be covered by proposal 1, i.e., the highlight part in TS 38.331 V18.1..

1> else if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 2> if the resumption of the RRC connection is triggered due to cell reselection as specified in clause 5.3.13.6; or, 2> if the resumption of the RRC connection is triggered due to the need for SRS for positioning configuration and no stored srs-PosRRC-InactiveValidityAreaPreConfigList for the camped cell exists; or, 2> if the resumption of the RRC connection is triggered for activation of srs-PosRRC-InactiveValidityAreaPreConfigList when the UE is camped in one of the cells indicated in srs-PosConfigValidityArea; 3> if an emergency service is ongoing: 4> select '2' as the Access Category; 4> set the resumeCause to emergency; 3> else: 4> select '8' as the Access Category; 4> set the resumeCause to srs-PosConfigOrActivationReq;

- For srs-PosRRC-InactiveValidityAreaNonPreConfig, the resume cause srs-PosConfigOrActivationReq can also be used for UE request activating the SP SRS. That is, after the SP SRS is configured, when the UE has positioning requirement, it could send RRCResumeRequest message to the gNB with resume cause set to srs-PosConfigOrActivationReq to request activating the SP SRS. Then the NW could activate the SP SRS for the UE correspondingly.

By this way, the activation of SP SRS for preconfigured mechanism and non-preconfigured mechanism are aligned.

Proposal 3: For mechanisms of non-preconfigure SRS, if SP SRS is configured, the resume cause srs-PosConfigOrActivationReq can be used for RRC request activating the SP SRS. Adopt the TP in Annex with name P3.

Then the network releases the UE to RRC_INACTIVE state with the RRCRelease, after the UE receives RRCRelease and the activation MAC CE, the SP SRS is activated, i.e. the UE can apply the SP SRS.

Proposal 4: For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig, if SP SRS is configured, after the UE receives RRCRelease and the activation MAC CE, the UE can apply the transmission of SP SRS.

For the procedure of SP SRS activation between gNB and LMF, there may also some impacts on RAN3. RAN2 can send an LS to RAN3 on our agreements on activation of SP SRS for their discussion on possible protocol impacts.

Proposal 5: Send an LS to RAN3 on RAN2’s agreement on activation of SP SRS for their discussion on possible protocol impacts.

3	Conclusion

Issue#1: activation of srs-PosRRC-InactiveValidityAreaPreConfigList

Observation 1: The upper layer does not know whether the UE has valid SRS and whether the SRS type is srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig or R17 SRS configuration, so the SRS activation request cannot be triggered by upper layer.

Proposal 1: The activation of srs-PosRRC-InactiveValidityAreaPreConfigList should be triggered by RRC layer rather than upper layer, and the access category should also be determined by RRC layer because upper layer does not know whether the UE has valid SRS and whether the SRS type is srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig or R17 SRS configuration. Adopt the TP in Annex with name P1.

Issue#2: activation of SP SRS

Proposal 2: For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig, the transmission of SP SRS should be activated by MAC CE as legacy. There is no protocol impact.

Proposal 3: For mechanisms of non-preconfigure SRS, if SP SRS is configured, the resume cause srs-PosConfigOrActivationReq can be used for RRC request activating the SP SRS. Adopt the TP in Annex with name P3.

Proposal 4: For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig, if SP SRS is configured, after the UE receives RRCRelease and the activation MAC CE, the UE can apply the transmission of SP SRS.

Proposal 5: Send an LS to RAN3 on RAN2’s agreement on activation of SP SRS for their discussion on possible protocol impacts.

4	Reference

- 3GPP	TR 38.331: "Radio Resource Control (RRC) protocol specification ".

# Annex: Modifications to TS 38.331

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, upon requesting multicast reception as specified in clause 5.3.13.1d, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a, for NR sidelink positioning as specified in clause 5.3.13.1c, for requesting configuration for SRS for positioning, for activation of preconfigured Positioning SRS in RRC_INACTIVE, upon receiving RRCRelease message including resumeIndication) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging; or

1> if the resumption of the RRC connection is triggered by receiving RRCRelease message including resumeIndication; or

1>	if the resumption of the RRC connection is triggered for multicast reception as specified in clause 5.3.13.1d:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

3>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and/or in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

NOTE 0:	If there are multiple NSAGs with the same highest NAS-provided NSAG priority identified for access attempt as above, it is left to UE implementation to select the NSAG to be applied in the Random Access procedure.

2>	if the resumption occurs after release with redirect with mpsPriorityIndication:

3>	set the resumeCause to mps-PriorityAccess;

2>	else:

3>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE 1:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

1>	else if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured:

2>	if the resumption of the RRC connection is triggered due to cell reselection as specified in clause 5.3.13.6; or

2>	if the resumption of the RRC connection is triggered due to the need for SRS for positioning configuration and no stored srs-PosRRC-InactiveValidityAreaPreConfigList for the camped cell exists	; or

2>	if the resumption of the RRC connection is triggered for activation of srs-PosRRC-InactiveValidityAreaPreConfigList when the UE is camped in one of the cells indicated in srs-PosConfigValidityArea: or

2>	if the resumption of the RRC connection is triggered due to activation of SP SRS of srs-PosRRC-InactiveValidityAreaConfig when the UE is camped in one of the cells indicated in srs-PosConfigValidityArea:

3>	if an emergency service is ongoing:

4>	select '2' as the Access Category;

4>	set the resumeCause to emergency;

3>	else:

4>	select '8' as the Access Category;

4>	set the resumeCause to srs-PosConfigOrActivationReq;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection resume triggered either by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.13.1a, or by reception of the RemoteUEInformationSidelink message containing the connectionForMP as specified in 5.3.13.1a, the L2 U2N Relay UE sets the resumeCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as resumeCause, if the same cause value in the message received from the L2 U2N Remote UE via SL-RLC0.

1>	if the UE is in NE-DC or NR-DC:

2>	if the UE does not support maintaining SCG configuration upon connection resumption:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

2>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default PDCP configuration as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

2>	apply the default SRB1 configuration as specified in 9.2.1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	release idc-AssistanceConfig from the UE Inactive AS context, if stored;

1>	release drx-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346a, if running;

1>	release maxBW-PreferenceConfig and maxBW-PreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346b, if running;

1>	release maxCC-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346c, if running;

1>	release maxMIMO-LayerPreferenceConfig and maxMIMO-LayerPreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346d, if running;

1>	release minSchedulingOffsetPreferenceConfig and minSchedulingOffsetPreferenceConfigExt for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346e, if running;

1>	release rlm-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346j, if running;

1>	release bfd-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346k, if running;

1>	release releasePreferenceConfig from the UE Inactive AS context, if stored;

1>	release wlanNameList from the UE Inactive AS context, if stored;

1>	release btNameList from the UE Inactive AS context, if stored;

1>	release sensorNameList from the UE Inactive AS context, if stored;

1>	release obtainCommonLocation from the UE Inactive AS context, if stored;

1>	stop timer T346f, if running;

1>	stop timer T346i, if running;

1>	release referenceTimePreferenceReporting from the UE Inactive AS context, if stored;

1>	release sl-AssistanceConfigNR from the UE Inactive AS context, if stored;

1>	release musim-GapAssistanceConfig from the UE Inactive AS context, if stored and stop timer T346h, if running;

1>	release musim-GapConfig from the UE Inactive AS context, if stored;

1>	release musim-GapPriorityAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-LeaveAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-CapabilityRestrictionConfig from the UE Inactive AS context, if stored and stop timer T346n, if running;

1>	release propDelayDiffReportConfig from the UE Inactive AS context, if stored;

1>	release ul-GapFR2-PreferenceConfig, if configured;

1>	release rrm-MeasRelaxationReportingConfig from the UE Inactive AS context, if stored;

1>	release multiRx-PreferenceReportingConfigFR2 if configured, and stop timer T346m, if running;

1>	release aerial-FlightPathAvailabilityConfig from the UE Inactive AS context, if stored;

1>	release ul-TrafficInfoReportingConfig from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346l, if running;

1>	if the UE is acting as L2 U2N Remote UE:

2>	apply the specified configuration of SL-RLC0 used for the delivery of RRC message over SRB0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ncd-SSB-RedCapInitialBWP-SDT is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored ncd-SSB-RedCapInitialBWP-SDT:

3>	release the stored ncd-SSB-RedCapInitialBWP-SDT;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

1>	set the variable pendingRNA-Update to false;

1>	release successHO-Config from the UE Inactive AS context, if stored;

1>	release successPSCell-Config configured by the PCell from the UE Inactive AS context, if stored;

1>	release successPSCell-Config configured by the PSCell from the UE Inactive AS context, if stored;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.