3GPP TSG-RAN WG2 Meeting #126	R2-2404285

Fukuoka, Japan, 20 – 25 May 2024

Agenda item:	7.5.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	RAN3 Overview

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Overview

The following topics have been discussed in RAN3:

-	MDBV added in the Alternative QoS (as per SA2 agreement) endorsed in R3-242127 (38.413), R3-242129 (38.423), R3-242128 (38.473), and R3-242130 (37.483).

-	Scenario for non-homogeneous deployment of PDU set handling (i.e. UE transition from RRC INACTIVE to RRC CONNECTED with a new serving gNB, and RRC re-establishment with a new serving gNB) endorsed in R3-242131 for 38.300.

-	Correction on PSI based discard endorsed in R3-242132 for TS 38.300.

-	Non-integer DRX cycle in F1AP relying on a new non-integer DRX Cycle IE endorsed in R3-242216 for TS 38.473.

-	Correction on burst arrival time in handover endorsed in R3-242194 for TS 38.300: in case of handover the target gNB needs to recalculate the burst arrival time of the QoS flow according to the SFN offset between the source and target gNBs.

-	Regarding 2RX for XR:

-	RAN3 endorsed TS 38.423 CR (R3-241531) and TS 38.473 CR (R3-241696) to indicate the cell is barred for 2Rx XR UE.

-	RAN3 endorsed TS 38.413 CR (R3-242108) to add a new cause value “2Rx XR UE not supported”, which is used to inform the source gNB that the HO is failed due to 2Rx UE not supported in target gNB.

# 2	Conclusion

For the Rel-18 work on XR, RAN3 discussed and made new agreements on MDBV for alternative QoS, PDU set handling, PSI based discard, the signalling of the Non-integer DRX cycle in F1AP, burst arrival time handling at handover and 2RX UEs. From RAN2 viewpoint, the following is worth noting:

-	Correction on PSI based discard endorsed in R3-242132 for TS 38.300.