3GPP TSG RAN WG2 #126                                                                           R2-2404371

Fukuoka, Japan May 20th – 24th, 2024

Source:	TCL

Title:	Discussion on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

Data collection is a very important function in the AIML-based LCM, which is used to provide the input data to the different LCM aspects (i.e., model inference, monitoring, and training). RAN2#125 bis meeting has achieved some general agreements about NW-side data collection [1].

Agreements 1 For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches 2 We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection. 3 RAN2 supports enhancements to MDT for data collection framework for training. FSS Whether to enhance logged or immediate MDT

In this contribution, we provide our views on NW-side data collection for training in beam management use cases.

- Discussion

During the Rel-18 study, RAN1 sent an LS reply about data collection in R1-2308730 [2], which confirmed the requirements of data collection for different LCM aspects (i.e., model training, model inference, and model monitoring) per use case, including data content, typical data size, reporting type, and typical latency requirement. Moreover, RAN2 has fully discussed the existing data collection frameworks and evaluated their applicability and usefulness for AI/ML-based data collection, which are listed in the following Table 7.3.1.2-1, captured in TR38.843[3].

The agreements about NW-sided training data collection are outlined below, which consider the gNB-centric data collection mechanism and OAM-centric data collection mechanism.

TR 38.843 ……. Considerations for network-side data collection A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging, - UE to report the collected data periodically, event-based, and on-demand, - The UE memory, processing power, energy consumption, signalling overhead should be considered. Note: The above principles can be revised depending on RAN1 requirements. Furthermore, and regarding the use cases in this study, the following is considered. For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms. The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed. On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed. For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. Note: For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved. Note: For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

Observation 1: NW-sided training data collection includes gNB-centric data collection mechanism and OAM-centric data collection mechanism.

2.1 gNB-centric data collection

For the gNB-centric data collection for model training, means that the gNB can configure the UE to initiate/terminate the data collection procedure. As described in TR 38.843[3], it mentioned that UE may use L3 signalling to report the collected data to the gNB, which can be used in beam management use case. Considering the data size is the one of most important determining factors for assessing the existing data collection frameworks for training data collection, RAN1 has provided the examples  to roughly illustrate the typical training data size for beam management, which can be considered as guidance for RAN2 discussion，we summary that in Table1.

Table1: The typical data size for model training in beam management

| Use case | Data content for model training | Data size (example) |
| --- | --- | --- |
| Beam management | L1-RSRPs [and beam-IDs] for Set B | - For Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), - For Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits). |
|  | L1-RSRPs and/or beam-IDs from Set A is the training data for the NW-side model |  |

Observation 2: For data collection of NW-sided training, the collected data (i.e., L1-RSRP/beam ID) is generated by UE and terminated at the NW, the corresponding typical data size is up to 500 bits (Set A = 128), or 100 bits (Set B = 16).

From our perspective, L3 signalling may be used for gNB-centric training data collection in the beam management use case. Given the limitation of L3 signalling (RRC message) size, the maximum size per RRC signalling reporting is 9kbyte. Moreover, if the RRC segmentation is supported, the uplink RRC message can support a payload with 144kbyte. Hence, a single RRC signalling can carry hundreds sets of training data for beam management based on the data size in Table1. On the other hand, since the model trained by NW is usually area-specific, or cell-specific, so NW can gather more training data from many UEs. In this sense, the L3 signalling can achieve a complete training data collection for beam management.

Proposal 1: For gNB-centric training data collection, L3 signalling can be used for the beam management use cases.

In our understanding, the L3 signalling mechanism includes two kinds of data collection frameworks, including L3 measurement reporting (RRM reporting) and UE assistant information (UAI).

- L3 measurement reporting (RRM reporting)

In the legacy L3 measurement reporting framework, UE reports the measurement results based on the measurement configurations, the framework with beam measurement results is straightforward to be used for the data collection. However, the legacy measurement results only include the beam index, it is not supported if the measurement result per beam is L1-RSRP, therefore, the new AIML data content should be indicated in the L3 measurement configuration and/or reporting configuration. Moreover, there is only a single measurement result can be collected and reported to the gNB in the legacy L3 measurement reporting, which is unable to satisfy the large data size requirement of model training. One alternative is to use UE to perform multiple reports for the collected training data, while it will increase the signalling overhead and power consumption. Then, the current event-trigger reporting about measurement results is used to assist the HO decision, the measurement event conditions seem not applicable to trigger the training data for AIML. Hence, we suggest to consider the new criteria for training data collection, which is used to log the training data in legacy L3 measurement reporting, e.g.:

- Data size is larger than a threshold.

- Collecting time of data collection meets a defined time duration.

- UE assistant information (UAI)

In the current UAI framework, UE sends the UAI message to gNB based on the configuration, or UE satisfies some criterion configured by NW. UAI message is used to transfer the UE’s preference or a specific status. The UAI framework is flexible for the configuration. Hence, we think it also can be used for the data collection of model training. Similar to the discussion of L3 measurement reporting, on the one hand, UAI needs to support the beam measurement results transmission, such as L1-RSRP. On the other hand, to reduce the signalling overhand, new criteria need to be introduced and can be configured in the RRCReconfiguration message. The criteria is same as the L3 measurement reporting

Proposal 2: L3 signalling used for training data collection, which can be L3 measurement reporting and UE assistant information (UAI).

Proposal 3: New AIML data content (e.g., L1-RSRP) should be supported in L3 measurement reporting and UE assistant information (UAI).

Proposal 4: NW should configure the new criteria for training data collection for logging the training data, e.g.,

- Data size is larger than a threshold.

- Collecting time of data collection meets a defined time duration.

In addition, in the current NR specification, UL dedicated message segmentation only applies to UECapabilityInformation and MeasurementReportAppLayer, where SRB1 is used at the transfer of segments of UECapabilityInformation and SRB4 is used at the transfer of segments of MeasurementReportAppLayer. Thus, if the size of the training data is larger than the limit of 9kbytes, in order to support the training data collection based on the L3 measurement reporting and UAI, it is suggested that L3 measurement reporting and UAI can support the RRC segmentation, with a payload size of 144kbyte.

Proposal 5: Suggest that L3 measurement reporting and UAI can support the RRC segmentation, if needed.

- New RB for gNB-centric training data collection

As above discussion, in certain use cases, the scale of training data is large, and using the conventional control plane transmission approach (i.e., RRC message) is impractical. This not only increases the signaling overhead but also occupies other control information transmission resources. Hence, we think introduce a special Radio Block (RB) for collecting the training data. Contrary to the legacy DRB, the establishment of this RB is terminated between the UE and gNB.

Proposal 6: Consider using a new RB to support the training data collection, which is terminated between the UE and gNB.

2.2 OAM-centric data collection

MDT framework is natural to use for OAM-centric training data collection, since the terminated node is OAM/TCE. Considering the corresponding Uu signalling is also smaller than 9kbyte. There, similar to the discussion of gNB-centric data collection, the MDT framework is only applicable to beam management use case.

Proposal 7: For OAM-centric training data collection, the MDT framework can be used for beam management use cases.

In the current logged MDT framework, gNB sends the LoggedMeasurementConfiguration message to the connected UE, which is used to configure the UE to log the measurement results, while the UE starts to perform the logging when it is in inactive/idle state. Subsequently, if the UE accesses NW again, gNB may retrieve stored logged measurement information utilizing the UE information procedure. Considering the logged MDT can only be enabled for idle/inactive UEs, there is a limitation for AIML-based data since the training data (L1-RSRP) for beam management is only obtained in the connected UEs. Therefore, we suggest the logged MDT framework should not be considered for the training data collection framework.

Proposal 8: Logged MDT framework should not be considered for the training data collection framework.

The immediate MDT framework is applied in connected UE, which includes the UE measurement configuration. More specifically, the configuration for UE measurements is based on the existing RRC measurement procedures for configuration and reporting RRM measurement results, also with some extensions for location information, the information is terminated at TCE/OAM by gNB in the immediate MDT.  It is obvious that the immediate MDT framework is similar to the RRM measurement framework over the air interface. Hence, we think the mentioned enhancements for collecting the training data in the RRM measurement framework can be reused in the immediate MDT.

Observation 3: Immediate MDT framework is similar to the RRM measurement framework over the air interface.

Proposal 9: Immediate MDT framework is recommended to be used for training data collection.

As the above discussion, we can find that the legacy frameworks for collecting the training data in both gNB-centric data collection and OAM-centric data collection need to do some similar enhancements, for example, they may need to support transmitting the L1-related data or logging the training data. Hence, a unified data collection framework for gNB-centric data collection and OAM-centric data collection can be considered, which is useful to reduce RAN2’s workload. From our perspective, the RRM measurement framework can be regarded as a starting point to explore the unified data collection framework.

Proposal 10: For beam management, RAN2 considers a unified scheme for both gNB- and OAM-centric data collection, in which the RRM measurement framework can be regarded as a starting point.

- Conclusion

We have the following observations and proposals:

Observation 1: NW-sided training data collection includes gNB-centric data collection mechanism and OAM-centric data collection mechanism.

Observation 2: For data collection of NW-sided training, the collected data (i.e., L1-RSRP/beam ID) is generated by UE and terminated at the NW, the corresponding typical data size is up to 500 bits (Set A = 128), or 100 bits (Set B = 16).

Observation 3: Immediate MDT framework is similar to the RRM measurement framework over the air interface.

gNB-centric data collection

Proposal 1: For gNB-centric training data collection, L3 signalling can be used for the beam management use cases.

Proposal 2: L3 signalling used for training data collection, which can be L3 measurement reporting and UE assistant information (UAI).

Proposal 3: New AIML data content (e.g., L1-RSRP) should be supported in L3 measurement reporting and UE assistant information (UAI).

Proposal 4: NW should configure the new criteria for training data collection for logging the training data, e.g.,

- Data size is larger than a threshold.

- Collecting time of data collection meets a defined time duration.

Proposal 5: Suggest that L3 measurement reporting and UAI can support the RRC segmentation, if needed.

Proposal 6: Consider using a new RB to support the training data collection, which is terminated between the UE and gNB.

OAM-centric data collection

Proposal 7: For OAM-centric training data collection, the MDT framework can be used for beam management use cases.

Proposal 8: Logged MDT framework should not be considered for the training data collection framework.

Proposal 9: Immediate MDT framework is recommended to be used for training data collection.

Proposal 10: For beam management, RAN2 considers a unified scheme for both gNB- and OAM-centric data collection, in which the RRM measurement framework can be regarded as a starting point.

- References

- RAN2 #125 bis Chair Notes

- R1-2308730 Reply LS on Data Collection Requirements and Assumptions

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0 (2023-12)