3GPP TSG-RAN2 Meeting #126	R2-2404537

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4687 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [E075] SDT corrections for harmonizing T319a and SDT ongoing labels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation (rapporteur), Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core, NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At RAN2#125-bis the following agreements were reached for “SDT ongoing” description in RRC: Agreements - Clarify that “The UE in RRC_INACTIVE state while T319a is running SDT procedure is ongoing, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.” – Apart from this the UE behaviour in RRC as specified currently is clear. - We will keep both labels but clarify when SDT is considered ongoing (e.g. in definitions section) The above agreements need to be captured in the RRC spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the definitions section capture a definition for when SDT is considered ongoing in RRC. - Clarify in NOTE1 in clause 5.2.2.3.1 that the UE, during SDT needs to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception only whilst T319a is running. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The description of when SDT is ongoing is missing RRC - The requirement for the UE to receive SIB1 and MIB according to NOTE1 in clause 5.2.2.3.1 is applicable even before T319a starts when there is no unicast data to send/receive. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 5.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

2Rx XR UE: Two antenna port XR UE as specified in TS 38.101-1 [15].

A2X communication: A communication to support A2X services leveraging PC5 reference points, as defined in TS 23.256 [76]. A2X services are realized by various types of A2X applications, e.g., BRID or DAA.

Additional RLC bearer: If the sidelink PDCP entity is associated with two sidelink RLC entities, the additional RLC bearer is the RLC bearer configured by sl-RLC-BearerToAddModListSizeExt in sl-ConfigDedicatedNR, or sl-RLC-BearerConfigListSizeExt in SIB12 or in SidelinkPreconfigNR.

Aerial UE: UE performing Aerial UE communication, as defined in TS 38.300 [2], clause 16.18 and TS 23.256 [76].

AM MRB: An MRB associated with at least an AM RLC bearer for PTP transmission.

BH RLC channel: An RLC channel between two nodes, which is used to transport backhaul packets.

Broadcast MRB: A radio bearer configured for MBS broadcast delivery.

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time, as defined in TS 23.501 [32].

Dedicated signalling: Signalling sent on DCCH logical channel between the network and a single UE.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signalling. In the dormant BWP, the UE stops monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured. For each serving cell other than the SpCell or PUCCH SCell, the network may configure one BWP as a dormant BWP.

eRedCap UE: A UE with enhanced reduced capabilities as specified in clause 4.2.22.1 in TS 38.306 [26].

Field: The individual contents of an information element are referred to as fields.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Frequency Selection Area ID: An identity used for broadcast MBS session to guide the frequency selection of the UE as defined in TS 23.247 [67].

Global cell identity: An identity to uniquely identifying an NR cell. It is consisted of cellIdentity and plmn-Identity of the first PLMN-Identity in plmn-IdentityList in SIB1.

Information element: A structural element containing single or multiple fields is referred as information element.

Candidate configuration: A configuration part of an RRCReconfiguration message associated with a candidate cell, e.g., for LTM or subsequent CPAC. A candidate configuration can be a complete candidate configuration or a delta configuration relatively to a reference configuration.

Reference configuration: A configuration provided by the network to the UE that is common, within the same cell group, to a group of configured non-complete candidate configurations.

MBS Radio Bearer: A radio bearer that is configured for MBS delivery.

Mobile IAB-MT: mobile IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. The mobile IAB-MT uses the same procedures and behaviours specified for the IAB-MT, unless explicitly stated otherwise.

Mobile IAB-node: RAN node that supports NR access links to UEs and an NR backhaul link to a parent node, and that can conduct physical mobility across the RAN area. The mobile IAB-node function used in 38-series of 3GPP Specifications corresponds to the MBSR function defined in TS 23.501 [32]. The mobile IAB-node uses the same procedures and behaviours specified for the IAB-node, unless explicitly stated otherwise.

Multicast/Broadcast Service: A point-to-multipoint service as defined in TS 23.247 [67].

Multicast MRB: A radio bearer configured for MBS multicast delivery.

MUSIM gap: Period that the UE may use to perform MUSIM operations.

Multi-path: Mode of operation of a UE in RRC_CONNECTED configured with one direct path on which the UE connects to gNB using NR Uu, and one indirect path on which the UE connects to the same gNB via another UE using PC5 unicast link or Non-3GPP Connection.

MP remote UE: A UE configured with Multi-path. When the connectivity of indirect path is PC5 unicast link, the MP remote UE is acting as a L2 U2N Remote UE. When the connectivity of indirect path is Non-3GPP Connection, the MP remote UE is acting as a N3C remote UE.

MP relay UE: A UE that provides connectivity of indirect path to a MP remote UE. When the connectivity is PC5 unicast link, the MP relay UE is acting as a L2 U2N Relay UE. When the connectivity is Non-3GPP Connection, the MP relay UE is acting as a N3C relay UE.

NCSG: Network controlled small gap as defined in TS 38.133 [14].

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

N3C indirect path: In Multi-path, the indirect path using Non-3GPP Connection between remote UE and relay UE.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [55] and/or A2X Communication as defined in TS 23.256 [76] and/or ProSe Communication (including ProSe UE-to-Network Relay, non-Relay communication and ProSe UE-to-UE Relay Communication) as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery, ProSe UE-to-Network Relay discovery and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink positioning: AS functionality which determines geographical or relative location and possibly velocity of a target UE or ranging via PC5 interface using SL-PRS transmission and reception as defined in TS 38.305 [73] and TS 38.355 [77].

PNI-NPN identity: an identifier of a PNI-NPN comprising of a PLMN ID and a CAG-ID combination.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

PC5 Relay RLC channel: An RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, or between L2 U2U Remote UE and L2 U2U Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network relay or L2 UE-to-UE relay.

PDU Set: one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s) or video slice(s) for XR Services), as defined in TS 23.501 [32].

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH by PUCCH-Config.

PUSCH-Less SCell: An SCell configured without PUSCH.

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [26].

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Small Data Transmission: A procedure used for transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state (i.e. without the UE transitioning to RRC_CONNECTED state). The SDT procedure is considered to be ongoing once the conditions for initating SDT as specified in clause 5.3.13.1b are fulfilled until the SDT procedure is completed either successfully or unsuccessfully as specified in clause 18.0 in TS 38.300 [2].

SNPN identity: an identifier of an SNPN comprising of a PLMN ID and an NID combination.

SL indirect path: In Multi-path, the indirect path using PC5 unicast link between remote UE and relay UE.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

Split DRB: In MR-DC, a DRB that supports transmission via MCG and SCG, as well as duplication of PDCP PDUs as defined in TS 37.340 [41]; or in MP, a DRB that supports transmission via direct path and indirect path, as well as duplication of PDCP PDUs.

Split SRB: In MR-DC, an SRB that supports transmission via MCG and SCG as well as duplication of RRC PDUs as defined in TS 37.340 [41]; or in MP, a SRB that supports transmission via direct path and indirect path, as well as duplication of PDCP PDUs.

SSB Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: A UE that communicates with the network via a U2N Relay UE.

U2U Relay UE: A UE that provides functionality to support connectivity between two U2U Remote UEs.

U2U Remote UE: A UE that communicates with other UEs via a U2U Relay UE.

Uu Relay RLC channel: An RLC channel between L2 U2N Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network relay or for indirect path in case of MP.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.3.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [56], between nearby UEs, using E-UTRA technology but not traversing any network node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

A2X	Aircraft-to-Everything

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

ATG	Air to Ground

BAP	Backhaul Adaptation Protocol

BCD	Binary Coded Decimal

BFD	Beam Failure Detection

BH	Backhaul

BLER	Block Error Rate

BRID	Broadcast Remote Identification

BSR	Buffer Status Report

BWP	Bandwidth Part

CA	Carrier Aggregation

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CAPC	Channel Access Priority Class

CBR	Channel Busy Ratio

CCCH	Common Control Channel

CFR	Common Frequency Resources

CG	Cell Group

CHO	Conditional Handover

CLI	Cross Link Interference

CMAS	Commercial Mobile Alert Service

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

C-RNTI	Cell RNTI

CSI	Channel State Information

DAA	Detect And Avoid

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DFN	Direct Frame Number

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

DL-SCH	Downlink Shared Channel

DM-RS	Demodulation Reference Signal

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DSR	Delay Status Report

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered Inertial

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

G-CS-RNTI	Group Configured Scheduling RNTI

GERAN	GSM/EDGE Radio Access Network

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

G-RNTI	Group RNTI

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

HRNN	Human Readable Network Name

HSDN	High Speed Dedicated Network

H-SFN	Hyper SFN

HST	High Speed Train

IAB	Integrated Access and Backhaul

IAB-DU	IAB-node DU

IAB-MT	IAB Mobile Termination

IDC	In-Device Coexistence

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LBT	Listen Before Talk

LEO	Low Earth Orbit

LTM	L1/L2 Triggered Mobility

MAC	Medium Access Control

MBS	Multicast/Broadcast Service

MBS FSAI	MBS Frequency Selection Area Identity

MCCH	MBS Control Channel

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MIB	Master Information Block

MO-SDT	Mobile Originated SDT

MPE	Maximum Permissible Exposure

MP	Multi-path

MRB	MBS Radio Bearer

MR-DC	Multi-Radio Dual Connectivity

MTCH	MBS Traffic Channel

MT-SDT	Mobile Terminated SDT

MTSI	Multimedia Telephony Service for IMS

MUSIM	Multi-Universal Subscriber Identity Module

N3C	Non-3GPP Connection

N/A	Not Applicable

NCR	Network-Controlled Repeater

NCR-Fwd	NCR Forwarding

NCR-MT	NCR Mobile Termination

NE-DC	NR E-UTRA Dual Connectivity

NES	Network Energy Savings

(NG)EN-DC	E-UTRA NR Dual Connectivity (covering E-UTRA connected to EPC or 5GC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to 5GC

NID	Network Identifier

NPN	Non-Public Network

NR-DC	NR-NR Dual Connectivity

NR/5GC	NR connected to 5GC

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

posSIB	Positioning SIB

PPW	PRS Processing Window

PRS	Positioning Reference Signal

PSCell	Primary SCG Cell

PSI	PDU Set Importance

PTM	Point to Multipoint

PTP	Point to Point

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RLM	Radio Link Monitoring

RMTC	RSSI Measurement Timing Configuration

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RS	Reference Signal

SBAS	Satellite Based Augmentation System

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SD-RSRP	Sidelink Discovery RSRP

SDT	Small Data Transmission

SFN	Single Frequency Network

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SL-PRS	Sidelink Positioning Reference Signal

SLSS	Sidelink Synchronisation Signal

SNPN	Stand-alone Non-Public Network

SpCell	Special Cell

SRAP	Sidelink Relay Adaptation Protocol

SRB	Signalling Radio Bearer

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDCP	Time Domain Channel Property

TDD	Time Division Duplex

TEG	Timing Error Group

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

TN	Terrestrial Network

TSS	Timing Synchronization Status.

U2N	UE-to-Network

U2U	UE-to-UE

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

VR	Virtual Reality

VSAT	Very Small Aperture Terminal

XR	eXtended Reality

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

Next change

## 5.2	System information

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB;

NOTE 1:	If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB.

-	the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 and posSIBs are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs or posSIBs having the same periodicity can be mapped to the same SI message. SIBs and posSIBs are mapped to different SI messages, i.e. an SI message contains either only SIBs or only posSIBs. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted. An SI message may be repeated with the same content a number of times within the SI-window. Any SIB or posSIB except SIB1 can be configured to be cell specific or area specific, using an indication in SIB1. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells and is identified by systemInformationAreaID;

-	The mapping of SIBs to SI messages is configured in schedulingInfoList and schedulingInfoList2, while the mapping of posSIBs to SI messages is configured in posSchedulingInfoList and schedulingInfoList2.
Each SIB and each posSIB is mapped to a single SI message. posSIBs of the same posSibType carrying GNSS Generic Assistance Data for different GNSS/SBAS (identified by gnss-id/sbas-id, see TS 37.355 [49]) are mapped to different SI messages.
Each SIB and posSIB is contained at most once in an SI message.
For SIBs and posSIBs with segments, the segments contained in SI messages are transmitted according to the SI message periodicity, with one segment of a particular sibType/posSibType in each SI message;

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information, paging, or upon request from the UE.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, the network releases and adds the concerned SCell. For PSCell, the required SI can only be changed with Reconfiguration with Sync.

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

### 5.2.2	System information acquisition

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB17 (if the UE is using TRS resources for power saving in RRC_IDLE and RRC_INACTIVE), SIB19 (if UE is accessing NR via NTN access) and SIB22 (for ATG access).

The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20, regardless of the RRC state the UE is in.

The UE shall ensure having a valid version of the posSIB requested by upper layers.

#### 5.2.2.2	SIB validity and need to (re)-acquire SIB

##### 5.2.2.2.1	SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after satellite switch with resynchronization, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49].

A L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected L2 U2N Relay UE as defined in clause 5.8.9.8.2 and receive the SIB(s) from the L2 U2N Relay UE as defined in clause 5.8.9.9.3. A L2 U2N Remote UE in RRC_CONNECTED receives SIB1 and other SIB(s) in RRCReconfiguration message and performs on-demand SI request if required, as defined in clause 5.2.2.3.5 and 5.2.2.3.6. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED (when MP is not configured) is not required to obtain SI over Uu interface, but it may decide to perform the SI acquisition procedure over Uu interface as defined in clause 5.2.2.3 by UE implementation.

NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

1>	for each stored version of a posSIB:

2>	if the areaScope is associated and its value for the stored version of the posSIB is the same as the value received in the posSIB-MappingInfo for that posSIB from the serving cell and the systemInformationAreaID included in the si-SchedulingInfo is identical to the systemInformationAreaID associated with the stored version of that posSIB:

3>	if the valueTag (see TS 37.355 [49]) for the posSIB received from the serving cell is identical to the valueTag associated with the stored version of that posSIB; or if the expirationTime (see TS 37.355 [49]) associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the posSIB and the areaScope value is not included in the posSIB-MappingInfo for that posSIB from the serving cell and the cellIdentity for the posSIB received from the serving cell is identical to the cellIdentity associated with the stored version of that posSIB:

3>	if the valueTag (see TS 37.355 [49]) for the posSIB received from the serving cell is identical to the valueTag associated with the stored version of that posSIB; or if the expirationTime (see TS 37.355 [49]) associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

##### 5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI message (other than SI message for ETWS, CMAS, positioning assistance data, and some NTN-specific information as specified in the field descriptions ) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SIB1, and UE is configured with eDRX, modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m = 0.

For UEs in RRC_IDLE or RRC_INACTIVE configured to use an IDLE eDRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 = 0.

The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period or within preceding eDRX acquisition period. SI change indication is not applicable for SI messages containing posSIBs.

UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

During a modification period where ETWS or CMAS transmission is started or stopped, the SI messages carrying the posSIBs scheduled in posSchedulingInfoList may change, so the UE might not be able to successfully receive those posSIBs in the remainder of the current modification period and next modification period according to the scheduling information received prior to the change.

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging.

ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

For Short Message reception in a paging occasion, the UE monitors the PDCCH monitoring occasion(s) for paging as specified in TS 38.304 [20] and TS 38.213 [13].

A L2 U2N Remote UE is not required to monitor paging occasion for SI modifications and/or PWS notifications. It obtains the updated system information and SIB6/7/8 from the connected L2 U2N Relay UE as defined in clause 5.8.9.9.3.

If the UE receives a Short Message, the UE shall:

1>	if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceSIB1 and searchSpaceOtherSystemInformation on the active BWP or the initial BWP:

2> immediately re-acquire the SIB1;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6:

3>	acquire SIB6, as specified in clause 5.2.2.3.2, immediately;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7:

3>	acquire SIB7, as specified in clause 5.2.2.3.2, immediately;

2>	if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8:

3>	acquire SIB8, as specified in clause 5.2.2.3.2, immediately;

NOTE:	In case SIB6, SIB7, or SIB8 overlap with a measurement gap it is left to UE implementation how to immediately acquire SIB6, SIB7, or SIB8.

1>	if the UE does not operate an IDLE eDRX cycle longer than the modification period and the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next modification period;

1>	if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary.

#### 5.2.2.3	Acquisition of System Information

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

NOTE 1:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 and MBS broadcast if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 2:	UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]).