3GPP TSG-RAN WG2 Meeting #126                                      R2-2404611

Fukuoka, Japan, May 20th – 24th, 2024

Source:         ZTE Corporation

Title:           Discussion on reporting PRS resource ID in PRS BW aggregation

Agenda item:    7.2.1

Document for:	Discussion and Decision

- Introduction

This discussion paper discusses the LS from RAN1 on the reporting of PRS resource ID.

- Discussion

The RAN1 LS R1-2403728[1] indicates the PRS resource ID should be optionally reported in aggregation reporting:

However, from RAN1 perspective, PRS resource ID can be optionally reported for PRS bandwidth aggregation. Hence, RAN1 made the following agreement. Agreement Send an LS to RAN2 indicating that at least one PRS resource ID from one of aggregated resource sets may be reported in one measurement report element for the main and additional measurements, respectively. The existing PRS resource ID can be reused, details up to RAN2..

- Also RAN1 indicates the existing Rel-16 reported PRS resource ID can be reused. This implies that the reported aggregated PRS resource set of Rel-18 and the reported PRS resource set of Rel-16 should be aggregated, and the reported PRS resource ID of Rel-16 can let LMF know the PRS resource linage, i.e., PRS resource linkage is determined by the same set of PRS configuration, if LMF knows one Rel-16 PRS resource ID, LMF will know the linked Rel-18 PRS resource ID. So there is no need to additionally include PRS resource ID in NR-AggregatedDL-PRS-ResourceSetID-Element.

- To achieve using the existing PRS resource ID, the changes are summarized below:

- Change nr-AggregatedDL-PRS-ResourceSetID-List-r18 from (SIZE (2.. 3)) to (SIZE (1.. 2));

- Change field description in nr-AggregatedDL-PRS-ResourceSetID-List-r18 to ‘if the field is present, the nr-DL-PRS-ResourceSetID-r16 should be present, and the nr-DL-PRS-ResourceID-r16 is taken as one of aggregated PRS resource ID’.

- The TP of DL-TDOA is provided as below, based on R2-2403818 [2]:

-- ASN1START NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE { dl-PRS-ReferenceInfo-r16 DL-PRS-ID-Info-r16, nr-DL-TDOA-MeasList-r16 NR-DL-TDOA-MeasList-r16, ..., [[ nr-UE-RxTEG-TimingErrorMargin-r17 TEG-TimingErrorMargin-r17 OPTIONAL -- Cond UERxTEG ]] } NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ..., kMinus6-r18 INTEGER (0..126083073), kMinus5-r18 INTEGER (0..63041537), kMinus4-r18 INTEGER (0..31520769), kMinus3-r18 INTEGER (0..15760385), kMinus2-r18 INTEGER (0..7880193), kMinus1-r18 INTEGER (0..3940097) }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16 OPTIONAL, ..., [[ nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17) OPTIONAL, nr-DL-PRS-FirstPathRSRP-Result-r17 INTEGER (0..126) OPTIONAL, nr-los-nlos-Indicator-r17 CHOICE { perTRP-r17 LOS-NLOS-Indicator-r17, perResource-r17 LOS-NLOS-Indicator-r17 } OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL, nr-DL-TDOA-AdditionalMeasurementsExt-r17 NR-DL-TDOA-AdditionalMeasurementsExt-r17 OPTIONAL ]], [[ nr-MeasBasedOnAggregatedResources-r18 ENUMERATED {true} OPTIONAL, nr-AggregatedDL-PRS-ResourceSetID-List-r18 SEQUENCE (SIZE (1.. 2)) OF NR-AggregatedDL-PRS-ResourceSetID-Element-r18 OPTIONAL, nr-RSCPD-r18 INTEGER (0..3599) OPTIONAL, nr-PhaseQuality-r18 NR-PhaseQuality-r18 OPTIONAL, nr-RSCPD-AddMeasurementSamples-r18 SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF NR-RSCPD-AdditionalMeasurementSamplesElement-r18 OPTIONAL, nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 ENUMERATED { singleHop, multipleHop } OPTIONAL ]] } NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16 NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-ResultDiff-r16 CHOICE { k0-r16 INTEGER (0..8191), k1-r16 INTEGER (0..4095), k2-r16 INTEGER (0..2047), k3-r16 INTEGER (0..1023), k4-r16 INTEGER (0..511), k5-r16 INTEGER (0..255), ..., kMinus6-r18 INTEGER (0..524224), kMinus5-r18 INTEGER (0..262112), kMinus4-r18 INTEGER (0..131056), kMinus3-r18 INTEGER (0..65528), kMinus2-r18 INTEGER (0..32764), kMinus1-r18 INTEGER (0..16382) }, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-ResultDiff-r16 INTEGER (0..61) OPTIONAL, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, ..., [[ nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17) OPTIONAL, nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 INTEGER (0..61) OPTIONAL, nr-los-nlos-IndicatorPerResource-r17 LOS-NLOS-Indicator-r17 OPTIONAL, nr-AdditionalPathListExt-r17 NR-AdditionalPathListExt-r17 OPTIONAL ]], [[ nr-MeasBasedOnAggregatedResources-r18 ENUMERATED {true} OPTIONAL, nr-AggregatedDL-PRS-ResourceSetID-List-r18 SEQUENCE (SIZE (1.. 2)) OF NR-AggregatedDL-PRS-ResourceSetID-Element-r18 OPTIONAL, nr-RSCPD-r18 INTEGER (0..3599) OPTIONAL, nr-PhaseQuality-r18 NR-PhaseQuality-r18 OPTIONAL, nr-RSCPD-AdditionalMeasurementsAddSamples-r18 SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF NR-RSCPD-AdditionalMeasurementSamplesElement-r18 OPTIONAL, nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 ENUMERATED { singleHop, multipleHop } OPTIONAL ]] } NR-RSCPD-AdditionalMeasurementSamplesElement-r18 ::= SEQUENCE { nr-RSCPD-r18 INTEGER (0..3599) OPTIONAL, nr-PhaseQuality-r18 NR-PhaseQuality-r18 OPTIONAL, nr-TimeStamp-r18 NR-TimeStamp-r16 OPTIONAL, ... } -- ASN1STOP nr-AggregatedDL-PRS-ResourceSetID-List This field provides the DL-PRS Resource Set IDs which are used for the aggregated RSTD, RSRP, or RSRPP measurement results. This field is optionally present if the field nr-MeasBasedOnAggregatedResources is present; otherwise, it is not present. If the field is present, the field nr-DL-PRS-ResourceSetID should be included, and the nr-DL-PRS-ResourceID-r16 is taken as one of aggregated PRS resource ID, and the dl-PRS-ID in IE NR-DL-TDOA-MeasElement shall be ignored by a receiver..

- Proposal 1: To achieve using the existing PRS resource ID for PRS aggregation, RAN2 to adopt the following LPP changes:

- Change nr-AggregatedDL-PRS-ResourceSetID-List-r18 from (SIZE (2.. 3)) to (SIZE (1.. 2));

- Change field description in nr-AggregatedDL-PRS-ResourceSetID-List-r18 to ‘if the field is present, the nr-DL-PRS-ResourceSetID-r16 should be present, and the nr-DL-PRS-ResourceID-r16 is taken as one of aggregated PRS resource ID’.

- Adopt the corresponding TP.

- Conclusion

In this contribution, we propose the following proposal:

- Proposal 1: To achieve using the existing PRS resource ID for PRS aggregation, RAN2 to adopt the following LPP changes:

- Change nr-AggregatedDL-PRS-ResourceSetID-List-r18 from (SIZE (2.. 3)) to (SIZE (1.. 2));

- Change field description in nr-AggregatedDL-PRS-ResourceSetID-List-r18 to ‘if the field is present, the nr-DL-PRS-ResourceSetID-r16 should be present, and the nr-DL-PRS-ResourceID-r16 is taken as one of aggregated PRS resource ID’.

- Adopt the corresponding TP.

- References

- R1-2403728, LS on PRS resource ID for bandwidth aggregation, RAN1, 2024-04

- R2-2403818, Corrections to TS 37.355, CATT, 2024-04