3GPP TSG-RAN2 Meeting #126	R2-2404709

Fukuoka, Japan May 22nd – 26th, 2024

Agenda Item:	7.5.3.1

Source: 	Xiaomi Communications, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Title:  	Discussing on issues on DSR and proposed TP to MAC

Document for:	Discussion and Decision

Introduction

In this contribution, we provide some views on the remaining issues on DSR.

Discussion

SDUs associated with a DSR is defined in MAC to help describe cancelation of a pending DSR, which is performed when the SDUs are either transmitted or discarded. Currently, the procedure is captured as below [1]:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and it is associated with the LCH which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Obviously, it implies that only the SDUs whose discardTimer drops below the threshold cancel the associated DSR when they are either transmitted or discarded. As agreed, DSR should report data volume of the packets which are delay-critical, i.e. if they belong to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold for the pdu-SetDiscard configured case, even if their respective discardTimers do not reach the threshold as shown below in TS38.323:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

The current writing in MAC limits the association to only those SDUs whose PDCP discard timers are below the remaining time threshold. Thus, potentially some delay critical data may not be scheduled on time.

Since gNB should issue UE UL grants by taking all delay critical data into consideration, it is more reasonable that UE cancels the DSR when all delay critical data are transmitted or discarded. We do not think we need to have different criteria for the packets for DSR reporting and DSR cancelling.

Since delay-critical PDCP SDU has been defined in TS38.323, we can generally refer to TS38.323. Hence, we propose:

Proposal 1.   A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS38.323) associated with the LCH which triggered the DSR.

Proposal 2.   Consider adopting the TP in Annex in the next MAC CR for DSR.

Conclusions

Based on the discussion, our proposals are provided as follows:

Proposal 1.   A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS38.323) associated with the LCH which triggered the DSR.

Proposal 2.   Consider adopting the TP in Annex in the next MAC CR for DSR.

References

R2-2404172 [POST125bis][015][XR] CR to 321 (Qualcomm)

Annex for DSR remaining issues:

Start of changes

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS38.323) associated with the LCH which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE: 	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

End of changes