3GPP TSG-RAN WG2 Meeting #126	R2-2404745

Fukuoka, Japan May 22nd – 26th, 2024

Source:        ZTE Corporation, Sanechips

Title:           Consideration on the Reconfiguration Failure Processing when T348 is Running

Agenda item:    7.17.2

Document for:   Discussion and Decision

- Introduction

In the last meeting, a Note was agreed to the Reconfiguration failure case for the MUSIM operation [1].

##### 5.3.5.8.2 Inability to comply with RRCReconfiguration /*********************omit the unrelated part*******************************************/ NOTE 1: The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message. NOTE 2: If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction. For such case, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.3.7 and still considers the configuration resulting from RRCReconfiguration message as the current configuration as the baseline for delta configuration for future reconfigurations.

During the online discussion, whether to stop the T348 is also discussed and the conclusion is that the T348 would not stop for this case. In this paper, we’d like to share our further thinking on this issue.

- Discussion

In this chapter, we first give a Figure to describe the related procedure, then analyse the potential issues, at last we provide the solution together with TP.

For the UAI, the general rule is that it depends on the NW implementation on whether/how to response the UAI info. As shown in the Fig 1, the NW may send a reconfiguration message that doesn't comply with the UE’s temporary capability restriction.

Figure 1: Reconfiguration Failure Processing when T348 is Running with Note

Then according to the above Note, the UE does not apply failure handling and UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.3.7. However, in the chapter 5.3.7, the reactive temporary capability restriction can only be sent when the timer T348 is not running, which may lead to dozens of milliseconds mismatch between the UE and the network side.

1> if configured to provide MUSIM assistance information for temporary capability restriction: 2> if the UE has temporary capability restriction on the current configuration and timer T348 is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList; 3> start the timer T348 with the timer value set to the musim-WaitTimer.

musim-WaitTimer-r18 ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1}

Observation 1: There may be dozens of milliseconds mismatch between the UE and the network side if the UE does not apply the reconfiguration failure handling for the T348 is running case.

To solve the issue in the Observation 1, the simple way would be not to apply the Note for the reconfiguration when the T348 is running, and the wording can be simple as below:

NOTE 2: If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction and T348 is not running. For such case, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.3.7 and still considers the configuration resulting from RRCReconfiguration message as the current configuration as the baseline for delta configuration for future reconfigurations.

This modification was also included in the TP of Annex. With this TP, the procedure would be changed as in the Fig 2.

Figure 2: Normal Reconfiguration Failure Processing when T348 is Running

As shown in the Fig2, the UE would trigger re-establish procedure and the UE sync up with network again when receives the reconfiguration with limited configuration. Furthermore according to the T348 stop condition, the UE would stop timer when receiving a reconfiguration with limited configuration and can resend the UAI if needed.

| T348 | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for serving cell(s) with capabilities restricted, release of SCell or PSCell or release of SCG. | Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction transmitted via musim-CapRestriction. | UE may apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction. UE may apply the temporary capability restriction that SCG is not supported if ServCellIndex of PSCell was included in indicated MUSIM-CellToRelease-r18, UE releases SCG. |
| --- | --- | --- | --- |

Proposal 1: When the T348 is running, the UE shall apply the normal reconfiguration failure processing and thus agree the TP of Annex.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: There may be dozens of milliseconds mismatch between the UE and the network side if the UE does not apply the reconfiguration failure handling for the T348 is running case.

Proposal 1: When the T348 is running, the UE shall apply the normal reconfiguration failure processing and thus agree the TP of Annex.

- Reference

[1] R2-2404000  Correction on NR MUSIM enhancements  Vivo

- Annex

5.3.5.8.2	Inability to comply with RRCReconfiguration

/*********************omit the unrelated part*******************************************/

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the case that the UE is configured to provide MUSIM assistance information for temporary capability restriction and T348 is not running. For such case, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.3.7 and still considers the configuration resulting from RRCReconfiguration message as the current configuration as the baseline for delta configuration for future reconfigurations.