3GPP TSG-RAN WG2 Meeting #126	R2-2404766

# Fukuoka, Japan, 20th - 24th May., 2024

Title:		Remaining issues for R18 POS stage2 TS 38.305

Source:	Huawei, HiSilicon

Agenda item:	7.2.6

Document for:	Discussion and Decision

- Background

This paper discusses the remaining RAN2 issues in the stage2 spec TS 38.305 for R18 positioning

- Discussions

In the previous RAN3 meetings,stage2 for LPHAP has been agreed and added to the current RAN2 spec TS 38.305. While in the current RRC spec, 3 cases have been defined for the transmission of RRC resume request for SRS configuration activation/configuration request:

- Event triggered when posSRS is pre-configured in RRC_INACTIVE

- When positioning SRS tarnsmission is on-going in RRC_INACTIVE:

  - The UE moves from one validity area to another validity area

  - The UE moves out of validity area

Proposal1: Add the following descriptions for RAN stage2 spec for LPHAP with pre-configured SRS when the UE sends RRC resume request. Adopt the TP in Annex A

- Event triggered when posSRS is pre-configured in RRC_INACTIVE

- When positioning SRS tarnsmission is on-going in RRC_INACTIVE:

  - The UE moves from one validity area to another validity area

  - The UE moves out of validity area

- Conclusions:

We propose the following for the description in TS 38.300

Proposal1: Add the following descriptions for RAN stage2 spec for LPHAP with pre-configured SRS when the UE sends RRC resume request. Adopt the TP in Annex A

- Event triggered when posSRS is pre-configured in RRC_INACTIVE

- When positioning SRS tarnsmission is on-going in RRC_INACTIVE:

  - The UE moves from one validity area to another validity area

  - The UE moves out of validity area

- Annex: Text Proposal

3GPP TSG-RAN WG2 Meeting #126	R2-240

# Fukuoka, Japan, 20th - 24th May., 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR |  | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38305 for R18 positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1/ Change to LPHAP on addition of the description of SRS configuration request and activation request from the UE side on top of what has been agreed in RAN3 to make the description complete 2/ Some editorial changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are made per above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage2 description for LPHAP will not be accurate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================================CHANGE BEGIN====================================

7	General NG-RAN UE Positioning procedures

7.14	Procedures for Area-specific SRS Configuration

7.14.1	General

To support Low Power and High Accuracy Positioning (LPHAP) as defined in TS 23.273 [35], area-specific SRS configuration is used to enable SRS transmission by the RRC_INACTIVE UE, within positioning validity cell list(s).

The UE can be pre-configured with are-specific SRS as in 7.14.2. When positioning event is triggered, if the UE's camped cell is within the pre-configured validity area, the UE sends SRS activation request as in TS 38.331 [14]; when there is an on-going positioning SRS transmission in RRC_INACTIVE, if the UE moves from one validity area to another validity area, the UE sends SRS activation request as in TS 38.331 [14]; when there is an on-going positioning SRS transmission in RRC_INACTIVE, if the camped cell is out of the pre-configured validity area, the UE sends SRS configuration update request to gNB for update of SRS configuration, as in clause 7.14.3.

7.14.2	Area-specific SRS (Pre-)configuration Allocation Procedure

Figure 7.14.2-1 shows the Area-specific SRS (Pre-)configuration Allocation procedure.

Figure 7.14.2-1: Area-specific SRS (Pre-)configuration Allocation Procedure

1.	The LMF sends the NRPPa Positioning Information Request message to the serving gNB of the UE for Area-specific SRS (Pre-)configuration allocation. In case of Area-specific SRS configuration allocation, the LMF includes the Requested SRS Transmission Characteristics including an associated Positioning Validity Area Cell List. In case of Area-specific SRS pre-configuration allocation, the LMF includes a list of Requested SRS Transmission Characteristics, each with the associated Positioning Validity Area Cell List.

2.	The serving gNB allocates the area-specific SRS resources, and moves the UE to RRC_INACTIVE state by sending the RRC Release message, which includes the area-specific SRS (pre-)configuration(s).

3.	The serving gNB responds with the NRPPa Positioning Information Response message to the LMF, including one or more SRS configuration(s), each with the associated Positioning Validity Area Cell List.

4.	The LMF notifies the gNBs within the positioning validity area(s) to reserve the SRS resources.

NOTE:	Step 4 may occur prior to any of steps 1 through 3.

7.14.3	Area-specific SRS Configuration Update Procedure

Figure 7.14.3-1 shows the Area-specific SRS Configuration Update procedure.

Figure 7.14.3-1: Area-specific SRS Configuration Update Procedure

0.	The UE in RRC_INACTIVE state is configured with an area-specific SRS configuration and reselects to a cell that is not included in the Validity Area Cell List.

1.	The UE sends the RRC Resume Request message with the resume cause "srs-PosConfigOrActivationReq" to request for new SRS configuration.

2.	The receiving gNB which receives the request from the UE triggers the Retrieve UE Context procedure towards the last serving gNB.

3.	The last serving gNB sends the NRPPa Positioning Information Update message to notify the LMF the UE moved out of the validity area by providing the Cell ID of the receiving gNB where the UE resumes at.

4.	The last serving gNB relocates the full UE context to the receiving gNB.

5.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

6.	The AMF responds with the NGAP Path Switch Request Acknowledge message.

7.	The LMF requests the receiving gNB to allocate a new SRS configuration for the UE. If the Positioning Validity Area Cell List is included in the Requested SRS Transmission Characteristics, the Area-specific SRS Configuration Allocation procedure as specified in section 7.14.2 is applied. Otherwise, the legacy SRS allocation procedure is applied.

8.	The receiving gNB indicates to the last serving gNB to release the UE context.

==================================CHANGE ENDS=======================================