3GPP TSG-RAN WG2 Meeting #126	 R2-2404773

Fukuoka City, Japan, 20th May – 24th May, 2024

Agenda Item:		7.5.3.3

Source:			Huawei, HiSilicon

Title:				Discussion on MAC procedure for UTO-UCI

Document for:	Discussion and Decision

- Introduction

RAN2 has discussed that the MAC entity should determine the content of UTO-UCI based on at least the amount of buffered data from the LCH(s) in the previous meeting. In this paper, we discuss detailed impacts for determining UTO-UCI.

- Discussion on UTO-UCI

Data volume calculation for determining UTO-UCI

RAN2 has made the agreement that the UE determines the unused CG occasion based on at least the amount of buffered data from the LCH(s) in #125-bis meeting [1] and that the UE should determine whether a CG occasion PUSCH transmission should be used or not before PUSCH transmission, as given below:

Agreements on UTO-UCI 1 Remove the restriction of only considering the already buffered data in UE’s determination of unused CG occasions. 2 Add a clarification to the current spec that before PUSCH transmission over a CG occasion, UE should have determined whether a CG occasion within the subsequent nrofBitsInUTO-UCI valid occasions is going to be used for PUSCH transmission or not.

Hence, the enhancement of supporting UTO-UCI for determining CG occasion was introduced [2]:

For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI .

However, the procedure on how the MAC entity determines the content of UTO-UCI in the current specification remains unclear. For example, the MAC entity shall not determine the UTO-UCI that will be sent excessively earlier than the time point of the transmission occasion, due to its inaccuracy of predicted data volume. Since the UTO-UCI is determined by considering at least the buffered data volume, it is beneficial that the MAC entity should determine the content of UTO-UCI at a given point in time for the specific UTO-UCI with the CG occasion it is multiplexed that is being assembled, as shown in the annexed TP.

Proposal1: The MAC entity shall determine UTO-UCI at MAC PDU assembly.

- Conclusions

In this paper, we provide our view that the MAC entity should determine the UTO-UCI for the CG occasion that is being assembled.

The following proposal is given:

Proposal1: The MAC entity shall determine UTO-UCI at MAC PDU assembly.

# 4	References

[1] Chair note of RAN2#124,

https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_124/Inbox/Chair_Notes/R2_124_ChairNotes_23-11-17_final.docx

[2] R2-2404172 Corrections for XR services

- Annex A: TP for TS 38.321

CHANGES START

- 5.8.2 Uplink For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources, when assembling the MAC PDU for a transmission occasion of a configured grant configuration that is configured to report UTO-UCI. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI .

NEXT CHANGE