3GPP TSG-RAN WG2 Meeting #126	R2-2405046

Fukuoka, Japan, May 22nd – 26th, 2024

Agenda Item:	7.3.3

Source:	OPPO

Title:	Discussion on the TRS meas capability for intra-band SSB-less CA

Document for:	Discussion, Decision

Introduction

This paper will discuss the issue related to TRS meas capability for intra-band SSB-less CA.

Discussion

RAN2#125bis discusses how to implement the UE capability for A-TRS-based SSB-less SCell activation based on RAN4’s agreement. As only the case of A-TRS based inter-band SSB-less SCell activation is concluded in RAN4, RAN2 leaves an FFS on the case of A-TRS based intra-band SSB-less SCell activation.

Missing CSI-RS and TRS meas capability for inter-band SSB-less CA R2-2402854 Remaining control plane open issues on NES Apple discussion Rel-18 Netw_Energy_NR-Core Proposal 1: Introduce a separate capability of CSI-RSRP and CSI-RSRQ measurements in inter-band SSB-less CA. - Vivo thinks we would need two other agreements if we go with legacy capability so we should have separate capability. - Huawei interband SSB-less there is no requirement for measurement so there is no UE capability needed. => Not agreed Proposal 3: In the field description of capability aperiodicCSI-RS-FastScellActivation-r17, clarify that it is applied to both intra-band SSB-less SCell activation and inter-band SSB-less SCell activation. => Noted Agreements on UE capabilities => In the field description of capability aperiodicCSI-RS-FastScellActivation-r17, clarify that it is applied to inter-band SSB-less SCell activation. FFS whether it is also applied to intra-band SSB-less SCell activation

We believe the answer to this issue depends on whether to extend SSB-less SCell activation requirements of inter-band CA to the case of intra-band non-contiguous CA (NCCA). The related discussion continues with two RAN4 meetings without consensus.

RAN4#110 R4-2403526 Issue 1-3-1: Intra-band NCCA Agreement: Regarding FR1 intra-band NCCA with SSB-less operation, further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA. The study does not impact the completion timeline of the WI. RAN4#110bis R4-2406294 R4-2406294 Ad-hoc minutes on RRM requirements for Netw_Energy_NR Type: other For: Approval Source: Huawei Decision: Approved. Issue 1-1-5: Intra-band non-contiguous CA Moderator: No consensus on UE assumptions regarding single or separate RF chain. Further discuss in this meeting, and check how to move forward in Ad-hoc return to. Intel: the issue has been discussed for quite a long time. This is clear commercial need. Session Chair: Let us conclude the issue in the next meeting.

Per the reasons mentioned below, we feel reluctant to extend SSB-less SCell activation requirements of inter-band CA to the case of intra-band non-contiguous CA (NCCA). Accordingly, further discussion in RAN2 would be avoided (e.g. on how to capture the related UE capability for intra-band SSB-less SCell activation).

- The discussion is clearly out of the scope of WID.

In the WID, the SSB-less operation focuses on the case of inter-band CA.

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

- The assumption at the UE side would be different between inter-band and intra-band cases.

For SSB-less SCell inter-band CA, the basic assumption is the UE has two separate RX chains. Therefore, independent AGC, T/F tracking was possible for the inter-band CA scenario. While, for the intra-band CA, two separate RX chains may not always apply based on the current discussion status in RAN4. Thus, it may be hard to directly reuse the conclusions for inter-band SSB-less SCell activation to intra-band SSB-less SCell activation.

Not consider the case of TRS-based intra-band non-contiguous SSB-less SCell activation in Rel-18 NES.

Conclusion

We have the following proposals:

Proposal 1	Not consider the case of TRS-based intra-band non-contiguous SSB-less SCell activation in Rel-18 NES.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR