3GPP TSG RAN WG2 Meeting #126	 R2-2405272

Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item:	8.1.2.1

Source:	Futurewei

Title:	Discussion on LCM for NW-Side Models

Document for:	Discussion

# Introduction

During RAN2 meeting #125bis, functionality-based LCM for NW-side models was discussed and some agreements have been achieved[3].

On the general aspect, RAN2 confirmed that UE will not involve in the decision-making process of LCM for NW-side models, except for data collection procedure for NW-sided model LCM.

Agreements

- RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

- RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

- RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

For beam management and positioning use cases, the following agreements were achieved.

Agreements

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training.

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed.

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

- FFS whether there is specification impact associated to gNB-side model monitoring.

- For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs.

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

Based on the above agreements, we will further discuss the details of LCM for NW-side models, including procedures and message/signaling exchanges.

# LCM for functionality and model selection, activation, deactivation, switching, and fallback

## Background

In the TR [2], the scope of LCM operations includes the following.

-	Functionality/model selection, activation, deactivation, switching, and fallback operation.

-	Including: Decision by the network (either network initiated or UE-initiated and requested to the network), decision by the UE (event-triggered as configured by the network, UE’s decision reported to the network, or UE-autonomous either with UE’s decision reported to the network or without it)

In the TR [2], the case where the LCM decision is taken and initiated by the network is depicted in Figure 7.2.1.1-1, which is quoted below.

- Decision by the network

  - Network-initiated

Figure 7.2.1.1-1(in [2]): Network decision, network-initiated AI/ML management

Note: The Management Instruction may be a result of model/functionality performance monitoring at the network.

Note: The Management Instruction may include information about the model or functionality.

Note that in Figure 7.2.1.1-1 of [2], the Management Instruction from NW to UE is not applicable for NW-side model, because model inference (and therefore LCM operations) is done at the network side.

The case where the LCM decision is taken by the network while the request is initiated by the UE is depicted in Figure 7.2.1.1-2 in [2], which is quoted below.

- Decision by the network

  - UE-initiated and requested to the network

Figure 7.2.1.1-2 (in [2]): Network decision, UE-initiated AI/ML management

Note: The Management Request may be a result of model/functionality monitoring at the UE.

Note: In response to the Management Request, the network may send a Management Instruction to the UE.

Note: The Management Request may include information about the model or functionality.

Note: The network may accept or reject the Management Request from the UE.

Note: The Management Request may include information related to model/functionality performance metrics.

Note: The Management Instruction may include information about the model or functionality.

Note that in Figure 7.2.1.1-2 of [2], the Management Instruction from NW to UE is also not applicable for NW-side model, because model inference (and therefore all LCM operations) is done at the network side.

Therefore, for NW-side model, there are two possible ways of implementations as indicated in the TR.

- Decision by the network

  - Network initiated

  - UE-initiated and requested to the network

- Decision by the UE

  - Event-triggered as configured by the network,

  - UE’s decision reported to the network, or

  - UE-autonomous either with UE’s decision reported to the network or without it.

Now, recall that Meeting #125bis has made the following agreements[3].

Agreements

- RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

- RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

- RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

Based on these agreements, it is clear that not all these solutions are feasible for NW-side models. We analyze NW-side model operations for each type/sub-type in the following table. Note that because the necessity of model identification and its corresponding model-ID-based LCM has not been determined (currently under discussion in RAN1), we only discuss functionality-based LCM in this contribution.

Table 1. LCM types and sub-types for network-side model operations

| LCM Type | LCM Sub-type | NW-side Model Operation |
| --- | --- | --- |
| Decision by the NW | Network initiated | In this case the operation is initiated by the NW and decided by the NW for NW-side model (everything is done at the NW side). The recent agreement confirmed that the UE only involves in measurement and reporting (e.g., measuring performance data and sending it to the NW). The following summarizes possible message exchanges for this sub-type. - NW sends measurement/reporting configurations to the UE. - UE sends measurement reports to NW. - NW makes decision and execute the operation. |
|  | UE-initiated and requested to the network | In this case the operation is initiated by the UE, and the request of operation is sent to the NW, for NW-side model. The UE is not actively involved except that the UE may need to receive measurement configurations from the NW and send the measurement results back to the NW. The following are the potential message exchanges for this sub-type. - NW sends measurement/reporting/event-trigger configurations to the UE. - UE sends measurement reports/triggered events to NW. - NW makes decision and execute the operation. |
| Decision by the UE | Event-triggered as configured by the network | This LCM type/sub-type does not apply to NW-side functionality; it is for UE-side functionality only. It makes no sense to allow UE to dictate NW-side operations for a NW-side model. |
|  | UE’s decision reported to the network |  |
|  | UE-autonomous either with UE’s decision reported to the network or without it |  |

As we can see from the analysis in the table, for NW-side functionality, LCM decisions made by the UE isn’t a valid option. Therefore, the LCM type of Decision by the UE is not discussed here in this contribution.

## Need for signaling/message

From the two figures and Table 1 above, we can see the following signaling needs for NW-side models.

From NW to UE:

- NW sends LCM-related configuration to the UE (e.g., measurement, reporting, or triggering events)

From UE to NW:

- UE sends measurement reports/performance results/triggering events to NW.

We hence have the following proposal. Note that there may be some optional signaling exchanges, for example, in the case UE-initiated the operations, NW may want to feedback to UE the result of the operation. To minimize the spec impacts, we only list essential message/signaling needs in this TDoc.

Proposal 1: Design the following messages for NW-side LCM operations.

- From NW to UE:

  - NW sends LCM-related configuration to the UE (e.g., measurement, reporting)

- From UE to NW:

  - UE reports measurement/performance results to NW.

## Operation Procedures

We can also see that for NW-side model, when the LCM decision is made by the NW, the only LCM sub-types that need signaling/protocol support are the cases where

- the LCM decision is made at the NW and the operation is done also by the NW.

- the LCM operation is initiated by the UE and the request is sent to the NW.

For these two cases, we have the following proposals for operating procedures.

Proposal 2: When the decision is made by the NW and the operation is also done at the NW, for NW-side model, the functionality-based LCM involves following procedures.

- NW sends measurement/reporting configurations to the UE.

- UE sends measurement reports/performance results to NW.

- NW makes decision and execute the operation.

Proposal 3: When the operation is initiated by the UE, and the decision is made by the NW, for NW-side model, the functionality-based LCM involves following procedures.

- NW sends measurement/reporting/event-trigger configurations to the UE.

- UE sends measurement reports/triggered events to NW.

- NW makes decision and execute the operation.

Note that by the agreement made of meeting #125bis, the UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

Applicable Operations for NW-side Model

Because at functionality level we do not need to directly control/select the model(s) to be used, for each functionality, there are only two solutions: the non-AI/ML based, i.e., the existing solution, and the AI/ML-based solution.

With this in mind, we look at the types of operations that are applicable for NW-side models for functionality-based LCM. Because we assume R19 only supports functionality-based LCM (for now, pending RAN1 decision on the necessity of model identification), the scope of the LCM operations will be limited to only the following.

- Selection: only between existing and AI/ML-based solution

- Activation: only to activate the AI/ML-based solution

- Deactivation: only to deactivate the AI/ML-based solution

- Switching: only between existing and AI/ML-based solution

- Fallback: only to the existing solution

If we further examine these operations, we can see some operations overlap in their functionalities. For example, the operation of Selection (selecting a solution to work) can also be done by Activation of the other solution. The operation of switching can be replaced by deactivating one solution and activating another solution. The operation of deactivation means it must fall back to the traditional approach. After removing these duplicate operations, we think for functionality-based LCM, we only need to support two LCM operations, activation, and fallback.

Proposal 4: For functionality-based LCM, the supported operations can be limited to activation and fallback.

# LCM for model training, inference, performance monitoring, and data collection

In this section, the necessity for high-level signaling/mechanism will be discussed for model training, inference, performance monitoring, and data collection.

## LCM for model training

For model training, because this is for NW-side model only (the support of two-sided models has not been decided, i.e., the support of CSI compression sub use case), the training does not involve collaboration between the two entities (i.e., the NW and the UE). Therefore, we don’t think any signaling/mechanism is needed to support model training for the NW-side model. Note data collection for model training will be discussed in its own agenda item.

Proposal 5: No LCM signaling/mechanism is needed to support model training for the NW-side model.

## LCM for model inference

For NW-side models, model inference is done at the NW-side. In this case, UE’s involvement includes the following two aspects.

- The UE side may generate and supply input data/measurement for model inference, upon the configuration from the NW side.

- The UE side obtains inference output data for performance monitoring or other purposes.

In both aspects (input and output), data transmission is use-case-dependent so the details should be handled by use cases.

Beam Management

For the use case of beam management, it was agreed in RAN2 meeting #125bis, “There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.”

However, based on the TR [2], standard impacts on the physical aspect include the following (Section 7.1.3).

- Enhanced or new configurations/UE reporting/UE measurement, e.g., enhanced or new beam measurement and/or beam reporting

- Enhanced or new signalling for measurement configuration/triggering

- Signalling of assistance information (if applicable)

Also, in meeting #125bis[3], it was agreed that

For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed.

Regarding the enhancement of UE measurement/reporting, for BM-Case1 and BM-Case2 with a NW-side AI/ML model, one specific requirement is that the current L1 beam reporting needs to be enhanced for AI/ML model inference to allow “UE to report the measurement results of more than 4 beams in one reporting instance”.

Based on the discussion above, it is too early to say that “There is no specification impact associated to gNB-side model inference…”, as we can see that the reporting function needs to be enhanced to accommodate more than 4 beams.

Regarding assistance information, although in R18 the group was not able to reach consensus, the following explicit assistance information from UE to NW for NW-side model has been discussed.

- UE location

- UE moving direction

- UE Rx beam shape/direction

Observation 1: For BM-Case1 and BM-Case2 with a NW-side AI/ML model, one specific requirement for spec enhancement is that the current L1 beam reporting needs to be enhanced to allow “UE to report the measurement results of more than 4 beams in one reporting instance”. The following agreement reached in RAN2 meeting #125bis conflicts with this requirement.

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

Proposal 6: Revisit the following agreement achieved in RAN2 meeting #125bis and double-check its correctness.

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

Proposal 7: For beam management use cases, the L1 measurement framework needs to be enhanced to report the measurement results of more than 4 beams in one reporting instance.

Positioning Accuracy

For positioning use cases, the NW side entities could be either gNB or LMF. Therefore, UE may generate necessary input data while the termination point for the input data depends on where the inference process is performed, either within the gNB or the LMF.

Other than this, the potential standard impacts of the Positioning use case are similar to the Beam Management use case.

Alignment of Additional Conditions

Regarding alignment of additional conditions between model training and model inference, because it is not practical to allow models trained at the UE-side to perform inference at the NW, we don’t think there is a need to support signaling/mechanisms for additional condition alignment.

Based on the above discussion, we have the following proposal.

Proposal 8: For model inference at the NW side, support the following signalling/mechanisms.

- NW to configure UE for supplying input data for model inference.

- UE to send measurement results/reports to NW for model inference, which could optionally include assistance information.

The details should be discussed use case by use case.

## LCM for performance monitoring

Based on the TR [2], there are multiple performance monitoring solutions, as quoted below.

The following metrics/methods for AI/ML model monitoring in lifecycle management per use case are considered:

-	Monitoring based on inference accuracy, including metrics related to intermediate KPIs

-	Monitoring based on system performance, including metrics related to system performance KPIs

-	Other monitoring solutions, at least the following 2 options.

-	Monitoring based on data distribution

-	Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or SNR, delay spread, etc.

-	Output-based: e.g., drift detection of output data

-	Monitoring based on applicable condition

Note:	Monitoring metric calculation may be done at NW or UE

For performance monitoring, the first thing the two sides need to communicate about is the method the NW adopted for performance monitoring; it could be based on either inference accuracy, or system performance, or other monitoring solutions listed above.

After the adopted method for performance monitoring has been aligned, the next thing to be aligned/configured includes the KPIs each method uses to gauge the performance of either the model or the system, which are use case dependent. To do this, the NW sends configuration to the UE and UE sends corresponding measurement reports back to the NW. The benchmark/reference for the performance comparison may also need to be communicated between the two sides.

Take beam management as an example, performance metric(s) for BM-Case 1 and BM-Case 2 have the following alternatives:

- Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

- Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

- Alt.3: Performance metric based on input/output data distribution of AI/ML

- Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

For beam management, benchmark/reference for the performance comparison include:

- Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A)

- Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams)

The use case of Positioning Accuracy Enhancement has the similar requirements for signalling/mechanisms.

NW may also configure/request UE for assistance information, if supported (Section 7.1.3 of [2]).

Note that according to the TR, performance of inactive functionalities may also be monitored, even though the TR does not provide a mechanism of doing it (only some examples), as quoted below.

Methods to assess/monitor the applicability and expected performance of an inactive model/functionality, including the following examples for the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities (if applicable):

-	Assessment/Monitoring based on the additional conditions associated with the model/functionality

-	Assessment/Monitoring based on input/output data distribution

-	Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy

-	Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs)

For NW-side models, even if performance monitoring of inactive models is supported, it will be running within the NW, so UE does not need to involve, except providing NW the measurement/reference/ benchmark reports for performance monitoring and comparison, which is the same signalling/mechanism used for active model (does not have different requirements for signalling or mechanism).

In summary, for NW-side models, the following signalling/mechanisms are needed for performance monitoring.

Proposal 9: For performance monitoring at the NW side, support the following signalling/mechanisms.

- NW communicates with UE about adopted monitoring approaches.

- NW configures UE for performance KPI measurement and reporting, including benchmark/reference for the performance comparison.

- UE sends measurement/reference/benchmark reports to the NW for performance monitoring and comparison.

- NW configures/requests UE for assistance information, if supported.

- UE measures/reports to NW the assistance information, if supported.

## LCM for data collection

Data collection is a function that provides input data to model training, inference, and monitoring functions. According to the TR, data collection may also be performed for different purposes in LCM, e.g., model selection, model update, etc.; each may be done with different requirements and potential specification impact.

For data collection, the following aspects need to be considered (Section 4.2.5 in [2]).

- Measurement configuration and reporting

- Contents, type and format of data

- Signalling of assistance information for categorizing the data

- Signalling for data collection procedure

Note, in this contribution, we focus only on the signalling and protocol aspects. The mechanism of data collection will be discussed in their own agenda items (i.e., AI 8.1.3 and AI 8.1.4).

For the BM use cases, the spec impacts include the following for NW-side models [2].

- Mechanism related to the reporting.

- Additional information for content of the reporting.

- Reporting overhead reduction approaches (e.g., omission/selection/compression of collected data).

- Signalling/configuration/measurement/report for data collection, e.g., signalling aspects related to assistance information (if supported), Reference signals.

For Positioning use cases, the TR identified the following information with potential spec impacts [2].

- Ground-truth label

- Measurement (corresponding to model input)

- Quality indicator

- RS configuration(s)

- Time stamp

- Details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data

- Assistance signalling indicating reference signal configuration(s) to derive label and/or other training data

- Request/report of training data: Ground-truth label; Measurement corresponding to model input; Associated information of ground-truth label and/or measurement corresponding to model input

- Assistance signalling and procedure to facilitate generating training data: Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier; Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc.

Although these two use cases have different requirements/needs for signalling and protocols, they can be summarized in the categories below, for NW-side models.

- NW sends measurement configurations to UE, including information related to both ground-truth and model input.

  - May also include other configurations such as supported overhead reduction approaches.

- UE sends collected data to NW based on NW configuration.

  - May also include other information such as selected overhead reduction approaches.

- NW sends request for assistance information/additional conditions to UE, if supported.

- UE sends applicable assistance information/additional conditions to NW, if supported.

Based on the discussion above, we have the following proposal for data collection for NW-side models.

Proposal 10: For data collection for NW-side models, support the following signalling/mechanisms.

- NW sends measurement configurations to UE, including information related to both ground-truth and model input.

  - May also include other configurations such as supported overhead reduction approaches.

- UE sends collected data to NW based on NW configuration.

  - May also include other information such as selected overhead reduction approaches.

- NW sends request for assistance information/additional conditions to UE, if supported.

- UE sends applicable assistance information/additional conditions to NW, if supported.

# Conclusions

In this contribution, we discussed our views on LCM for NW-side models. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Design the following messages for NW-side LCM operations.

- From NW to UE:

  - NW sends LCM-related configuration to the UE (e.g., measurement, reporting)

- From UE to NW:

  - UE reports measurement/performance results to NW.

Proposal 2: When the decision is made by the NW and the operation is also done at the NW, for NW-side model, the functionality-based LCM involves following procedures.

- NW sends measurement/reporting configurations to the UE.

- UE sends measurement reports/performance results to NW.

- NW makes decision and execute the operation.

Proposal 3: When the operation is initiated by the UE, and the decision is made by the NW, for NW-side model, the functionality-based LCM involves following procedures.

- NW sends measurement/reporting/event-trigger configurations to the UE.

- UE sends measurement reports/triggered events to NW.

- NW makes decision and execute the operation.

Proposal 4: For functionality-based LCM, the supported operations can be limited to activation and fallback.

Proposal 5: No LCM signaling/mechanism is needed to support model training for the NW-side model.

Observation 1: For BM-Case1 and BM-Case2 with a NW-side AI/ML model, one specific requirement for spec enhancement is that the current L1 beam reporting needs to be enhanced to allow “UE to report the measurement results of more than 4 beams in one reporting instance”. The following agreement reached in RAN2 meeting #125bis conflicts with this requirement.

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

Proposal 6: Revisit the following agreement achieved in RAN2 meeting #125bis and double-check its correctness.

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

Proposal 7: For beam management use cases, the L1 measurement framework needs to be enhanced to report the measurement results of more than 4 beams in one reporting instance.

Proposal 8: For model inference at the NW side, support the following signalling/mechanisms.

- NW to configure UE for supplying input data for model inference.

- UE to send measurement results/reports to NW for model inference, which could optionally include assistance information.

The details should be discussed use case by use case.

Proposal 9: For performance monitoring at the NW side, support the following signalling/mechanisms.

- NW communicates with UE about adopted monitoring approaches.

- NW configures UE for performance KPI measurement and reporting, including benchmark/reference for the performance comparison.

- UE sends measurement/reference/benchmark reports to the NW for performance monitoring and comparison.

- NW configures/requests UE for assistance information, if supported.

- UE measures/reports to NW the assistance information, if supported.

Proposal 10: For data collection for NW-side models, support the following signalling/mechanisms.

- NW sends measurement configurations to UE, including information related to both ground-truth and model input.

  - May also include other configurations such as supported overhead reduction approaches.

- UE sends collected data to NW based on NW configuration.

  - May also include other information such as selected overhead reduction approaches.

- NW sends request for assistance information/additional conditions to UE, if supported.

- UE sends applicable assistance information/additional conditions to NW, if supported.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

R1-2312764, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v1.3.0 (2023-11)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004