3GPP TSG-RAN2 #126	R2-2405292

# Fukuoka, Japan May 20th – 26th, 2024

Agenda item:		7.3.2 (Netw_Energy_NR-Core)

Source:	LG Electronics Inc.

Title: 	Coexistence of Cell DTX/DRX mechanism and RACH-less LTM

Document for:	Discussion and Decision

1.	Introduction

This document discusses coexistence of Cell DTX/DRX mechanism and RACH-less LTM. During R2-125bis meeting, it was discussed whether to support coexistence of Cell DTX/DRX mechanism and RACH-less LTM or not. However, it was not concluded and remained FFS as the following agreement of R2-125bis meeting.

Agreements

…

4	FFS if we will support the coexistence of the cell DTX/DRX and RACH-less LTM and whether there is any spec impact.

…

In this contribution, we would like to share our view on coexistence of Cell DTX/DRX mechanism and RACH-less LTM.

2.	Discussion

There has been a discussion on supporting coexistence of Cell DTX/DRX mechanism and RACH-less LTM. However, it was not concluded yet.

If a network triggers a RACH-less LTM during Cell DTX/DRX Active Period, there may be no issue to perform RACH-less LTM cell switch. However, it cannot be assumed that proper LTM cell switch timing is always aligned with Cell DTX/DRX Active Period. Therefore, we need to examine whether RACH-less LTM cell switch can be performed during Cell DTX/DRX non-active period or not for supporting coexistence of Cell DTX/DRX mechanism and RACH-less LTM.

With Rel-18 NES, synchronization signal and reference signals such as CSI-RS are transmitted regardless of cell DTX/DRX operation. Even, Cell DTX/DRX does not impact on random access procedure. It means that L1 measurement and TA acquisition for LTM cell switch can be performed even during Cell DTX/DRX non-active period.

However, UE does not monitor PDCCH during Cell DTX/DRX non-active period. It is noted that PDCCH monitoring is required to complete RACH-less LTM. There was a similar issue for UE C-DRX, and it was resolved by following agreements and the related MAC spec.

The related agreement (feMob, R2-125bis) RAN2 confirms that the DRX configuration and measurement gap configuration may be applied during on-going RACH-less LTM cell switch. But, as in the current MAC specification, during on-going RACH-less LTM, UE considers DRX Active Time and monitors PDCCH during measurement gap

The related MAC spec. When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or … - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

In case of UE C-DRX, in order to guarantee PDCCH monitoring during ongoing RACH-less LTM cell switch, the time while there is an ongoing RACH-less LTM cell switch is considered as UE C-DRX Active Time.

The same rule with UE C-DRX can be applied to Cell DTX/DRX mechanism to guarantee PDCCH monitoring during ongoing RACH-less LTM cell switch regardless of Cell DTX/DRX operation. In other words, the time while there is an ongoing RACH-less LTM cell switch is considered as Cell DTX/DRX Active Period. It requires MAC spec. change. But it seems simple as follows.

### 5.34.2 Cell Discontinuous Transmission … When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while: - celldtxdrx-onDurationTimer is running for the associated Serving Cell; or - there is an ongoing RACH-less LTM cell switch. … ### 5.34.3 Cell Discontinuous Reception … When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while: - celldtxdrx-onDurationTimer is running for the associated Serving Cell; or - there is an ongoing RACH-less LTM cell switch. …

With the proposed change, UE monitors PDCCH during ongoing RACH-less LTM cell switch regardless of Cell DTX/DRX operation.

Proposal 1. During on-going RACH-less LTM cell switch, UE considers the time as Cell DTX/DRX Active Period.

Proposal 2. If proposal 1 is accepted, adopt TP in Annex.

3.	Conclusion

This document discusses coexistence of Cell DTX/DRX mechanism and RACH-less LTM.

Proposal 1. During on-going RACH-less LTM, UE considers the time as Cell DTX/DRX Active Period.

Proposal 2. If proposal 1 is accepted, adopt TP in Annex.

4.	Annex: TP for TS38.321 (MAC spec.)

### 5.34.2	Cell Discontinuous Transmission

Cell DTX is configured if cellDTXDRXconfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DTX is configured and cellDTXDRXactivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if CellDTXDRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell; or

-	there is an ongoing RACH-less LTM cell switch.

… omitted …

### 5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell; or

-	there is an ongoing RACH-less LTM cell switch.

… omitted …