3GPP TSG-RAN WG2 Meeting #126	R2-2405576

Fukuoka, Japan May 20nd – 24th, 2024

Agenda Item:	    8.5.4

Source:	III

Title:	Discussion on adaptation of paging signal/channel

Document for:	Discussion

Introduction

In RAN2#125bis [1] meeting, the agreements of adaption of common signal/channel for paging are following.

Agreements on paging adaptation: 1. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. 2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. 3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, adaptation of the Paging Frame (PF) is discussed.

Discussion

Adaptation of paging frame by compact/bundle

In TR 38.864[2], it is shown that adaptation of PF/PO can achieve BS energy savings by 0.2%~42.3%. In RAN1#116 email discussion [3], compact/bundle PF periodicity were mentioned [4] [5].We propose a mechanism for compact/bundle PF adaptation.

In TS 38.304[6], PF formula as follows:

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) N: number of total paging frames in T Ns: number of paging occasions for a PF PF_offset: offset used for PF determination UE_ID: If an eDRX cycle is configured by RRC or upper layers and eDRX-Allowed is signalled in SIB1: - 5G-S-TMSI mod 4096 else: - 5G-S-TMSI mod 1024

An I-DRX cycle can be divided into M segments. All PFs can be compacted into one segment and the NES cell can sleep in other M-1 segments, as figure 1 (N=4, M=2).

(SFN + PF_offset/M) mod T = [((T/M) div N)*(UE_ID mod N)]

For compact PF, as figure 1:

Figure 1: Compact PF in paging cycle

For bundle PF with M=T/N implies the N PFs being consecutive, as figure 2:

Figure 2: Bundle PF in paging cycle

Observation 1: Compact PF is feasible for cell energy saving.

Observation 2: Bundle PF can be achieved by setting M=T/N in compact PF.

2.2 Compatibility of legacy RRC_IDLE/RRC_INACTIVE UE

Feasible solutions for option 1 and option 2 are discussed below:

- Option 1: Prevent the access of legacy UE via barring.

Legacy UEs are difficult to receive paging massage properly in compact/bundle PF. Therefore, legacy UEs should not be allowed to camp on the NES cell. That is, legacy idle/inactive mode UEs should be barred by NES cell following Rel-18 NES barring procedures.

Observation 3: Legacy idle/inactive mode UEs can’t support compact PF.

Proposal 1: Legacy idle/inactive mode UEs should be barred by NES cell following Rel-18 NES barring procedures.

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are  legacy UEs).

In RAN2#125bis agreement, the "Paging resources for legacy UEs and Rel-19 NES UEs" considers the compatibility of legacy UEs. Legacy UE design should not affect BS energy saving, especially when there is excessive PFs in paging cycle. The implementation of this agreement ensures that paging resources are balanced for legacy UEs and Rel-19 NES UEs. The following is considered for paging resource of NES cell:

- For legacy UEs, the number of PFs in each paging cycle should be decreased as small as possible NES cell. Therefore, NES cell is more sleep time for BS energy saving. In TR 38.331, PF determined by nAndPagingFrameOffset in PCCH-Config. The minimum number of PF for legacy UE is shown as the last row in table 1.

Table 1: number of PFs determined by nAndPagingFrameOffset for legacy UE

| T(cycle) N(PF) | 32rf | 64rf | 128rf | 256rf |
| --- | --- | --- | --- | --- |
| oneT | 32 | 64 | 128 | 256 |
| halfT | 16 | 32 | 64 | 128 |
| quarterT | 8 | 16 | 32 | 64 |
| oneEighthT | 4 | 8 | 16 | 32 |
| oneSixteenthT | 2 | 4 | 8 | 16 |

Extend the values of N to have increased interval between PFs for Rel-19 NES UE in table 2.

Table 2: number of PFs determined by nAndPagingFrameOffset for Rel-19 NES UE

| T(cycle) N(PF) | 32rf | 64rf | 128rf | 256rf |
| --- | --- | --- | --- | --- |
| oneT | 32 | 64 | 128 | 256 |
| halfT | 16 | 32 | 64 | 128 |
| quarterT | 8 | 16 | 32 | 64 |
| oneEighthT | 4 | 8 | 16 | 32 |
| oneSixteenthT | 2 | 4 | 8 | 16 |
| One32thT | 1 | 2 | 4 | 8 |
| One64thT |  | 1 | 2 | 4 |
| One128thT |  |  | 1 | 2 |
| One256thT |  |  |  | 1 |

Proposal 2: RAN2 should further discuss the cases of one32thT, one64thT, one128thT, one256thT.

- PFs of Rel-19 NES UEs should align with PFs of legacy UEs. There are two possible schemes for alignment as following.

  - The PFs of Rel-19 NES UEs should align with PFs of legacy UEs, as figure 3

  - Bundle PF of Rel-19 NES UEs should align with PFs of legacy UEs, as figure 4

Figure 3: PF of Rel-19 NES UEs in paging cycle

Figure 4: Bundle PF in paging cycle

2.3  Consideration for adaption of paging frame during Cell DTX/DRX non-active period

In RAN2#121bis-e agreement [7], the UE does not transmit SPS/SR/CG during Cell DTX/DRX non-active period.

There is no agreement about common

channel behavior during Cell DTX/DRX non-active period, e.g., paging channel for idle mode UE. The compact PF is one feasible method for NES UE receiving paging massage during cell DTX on-duration. Further clarification is needed for this issue:

- In R18 Cell DTX/DRX discussions, only connected mode UEs were addressed, How to extend the cell DTX/DRX mechanism for idle mode UE.

- How to align the compact/bundle PF with cell DTX on-duration for idle/inactive mode UE.

Proposal 3: Compact/bundle PF should be further studied at least in the scenario of cell DTX/DRX mechanism for idle/inactive mode UE.

Conclusions

In this contribution, the following proposals are made:

Observation 1: Compact PF is feasible for cell energy saving.

Observation 2: Bundling PF can be achieved by setting M=T/N in compact PF.

Observation 3: Legacy idle/inactive mode UEs can’t support compact PF.

Proposal 1: Legacy idle/inactive mode UEs should be barred by NES cell following Rel-18 NES barring procedures.

Proposal 2: RAN2 should further discuss the cases of one32thT, one64thT, one128thT, one256thT

Proposal 3: Compact/bundle PF should be further studied at least in the scenario of cell DTX/DRX mechanism for idle/inactive mode UE.

- References

- R2-2403731,” Report from session on V2X/SL, R19 NES and MOB”, RAN WG2 #125bis, April, 2024

- TR 38.864,” Study on network energy savings for NR”, Mar 2023.

- R1-2401870,” Final summary of AI 9.5.3 for R19 NES”, Ericsson, Feb 2024.

- R1-2400371,” Adaptation of common signal/channel transmissions for Network Energy Saving”, Intel, Feb 2024.

- R1-2401145,” Adaptation of common signal/channel transmissions for NES”, Ericsson, Feb 2024.

- TS 38.304 V17.2.0,” User Equipment (UE) procedures in Idle mode and RRC”, Sep 2022.

- R2-2304203,” Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”, InterDigital , April 2023.