3GPP TSG-RAN WG2 Meeting #126	R2-2405582

Fukuoka, Japan May 22nd – 26th, 2024

Agenda item:	7.11.3

Source: 	Langbo

Title: 	Error data handling for MBS

Document Type:	Discussion and Decision

1       Introduction

In the last meeting, we have a following proposal which has been disagreed.[1]

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

However, there may be some misunderstanding about the intention of this proposal. In this contribution, we re-discuss this issue with more explanation.

2       Discussion

According to the current TS 38.331, the multicast MRB(s) associated with multicast session(s) not configured to receive in RRC_INACTIVE will be suspended when UE enter into RRC_INACTIVE.

5.3.8.3 Reception of the RRCRelease by the UE The UE shall: … 2> suspend all SRB(s) and DRB(s), except SRB0 and broadcast MRBs; 2> suspend all multicast MRB(s) associated with multicast session(s) not configured to receive in RRC_INACTIVE;

In our view, as UE is supported to receive more than one different multicast services using a same G-RNTI, there is no restriction for network to allocate how many and which multicast sessions to a G-RNTI for R17 RRC_CONNECTED MBS, or else, it would impact the network scheduling flexibility and number of sessions supported by UE; analogously, there is no restriction assumption for association relationship of multicast sessions and G-RNTI with introduction of R18 RRC_INACTIVE MBS, since such restriction is essentially performance regression compared with R17.

TS 38.300 The following depicts the usage of RNTI for PTM transmission: - A UE can receive different services using same or different G-RNTIs; *********************************** TS 38.331 g-RNTI Used to scramble the scheduling and transmission of PTM for one or more MBS multicast services. g-RNTI-ConfigToAddModList List of G-RNTI configurations to add or modify. Up to 8 G-RNTIs can be configured in total in this release based on the UE capability. maxNumberG-RNTI-r17 INTEGER (2..8) OPTIONAL, -- R1 33-2f: Dynamic multicast with DCI format 4_2

Then, there may be a common case as shown in Figure 1.

Figure 1. multicast session configuration

For a UE configured with G-RNTI 1, if multicast session 1 is configured to be received in RRC_INACTIVE and multicast session 2 is NOT configured to be received in RRC_INACTIVE, it will suspends multicast MRB 2 in RRC_INACTIVE. Obviously, when the UE is in RRC_INACTIVE, it may receive a MAC PDU for MAC entity's G-RNTI 1 containing an LCID 2 associated with the suspended multicast MRB 2. If UE accepts the subPDU contains LCID 2 associated with the suspended multicast MRB 2, this subPDU will result in error.

Observation 1: UE may receive and accept a MAC subPDU for MAC entity's G-RNTI containing an LCID or eLCID associated with the suspended multicast MRB which results in error.

Meanwhile, note that there is a similar case for SDT, i.e., a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value associated with a suspended RB. Based on R2-2306799 [2] in RP-231415, this issue has been implemented in the current TS 38.321, i.e., see the green part below.

5.13 Handling of unknown, unforeseen and erroneous protocol data … When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least: 1> discard the received subPDU. …

Observation 2: It has been specified that when a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value associated with a suspended RB, UE will discard the received subPDU.

Based on the above analysis, it is necessary and reasonable to account the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB associated as unknown, unforeseen and erroneous protocol data and make it alined with SDT.

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

Proposal 2: Adopt one of the TP in the Annex.

3       Conclusion

In this contribution, we discuss error data handling for MBS, and propose as follows:

Observation 1: UE may receive and accept a MAC subPDU for MAC entity's G-RNTI containing an LCID or eLCID associated with the suspended multicast MRB which results in error.

Observation 2: It has been specified that when a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value associated with a suspended RB, UE will discard the received subPDU.

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

Proposal 2: Adopt one of the TP in the Annex.

4       Reference

[1] R2-2403203, Error data handling for MBS, April, 2024.

[2] R2-2306799, Clarification on unknown, unforeseen and erroneous protocol data during SDT, May, 2023.

5       Text proposal

Text proposal is based on 38.321 v18.1.0.

START OF CHANGE

5.13	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI or G-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for broadcast or groupcast, or an LCID value which is not configured, the MAC entity shall:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for unicast, the MAC entity shall:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

END OF CHANGE