3GPP TSG-RAN WG2 Meeting #126	R2-2405664

# Fukuoka, Japan, 20th May 2024 - 24th May 2024

Agenda Item:	7.25.1.7

Source:	Huawei, HiSilicon

Title:	Discussion on remaining issues for Advanced receivers [E174]

Document for: Discussion

1	Introduction

This paper is to discuss a remaining issue in RAN2#125bis for advanced receiver, and a RIL was also proposed on this issue.

R2-2403167	Remaining issues on advance receivers	Ericsson	discussion	Rel-18	NR_demod_enh3-Core

Proposal 1	RAN2 to agree to keep BOOLEAN value for the parameters in IE AdvancedReceiver-MU-MIMO (resolution for the first Editor’s note) and to delete the optionality.

=>	not supported

Proposal 3	RAN2 to remove the parameters advReceiver-MU-MIMO-DCI-1-1 within the IE AdvancedReceiver-MU-MIMO and add in the corresponding field description in IE PDSCH-Config: “Configure the presence of the co-scheduled UE information filed in DCI format 1_1 (see TS 38.212 [17], clause 7.3.1.2.2).”.

-	Qualcomm and ZTE explain that this parameter is needed by RAN1/4

=>	Noted

2	Discussion

In RIL E174, it says:

Only DCI format 1-1 can be used to indicate the MU-MIMO scheduling information the parameter “advReceiver-MU-MIMO-DCI-1-1-r18” is both redundant leads to an error case. This is because the presence of the field advancedReceiver-MU-MIMO in IE PDSCH-Config configures at the UE “the presence of the co-scheduled UE information filed in DCI format 1_1”. The error case happens if UE is configured with advancedReceiver-MU-MIMO in IE PDSCH-Config and the parameter advReceiver-MU-MIMO-DCI-1-1 in IE AdvancedReceiver-MU-MIMO is absent as this becomes a contradictive configuration  When parameter advancedReceiver-MU-MIMO is configured in IE PDSCH-Config, the UE is configured with this behaviours.

And then the proposal was “to remove parameter advReceiver-MU-MIMO-DCI-1-1 in IE AdvancedReceiver-MU-MIMO and add in the corresponding field description in IE PDSCH-Config that. ”

However, according to RAN4, we think the configuration of advReceiver-MU-MIMO-DCI-1-1-r18 is independent to other configurations in AdvancedReceiver-MU-MIMO-r18, e.g. precodingAndResourceAllocation-r18, pdsch-TimeDomainAllocation-r18, and mcs-Table-r18. The configuration of AdvancedReceiver-MU-MIMO-r18 doesn’t mean that the network has to configure the presence of the co-scheduled UE information field in DCI format 1_1.

AdvancedReceiver-MU-MIMO information element

-- ASN1START

-- TAG-ADVANCEDRECEIVER-MU-MIMO-START

AdvancedReceiver-MU-MIMO-r18 ::=      SEQUENCE {

precodingAndResourceAllocation-r18    BOOLEAN                                                OPTIONAL,    -- Need M

dmrsPowerBoosting-r18                 BOOLEAN                                                OPTIONAL,    -- Need M

pdsch-TimeDomainAllocation-r18        BOOLEAN                                                OPTIONAL,    -- Need M

mcs-Table-r18                         ENUMERATED {qam1024, qam256, qam64, spare1}            OPTIONAL,    -- Need R

advReceiver-MU-MIMO-DCI-1-1-r18       ENUMERATED {enabled}                                   OPTIONAL,    -- Need R

...

}

-- TAG-ADVANCEDRECEIVER-MU-MIMO-STOP

-- ASN1STOP

Observation 1: The configuration of advReceiver-MU-MIMO-DCI-1-1-r18 is independent to other configurations in AdvancedReceiver-MU-MIMO-r18, e.g. precodingAndResourceAllocation-r18, pdsch-TimeDomainAllocation-r18, and mcs-Table-r18. The configuration of AdvancedReceiver-MU-MIMO-r18 doesn’t mean that the network has to configure the presence of the co-scheduled UE information field in DCI format 1_1.

Proposal 1	The field advReceiver-MU-MIMO-DCI-1-1-r18 should be kept.

# 3	Conclusion

Based on the above discussion we have the following observations and proposals:

Observation 1: The configuration of advReceiver-MU-MIMO-DCI-1-1-r18 is independent to other configurations in AdvancedReceiver-MU-MIMO-r18, e.g. precodingAndResourceAllocation-r18, pdsch-TimeDomainAllocation-r18, and mcs-Table-r18. The configuration of AdvancedReceiver-MU-MIMO-r18 doesn’t mean that the network has to configure the presence of the co-scheduled UE information field in DCI format 1_1.

Proposal 1	The field advReceiver-MU-MIMO-DCI-1-1-r18 should be kept.