3GPP TSG-RAN WG2 Meeting #126	R2-2405676

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4853 | rev | - | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on when to include the rrc-TransactionIdentifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the guidelines in TS 38.331, Annex 5, regarding the inclusion of the transaction identifiers in RRC message, the following rules apply: 1: DL messages on CCCH that move UE to RRC-Idle should not include the RRC transaction identifier. 2: All network initiated DL messages by default should include the RRC transaction identifier. 3: All UL messages that are direct response to a DL message with an RRC Transaction identifier should include the RRC Transaction identifier. 4: All UL messages that require a direct DL response message should include an RRC transaction identifier. 5: All UL messages that are not in response to a DL message nor require a corresponding response from the network should not include the RRC Transaction identifier. According to this, rule (2:) it says basically that all DL message sent by the network should include the transaction identifier by default. However, this rule has not followed consistently and now the question is whether we go back and include the transaction identifier whenever needed, or if we leave the ASN.1 as it is and we just clarify this in the guidelines. Since the mistake has been already made since Rel-15, our preference would be to not touch the ASN.1 signalling and instead to clarify in the guidelines that the rule (2:) does not always apply (or that there are exceptions). This CR is to adress this issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Anne 5 - Clarified for the rule (2:) that the transaction identifier should be included in the DL message, unless specified otherwise. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: RRC transaction identifier Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there are no inter-operability issues, the UE receiving a message without the transaction idenfier (which it should be present according to Annex 5) may consider the message as erroneous and trigger the RRC re-establishment procedure. 2. If the UE is implemented according to the CR and the network is not, there are no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, there will be inconsistency in the presence of the transaction identifier according to the guidelines provided in Annex 5. This because according to Anne 5 all network initiated DL messages by default should include the RRC transaction identifier, but there are DL messages who did not follow this rule. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

# A.5	Guidelines regarding inclusion of transaction identifiers in RRC messages

The following rules provide guidance on which messages should include a Transaction identifier

1:	DL messages on CCCH that move UE to RRC-Idle should not include the RRC transaction identifier.

2:	All network initiated DL messages by default should include the RRC transaction identifier, unless specified otherwise.

3:	All UL messages that are direct response to a DL message with an RRC Transaction identifier should include the RRC Transaction identifier.

4:	All UL messages that require a direct DL response message should include an RRC transaction identifier.

5:	All UL messages that are not in response to a DL message nor require a corresponding response from the network should not include the RRC Transaction identifier.

END OF CHANGES