3GPP TSG-RAN WG2 Meeting #125       	       	R2-2405689

Fukuoka , Japan, May. 20th – May. 24th, 2024

Agenda item:	7.17.2

Source:	China Telecom

Title:	Discussion on clarification of the action upon T348 expiry

WID/SID:	NR_DualTxRx_MUSIM - Release 18

Document for:	Discussion and Decision

1	Introduction

In this contribution, we discuss on the clarification of the action upon T348 expiry.

2	Discussion

2.1 Clarification on T348 expiry

5.3.5.19	T348 expiry

The UE can apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction, as specified in 5.7.4.3.

|  | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T348 | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for serving cell(s) with capabilities restricted, release of SCell or PSCell or release of SCG. | Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction transmitted via musim-CapRestriction. | UE may apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction. |

Wait timer T348 is the timer related to the musim-Cell-SCG-ToRelease IE and musim-CellToAffectList IE included in musim-CapRestriction-r18 IE in UEAssistanceInformation message. In reactive case, if UE has a preference for serving cell(s), except Pcell, and/or SCG to be release, the release requirement is urgent, like RRC leaving in R17 MUSIM. The description of the action upon T346g(related to RRC leaving) expiry is ‘The UE shall’, and we think a similar definition should be adopted in R18 to clarify UE behaviour and it is good to maintain capability synchronization between network and UE. And we provide a TP as follow.

TP1:

If T348 expires:

- The UE shall release the serving cell(s) if it indicates its preference for SCell or PSCell to be released in the last transmission of the UEAssistanceInformation message;

- The UE shall release the SCG if it indicates its preference for release of SCG in the last transmission of UEAssistanceInformation message;

- The UE can apply the temporary UE capability restriction if it indicates its preference on capability restriction on serving cell(s) in the last transmission of UEAssistanceInformation message.

TP2:

If T348 expires:

1>The UE shall perform the actions upon releasing the serving cell(s) as specified in 5.7.4.3;

1>The UE shall perform the action upon releasing the SCG as specified in 5.7.4.3;

1>The UE can apply the temporary UE capability restriction if it indicates its preference on capability restriction on serving cell(s) in the last transmission of UEAssistanceInformation message.

Proposal 1: A similar description like the action upon T346g expiry should be adopted in the action upon T348 expiry to clarify UE behavior. And can take the following TP as baseline.

TP1:

If T348 expires:

- The UE shall release the serving cell(s) if it indicates its preference for SCell or PSCell to be released in the last transmission of the UEAssistanceInformation message;

- The UE shall release the SCG if it indicates its preference for release of SCG in the last transmission of UEAssistanceInformation message;

- The UE can apply the temporary UE capability restriction if it indicates its preference on capability restriction on serving cell(s) in the last transmission of UEAssistanceInformation message.

TP2:

If T348 expires:

1>The UE shall perform the actions upon releasing the serving cell(s) as specified in 5.7.4.3;

1>The UE shall perform the action upon releasing the SCG as specified in 5.7.4.3;

1>The UE can apply the temporary UE capability restriction if it indicates its preference on capability restriction on serving cell(s) in the last transmission of UEAssistanceInformation message.

3	Conclusion

Proposal 1: A similar description like the action upon T346g expiry should be adopted in the action upon T348 expiry to clarify UE behavior. And can take the following TP as baseline.

TP1:

If T348 expires:

- The UE shall release the serving cell(s) if it indicates its preference for SCell or PSCell to be released in the last transmission of the UEAssistanceInformation message;

- The UE shall release the SCG if it indicates its preference for release of SCG in the last transmission of UEAssistanceInformation message;

- The UE can apply the temporary UE capability restriction if it indicates its preference on capability restriction on serving cell(s) in the last transmission of UEAssistanceInformation message.

TP2:

If T348 expires:

1>The UE shall perform the actions upon releasing the serving cell(s) as specified in 5.7.4.3;

1>The UE shall perform the action upon releasing the SCG as specified in 5.7.4.3;

1>The UE can apply the temporary UE capability restriction if it indicates its preference on capability restriction on serving cell(s) in the last transmission of UEAssistanceInformation message.