3GPP TSG RAN WG2 #126		R2-2405696

Fukuoka, Japan, May 20th – May 24th, 2024

Agenda Item:	8.3.4.1

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on RLF/HO failure prediction

Document for:	Discussion

# Introduction

RAN #102 [1] identified the following use cases for AI/ML-based mobility.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This document includes our discussions, observations and proposals on Radio Link Failure (RLF) and Handover Failure (HOF) prediction.

# Discussion

  - Radio Link Failure (RLF) Prediction

In RRC_CONNECTED mode the UE assesses the radio link quality of the serving cell and several neighbouring cells. This is sent in the form of a report to the serving base station. When the radio link quality degrades below a certain threshold (Qout) for prescribed time duration (T310), the serving base station sends the HO command to the UE leading to HO from the serving cell to the target cell.

In this process, RRM measurements are crucial in mobility management.

Observation 1: Prediction of RRM measurements such as RSRP, RSRQ, SINR is important to minimize RLF.

Observation 2: In order to avoid RLF, it is required to predict the measurements in advance, which can be done by leveraging the procedure described in BM-case 2 i.e., temporal beam predictions.

Proposal 1:  Predict RSRP/RSRQ/SINR of Pt future samples based on history of samples of size (Mt). RAN 2 to define the values of Pt and Mt for the study.

The previous (history of samples) RSRP/RSRQ/SINR measurements (Mt) from serving cells and neighbouring cells can be used to predict the future measurements (Pt). The future measurements (predicted RSRP/RSRQ/SINR) can be used to detect RLF.

Proposal 2: The CDF of difference in dB of the predicted RSRP and actual  RSRP and prediction accuracy of RLF can be one of the KPIs. Other KPIs include RLF prediction accuracy.

Once the RSRP values are predicted, the UE can send a report to the serving base station for the HO command. The UE follows the HO procedure described in [2]. The difference in signal strength in dB of the predicted and actual RRM measurements can be a performance indicator of the AI/ML model. The percentage of RLFs with and without AI/ML can also be studied to evaluate the performance of AI/ML models.

  - Handover (HO) Failure prediction

Handover failure happens in the following scenarios,

- Failure of RRC configuration message from Source gNB to UE. That is, RLF occurs before HO or during HO.

- Failure of RRC configuration acknowledgement from UE to target gNB. That is, RLF occurs during HO or after HO.

Proposal 3: Measurements required for Conditional Handover (CHO) (serving cell and neighboring cells) could be obtained using BM-case 1 (spatial domain beam prediction)

# Conclusion

In this contribution, we have provided a discussion on procedures for AI/ML  based Radio Link Failure (RLF) and Handover Failure (HOF) prediction

Observation 1: Prediction of RRM measurements such as RSRP, RSRQ, SINR is important to minimize RLF.

Observation 2: In order to avoid RLF, it is required to predict the measurements in advance, which can be done by leveraging the procedure described in BM-case 2 i.e., temporal beam predictions.

Proposal 1:  Predict RSRP/RSRQ/SINR of Pt future samples based on history of samples of size (Mt). RAN 2 to define the values of Pt and Mt for the study.

Proposal 2: The CDF of difference in dB of the predicted RSRP and actual  RSRP and prediction accuracy of RLF can be one of the KPIs. Other KPIs include RLF prediction accuracy.

Proposal 3: Measurements required for Conditional Handover (CHO) (serving cell and neighboring cells) could be obtained using BM-case 1 (spatial domain beam prediction)

# References

[1] RP-234055 Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR, RAN 102 Edinburgh, December 2023

[2] TS 38.300 NR and NG-RAN Overall description;