3GPP TSG-RAN WG2 Meeting #126	R2-2405757

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	7.7.3

Source: 	Sequans Communications

Title: 	Report of Soft satellite switch SSB time offset and SMTC impact

Document for:	Discussion and Decision

# Introduction

This is the report for the following offline discussion:

[AT126][301][NR NTN Enh] SMTC impacts for soft satellite switch (Sequans)

Scope: discuss the impact on SMTC adjustment for soft satellite switch and possible impact on decision for H115

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Wednesday 2024-05-22 20:00 (but F2F discussion is invited)  F2F planned Wed 16:30-17:00	BO3

Deadline for rapporteur's summary (in R2-2405757):  Thursday 2024-05-23 08:00

R2-2405672	[H010][H115] and skipping MIB acquisition	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Proposal 2: RAN2 to discuss if any fields require updating, and why (particularly SMTCs)

Continue the discussion in [301]

R2-2405680	Soft satellite switch SSB time offset and SMTC impact	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Consider soft switch where SSB-timeOffset is applied only between t-serviceStart and t-service

Continue the discussion in [301]

# Discussion

## SMTC impacts of soft satellite switch

The soft satellite switch changes the SSB timing. Potential impacts on SMTCs have been investigated in ‎[1]. The SSB timing change might be realized at gNB either with or without changing the cell timing, so both options are considered.

We assume as an example a SSB periodicity of 20ms, and an SSB-offset of 10ms. We consider mobility with neighbor TN cells, and with other NTN cells “synched” with the NTN unchanged PCI cell (performing a similar change in a synchronized way).

Upon each soft satellite switch, potential impact on SMTCs would be as follows:

- NTN cell SMTC, in TN cell: off by 10ms => need to be updated

- TN cells SMTCs, in NTN cell

  - With shifted cell timing: off by 10ms => need to be updated

  - With shifted SSB location (no cell timing change): no impact

- NTN cells SMTCs, in NTN cell

  - With shifted cell timing: no impact

  - With shifted SSB location (no cell timing change): off by 10ms => need to be updated

“Need to be updated” above means:

- In IDLE/INACTIVE, SMTCs offsets update in SIB2/SIB4

- In CONNECTED, RRC reconfiguration of the SMTCs offsets (if configured)

Q1: Do companies agree on the impacts on SMTC configuration as described above?

| Company | Comments |
| --- | --- |
| Google | In case of no cell timing change, there are no issues |
| Apple | For connected, current procedure will enable to update the SMTCs. |
| QC | SMTC update should be handled autonomously by UE |
| Huawei | If cell timing is shifted, no need to update the SMTCs |
| Ericsson | If shifted SSB location, SMTC needs to be reconfigured |

## SIB acquisition at satellite switch (related to [H115])

SIB acquisition at satellite switch could enable to mitigate the signaling load (avoid SI change notification for IDLE/INACTIVE UEs in green cases above).

Q2: Companies views on SIB acquisition at satellite switch? (other SIBs than SIB19)

| Company | Comments |
| --- | --- |
| CMCC | Not needed |
| Nokia | Needed, e.g for SMTCs |
| Vivo | Needed (no spec change) |
| QC | Not needed |
| HW | Not needed |
| China Telecom | Not needed |
| LG | Needed (no spec change) |
| Google | Not needed |
| Ericsson | Needed (SMTCs configuration may change) but ok with legacy SI change notification |

## Soft switch without SSB timing change

An alternative would be to consider that the SSB-timeOffset would be applicable only between t-serviceStart and t-service, so that the overall SSB timing is not changed by the soft switch procedure, hence avoiding SMTCs impact.

Figure 1 - Soft switch without SSB timing change

The cell timing/SSB location would not change (as for the hard switch). The only difference of soft switch would be the additional SSBs sent during the soft switch duration, with SSB-timingOffset. The UE would just sync PSS/SSS to acquire timing (and freq) and can apply it at T-service by removing SSB-timingOffset.

Q3: Companies views on Soft switch without SSB timing change?

| Company | Comments |
| --- | --- |

# Conclusion

The output of the F2F is as follows:

Q1: Do companies agree on the impacts on SMTC configuration as described above?

There seems to be different views on the impact on SMTCs configuration, with at least a few companies not seeing an impact (Qualcomm, Google).

Q2: Companies views on SIB acquisition at satellite switch? (other SIBs than SIB19)

Not needed (5): CMCC, QC, HW, China Telecom, Google

Needed (4): Nokia, Vivo (?), LG, Ericsson (could also accept legacy SI change notification)

There is no consensus on this topic.

Q3: Companies views on Soft switch without SSB timing change?

Not discussed.

# References

- R2-2405680, “Soft satellite switch SSB time offset and SMTC impact”, Sequans Communications