3GPP TSG RAN WG2#126	R2-2405928

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 #117	R1-2405403

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Title:	LS on higher layer parameters for Rel-18 Positioning

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	RAN1

To:	RAN WG2

CC:

# Contact Person:

# Name:	Debdeep Chatterjee

E-mail Address:	debdeep.chatterjee@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# Overall description

During RAN1 #117 meeting, RAN1 identified that the following parameters for Rel-18 positioning need to be included in TS 38.331 as per the RAN1 agreements below:

Agreement Send an LS to RAN2 with the following relevant parameters related to the resource allocation mode 2 in a dedicated SL PRS resource pool. Sub-feature group; RAN1 specification; Parameter name in the spec; New or existing?; Description; Value range; Per (UE, cell, TRP, …); Required for initial access or IDLE/INACTIVE; Specification. SL PRS configuration in a dedicated resource pool; 38.214; sl-SelectionWindowList- Dedicated-SL-PRS-RP; New; List of selection window configuration; SEQUENCE (SIZE (8)) OF SL-PRS-SelectionWindowConfig-Dedicated-SL-PRS-RP; Per dedicated SL PRS resource pool; Yes; 38.331. SL PRS configuration in a dedicated resource pool; 38.214; sl-SelectionWindowConfig-Dedicated-SL-PRS-RP; New; Parameter that determines the end of the selection window in the resource selection for a SL PRS with respect to priority indicated in SCI; sl-PRS-Priority-r18 INTEGER (1..8) and sl-PRS-SelectionWindow-r18 ENUMERATED {n1, n5, n10, n20} Value n1 corresponds to 1*2µ, value n5 corresponds to 5*2µ, and so on, where µ = 0,1,2,3 refers to SCS 15,30,60,120 kHz respectively; Per dedicated SL PRS resource pool; Yes; 38.331. SL PRS configuration in a dedicated resource pool; 38.214; sl-Thres-RSRP-List- DedicatedSL-PRS-RP; New; Indicates a list of 64 thresholds, the threshold should be selected based on the priority in the decoded SCI and the priority in the SCI to be transmitted; SEQUENCE (SIZE (64)) OF SL-PRS-Thres-RSRP; Per dedicated SL PRS resource pool; Yes; 38.331. SL PRS configuration in a dedicated resource pool; 38.214; sl-PreemptionEnable- Dedicated-SL-PRS-RP; New; Indicates whether pre-emption is disabled or enabled in a resource pool. I f the field is present and the value is pl1, pl2, and so on (but not enabled), it means that pre-emption is enabled and a priority level p_preemption is configured. If the field is present and the value is enabled, the pre-emption is enabled (but p_preemption is not configured) and pre-emption is applicable to all levels.; ENUMERATED {enabled, pl1, pl2, pl3, pl4, pl5, pl6, pl7, pl8}; Per dedicated SL PRS resource pool; Yes; 38.331. Agreement Send an LS to RAN2 with the following RRC parameters: Sub-feature group; RAN1 specification; Parameter name in the spec; New or existing?; Description; Value range; Per (UE, cell, TRP, …); Required for initial access or IDLE/INACTIVE; Specification. Network scheduled SL positioning; 38.212; SL-PRS-RNTI; New; Parameter indicate the SL-PRS-RNTI used for monitoring the network scheduling to transmit NR sidelink positioning reference signal (i.e. the mode 1) for dynamic grants; RNTI-Value; Per UE; /; 38.331. Network scheduled SL positioning; 38.212; SL-PRS-CS-RNTI; New; Parameter indicate the RNTI used to scramble CRC of DCI format 3_2 for configured grants; RNTI-Value; Per UE; /; 38.331.

Further, RAN1 agreed to introduce the 4th codepoint for formats3-1-And-3-2 as reflected in the following agreement:

Agreement Send an LS to RAN2 indicating that RAN1 agrees to introduce the 4th codepoint for formats3-1-And-3-2 - Up to RAN2 to decide whether this codepoint can be added in the existing parameter of dci-FormatsSL or not.

On the value ranges and field description for sl-PRS-ResourceReservePeriodList, RAN1 agreed on the following:

Agreement Send an LS to RAN2 with the following action item(s): - Action Item 1: Include the additional values for the value range of “sl-PRS-ResourceReservePeriodList”: {ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240}. - Action Item 2: Suggest the removal of the sentence “The possible resource reservation period are periodicities for SL communication and the ones defined for DL-PRS” in the description of the sl-PRS-ResourceReservePeriodList field - Action Item 3: Suggest the addition of the following sentence “The value ms0 is always configured” in the description of the sl-PRS-ResourceReservePeriodList field

# Actions

To RAN WG2

ACTION:

- RAN1 respectfully requests RAN WG2 to include the above new parameters for SL positioning to TS 38.331 for Rel-18.

- RAN1 respectfully requests RAN WG2 to introduce the 4th codepoint for formats3-1-And-3-2 in TS 38.331 for Rel-18.

- RAN1 respectfully requests RAN WG2 to incorporate the following updates regarding sl-PRS-ResourceReservePeriodList in TS 38.331 for Rel-18:

  - Include the additional values for the value range of “sl-PRS-ResourceReservePeriodList”: {ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240}.

  - Remove the sentence “The possible resource reservation period are periodicities for SL communication and the ones defined for DL-PRS” in the description of the sl-PRS-ResourceReservePeriodList field.

  - Add the following sentence “The value ms0 is always configured” in the description of the sl-PRS-ResourceReservePeriodList field.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-WG1 Meeting #118	19th Aug – 23rd Aug, 2024	Maastricht, NL

TSG-WG1 Meeting #118bis	14th Oct – 18th Oct, 2024	China (TBC), CN