3GPP TSG-RAN WG2 Meeting #126		R2-2405935

Fukuoka, Japan, 20th May – 24th May 2024		(Revision of R2-2404420)

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In RAN#103 meeting, the revised WID for R19 low-power Wake-up Signal and Receiver for NR was agreed [1]. In this WID, the objective of the RRC_CONNECTED mode is as follows:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In this contribution, we will discuss the RAN2 impacts on RRC_CONNECTED procedures with LP-WUS/WUR.

- Discussion

  - General

In RAN1#116 and #116bis meetings, RAN1 has discussed the LP-WUS procedure to trigger PDCCH monitoring in RRC_CONNECTED and achieved some conclusions, which are summarized as follows [2]:

Agreement - Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

In RRC_CONNECTED mode, the LP-WUS is expected to be used to trigger the MR to monitor PDCCH. It could be found that RAN1 has identified several options on the LP-WUS to trigger PDCCH monitoring in RRC_CONNECTED. In our understanding, RAN2 should discuss the corresponding impacts of all potential options from RAN1, and make further progress or down selection. After that either RAN1 or RAN2 could make the final decision based on the discussion in RAN1 and RAN2.

Proposal 1: In RRC_CONNECTED mode, RAN2 to further discuss the corresponding impacts of different options on LP-WUS procedures to trigger PDCCH monitoring identified in RAN1.

  - RAN2 impacts on different options

2.1.1 RAN2 impacts on Option 1-1

For Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of drx-onDurationTimer, one common understanding is that this option may replace the DCP functionality.

In this section, we will briefly introduce the legacy DCP functionality and propose our views for LP-WUS procedure to trigger PDCCH monitoring.

Configuration

For DCP configuration, the DCP occasion is located at a configured time offset before the start of drx-onDurationTimer. We think the DCP configuration could be reused for LP-WUS design for Option 1-1, while the range of time offset could be further decided by RAN1. The following figure shows an example of the LP-WUS configuration:

Figure 1: LP-WUS to trigger the starting of the drx-onDurationTimer

Proposal 2: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, similar as DCP, the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

Collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR

In Rel-16 Power Saving, RAN2 has discussed the collision scenarios for DCP:

- DCP occasion is overlapped with the C-DRX Active Time;

- DCP occasion is overlapped with the measurement gap or within BWP switching interruption length;

- DCP collides with the RAR addressed to C-RNTI, i.e., DCP occasion is during ra-ResponseWindow for BFR via RACH.

When the above collision occurs, UE won’t monitor the DCP and will start the corresponding drx-onDurationTimer directly. For simplicity, we could just follow the same DCP design for the case that LP-WUS monitoring occasion collides with the C-DRX Active Time/measurement gap/BWP switching interruption length/RAR for BFR.

Observation 1: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow the similar solution as DCP, i.e. not monitor LP-WUS in case the LP-WUS monitor occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Miss-detection

Besides, without any collision, it is still possible that UE cannot detect the DCP, e.g. the channel quality is not good enough for DCP reception. In DCP design, it is up to the network to configure whether UE should start the corresponding drx-onDurationTimer when DCP is not detected, i.e. based on the RRC parameter ps-WakeUp. To handle the mis-detection of LP-WUS, we could introduce a similar RRC parameter, e.g. Ps-WakeUp-LPWUS, to control whether UE should start the corresponding drx-onDurationTimer.

Observation 2: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow similar solution as DCP if UE fails to detect the LP-WUS.

Impacts on CSI/L1 measurement report and SRS transmission

If the DCP indicates not to start the drx-onDurationTimer, for current symbol n within drx-onDurationTimer duration but UE is not within the C-DRX Active Time in symbol n, UE won’t transmit the SRS or report the semi-persistent CSI, and based on the RRC configuration to determine whether to report the periodic CSI that is L1-RSRP, i.e., the parameter ps-TransmitPeriodicL1-RSRP, or periodic CSI, i.e., the parameter ps-TransmitOtherPeriodicCSI. We think a similar mechanism could be re-used for LP-WUS.

Observation 3: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow similar solution as DCP for CSI/L1 measurement report and SRS transmission during the time within drx-ondurationTimer.

Impact on RRM/RLM/BFD/CSI measurement

The introduction of DCP won’t impact the RLM and RRM measurements. According to the LP-WUS WID, it could be observed that RRM/RLM/BFD/CSI measurements are performed via MR in RRC_CONNECTED mode, so the introduction of LP-WUS won’t impact the RRM/RLM/BFD/CSI measurement which is similar as the DCP.

Observation 4: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow similar solution as DCP for LP-WUS impact on RRM /RLM /BFD /CSI measurement.

In summary, there is no need for RAN2 to re-discuss the same issues as DCP for LP-WUS, same solutions or operations for DCP could be reused for LP-WUS for simplicity. Only if RAN1/RAN2 identifies some specific issues on re-using the DCP functionalities directly, we could discuss it case by case.

Proposal 3: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP are re-used by default, unless any critical issue is identified.

2.1.2 RAN2 impacts on Option 1-2

For Option 1-2: LP-WUS monitoring outside at least legacy C-DRX Active Time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Regarding whether to monitor PDCCH in legacy C-DRX Active Time triggered by the start of drx-onDurationTimer for Option 1-2, RAN1 has agreed to further discuss the following two options:

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS;

- Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS.

The following two figures show the examples of Option 1-2-1 and Option 1-2-2:

Figure 2: LP-WUS procedures to trigger PDCCH monitoring for Option 1-2-1 and Option 1-2-2

For Option 1-2-1, UE needs to monitor PDCCH in legacy drx-onDurationTimer, and PDCCH monitoring may be additionally triggered by LP-WUS outside the C-DRX Active Time. It could be observed that there is no power saving gain for Option 1-2-1, considering more power consumption will be consumed for additional LP-WUS monitoring and possible following PDCCH monitoring outside C-DRX Active Time. The only benefit for Option 1-2-1 is latency, as it could save some latency if the traffic comes outside C-DRX Active Time. Thus, RAN1 has another note to restrict the Option 1-2-1 to be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX, which will bring extra complexity comparing to Option 1-2-2.

Besides, the legacy drx-onDurationTimer is configured by RRC and starts periodicity, thus it may not perfectly match with the real arrival time of data burst, i.e. there may be no data transmission at the starting point of the duration or even during the whole period. Considering the network could trigger the UE to monitor PDCCH by LP-WUS when there is data burst arrival, the PDCCH monitoring triggered by the periodic drx-onDurationTimer could be omitted in case LP-WUS is monitored. Thus, we think the Option 1-2-2 is more preferable comparing to Option 1-2-1.

Observation 5: Option 1-2-1 could be beneficial only for latency, but no power saving gain is obtained for Option 1-2-1.

Observation 6:  The restriction of Option 1-2-1 configured together with Option 1-1 will bring extra complexity comparing to Option 1-2-2.

Proposal 4: RAN2 assumes Option 1-2-1 could excluded.

For Option 1-2-2, RAN1 agreed that PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS, and the LP-WUS monitoring is outside the legacy C-DRX Active Time. According to the MAC specification [3], the C-DRX Active Time includes not only the time duration when drx-onDurationTimer is running, but also other cases as below:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

The C-DRX Active Time other than the time when drx-onDurationTimer is running is quite different from the C-DRX Active Time when drx-onDurationTimer is running. The drx-onDurationTimer is configured to fit the traffic characteristic, e.g. arrival periodicity, which is not always expected there is PDCCH monitoring during drx-onDurationTimer. While the C-DRX Active Time other than the time duration when drx-onDurationTimer is running is designed for specific scheduling or necessary PDCCH monitoring, it is expected there is PDCCH monitoring during the C-DRX Active Time other than the time when drx-onDurationTimer is running. For example, during drx-RetransmissionTimer, it is expected there is scheduling for retransmission. During ra-ContentionResolutionTimer or msgB-ResponseWindow, it is expected there is scheduling for Msg.4 or Msg.B. Other cases are similar.

The design target for LP-WUS in RRC_CONNECTED mode is to reduce unnecessary PDCCH monitoring. The most beneficial case for using LP-WUS is to control uncertain PDCCH monitoring, i.e. the C-DRX Active Time when drx-onDurationTimer is running. But for the C-DRX Active Time other than the time when drx-onDurationTimer is running, the benefit will be small as there will be PDCCH in most cases, while the only benefit is the uncertain location of PDCCH in the corresponding C-DRX Active Time. Thus, in order to keep the simple design for LP-WUS in this release, it is assumed that the UE will keep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running as legacy, even no LP-WUS has been detected. That is, LP-WUS only impacts the PDCCH monitoring outside C-DRX Active Time and during the drx-onDurationTimer duration.

Proposal 5: For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, LP-WUS only impacts the PDCCH monitoring outside C-DRX Active Time and during legacy drx-onDurationTimer, i.e. UE should keep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running, even no LP-WUS has been detected.

In Option 1-2-2, the start of PDCCH monitoring is controlled by LP-WUS, while the stop of PDCCH monitoring is not controlled by drx-onDurationTimer as legacy. Thus, the mechanism on how to stop the PDCCH monitoring should be considered. The most straightforward method is to define a timer to control the stop of PDCCH monitoring triggered by LP-WUS. Whether to introduce a new timer or reuse the legacy DRX timers, e.g. drx-InactivityTimer, could be further discussed. Besides, since PDCCH skipping has been introduced to terminate the PDCCH monitoring in Rel-17, the mechanism could be also considered here to stop the PDCCH monitoring.

Proposal 6: For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, the following solutions on how to stop the PDCCH monitoring could be considered:

- The UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication;

- Using a timer for PDCCH monitoring triggered by LP-WUS, and the UE performs PDCCH monitoring until the timer is expired. FFS whether a new timer or reuse legacy DRX timer, e.g. drx-InactivityTimer.

2.1.3 RAN2 impacts on Option 1-3

For Option 1-3: LP-WUS monitoring inside C-DRX Active Time according to the corresponding configuration to trigger PDCCH monitoring. In our view, this solution is mainly used to resolve jitter issue in drx-onDurationTimer, which is useful for traffic with jitter, e.g. XR, to reduce the unnecessary PDCCH monitoring due to jitter while guaranteeing the service performance. In this case, the data packets may have a wide range of jitter and the network couldn’t predict when the data will arrive exactly, so the drx-onDurationTimer is configured with a long period to accommodate potential jitter, and the UE has to perform intensive PDCCH monitoring in the entire jitter range. The following figure shows the example of Option 1-3:

Figure 3: LP-WUS monitoring inside at least legacy C-DRX Active Time to trigger PDCCH monitoring

Observation 7: The LP-WUS procedures to trigger PDCCH monitoring Option 1-3 is useful for traffic with jitter, e.g. XR, as it can reduce unnecessary PDCCH monitoring due to jitter without loss of performance, e.g. latency.

Regarding the PDCCH monitoring period in this option, the UE starts to monitor LP-WUS instead of PDCCH from the beginning of drx-onDurationTimer, and the UE starts PDCCH monitoring via MR once LP-WUS is detected and indicates the PDCCH monitoring. With this, power saving gain as well as no latency for data transmission could be obtained.

Proposal 7: For LP-WUS procedures to trigger PDCCH monitoring Option 1-3, UE monitors the LP-WUS within the drx-onDurationTimer duration and the UE starts to monitor PDCCH only if the detected LP-WUS indicates UE to monitor PDCCH.

Besides, according to the agreement in RAN1, it seems that for Option 1-3, UE should monitor LP-WUS in all C-DRX Active Time. As mentioned above, there are several cases for C-DRX Active Time as specified in MAC specification [3]. However, in our view, there is no jitter issue in C-DRX Active Time other than the time when drx-onDurationTimer is running. Besides, there is no discussion in RAN1 on the C-DRX Active Time other than the time when drx-onDurationTimer is running. Similar as the Option 1-2-2, there is no need for UE to monitor LP-WUS inside the C-DRX Active Time other than the time when drx-onDurationTimer is running, and UE could monitor PDCCH directly. In this way, we think it is better for RAN2 to further clarify the Option 1-3 that the LP-WUS only impact the drx-onDurationTimer, but not impact the other C-DRX Active Time.

Proposal 8: For LP-WUS procedure to trigger PDCCH monitoring Option 1-3, LP-WUS only impacts the PDCCH monitoring during legacy drx-onDurationTimer, i.e. UE should keep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running, even no LP-WUS has been detected.

2.1.4. Comparison between Option 1-1 and Option 1-2-2

In our understanding, Option 1-3 could be also used together with Option 1-1, which could be discussed later. Since Option 1-1 and Option 1-2-2 has similar function and application scenario, we will compare the power saving gain (PSG) and user-perceived throughput (UPT) gain between Option 1-1 and Option 1-2-2. The performance comparison is provided in Figure 4 and Figure 5. Further detailed evaluation assumptions and illustration of LP-WUS replacement of DCP (i.e. option 1-1) and LP-WUS to trigger MR PDCCH monitoring (i.e. option 1-2-2) can be found in Annex.

In Figure 4, we provided evaluation results assuming DRX configuration of (DRX cycle-Inactivity_timer-Onduration_timer) = (80ms-8ms-8ms), and following cases were evaluated for FR1.

  - Always_On (i.e. No DRX)

  - Legacy DRX (best baseline by Rel-15)

  - Legacy DRX with DCP (best baseline by Rel-16)

  - Legacy DRX with DCP and Rel-17 PDCCH skipping (best baseline by Rel-17)

  - DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (LP-WUS option 1-1)

  - LP-WUS to trigger PDCCH monitoring from different MR sleep states (LP-WUS option 1-2-2)

Figure 4: Performance comparison for different options with 80ms DRX cycle for FR1

According to results in Figure 4, The following observations can be obtained:

- “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline provided by Rel-17) can achieve 12.5% power saving gian with 35.8% UPT loss compared with Legacy DRX (best baseline by Rel-15)

- “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping” (LP-WUS option 1-1) can achieve additional 14.3% power saving gain with almost the same UPT, compared with “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17)

- “LP-WUS to trigger PDCCH monitoring from MR deep sleep” (LP-WUS option 1-2-2) can achieve additional 38.1% power saving gain with 107% UPT gain, compared with “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17)

In Figure 5, we provided evaluation results assuming another DRX configuration of (DRX cycle-Inactivity_timer-Onduration_timer) = (160ms-100ms-8ms), and following cases were evaluated for FR1.

  - Always_On (i.e. No DRX)

  - Legacy DRX (best baseline by Rel-15)

  - Legacy DRX with DCP (best baseline by Rel-16)

  - Legacy DRX with DCP and Rel-17 PDCCH skipping (best baseline by Rel-17)

  - DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (LP-WUS option 1-1)

  - LP-WUS to trigger PDCCH monitoring from different MR sleep states (LP-WUS option 1-2-2)

Figure 5: Performance comparison for different options with 160ms DRX cycle for FR1

According to results in Figure 4-2, the following observations can be obtained:

- “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17) can achieve 52.4% power saving gian with 71.4% UPT loss compared with Legacy DRX (best baseline by Rel-15)

- “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping” (LP-WUS option 1-1) can achieve additional 15% power saving gain with almost the same UPT, compared with “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17)

- “LP-WUS to trigger PDCCH monitoring from MR deep sleep” (LP-WUS option 1-2-2) can achieve additional 35% power saving gain with 229% UPT gain, compared with “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping” (LP-WUS option 1-1)

Observation 8: For power saving gain, if the MR enters deep sleep during LP-WUS monitoring for Option 1-2-2, more power saving gain of around 30% could be obtained compared with Option 1-1.

Observation 9: For UPT gain, Option 1-2-2 could obtain up to more gain while Option 1-1 has some UPT performance loss.

Based on the above observation, Option 1-2-2 could achieve more power saving gain than option 1-1, and Option 1-2-2 has additional UPT gain while Option 1-1 has some UPT performance loss. Thus, Option 1-2-2 is the best option for LP-WUS in RRC_CONNECTED.

Proposal 9: RAN2 assumes Option 1-2-2 should be supported for LP-WUS in RRC_CONNECTED mode.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that gNB RRC signaling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID in [1], current RRM/RLM/BFD/CSI/BM measurement and corresponding report is performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

Regarding the RRC message to carry the LP-WUS configuration, it is straightforward to use the RRCResume and RRCReconfiguration message as usual.

Proposal 10: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 11: LP-WUS configuration is configured in RRCResume/RRCReconfiguration message.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

In case LP-WUS monitoring is already enabled by the RRC signaling, we think anyway there should be a mechanism for activation/deactivation of LP-WUS monitoring. Otherwise, the UE may monitor LP-WUS even the channel quality for LP-WUS signalling is not good enough, which leads to high mis-detection rate of LP-WUS.

Regarding Option 2 discussed in RAN1, in our view, the L1/L2 activation/deactivation signaling can be a new signalling or reuse the existing PDCCH skipping signaling. Option 2 is most straightforward and gNB can dynamically switch the PDCCH monitoring and LP-WUS monitoring. It is better to keep such flexibility for gNB. Thus, Option 2 is preferred.

Regarding Option 3 discussed in RAN1, activation/deactivation of LP-WUS monitoring based on condition(s). In our view the condition could be a timer or related to channel quality. However, using the timer for LP-WUS monitoring is not flexible since the case that the channel quality of LP-WUS is not good enough while the timer is still running may occur. In this case UE have to monitor the LP-WUS though it is not suitable for LP-WUS monitoring. For the channel quality as condition, it may happen that the UE stops LP-WUS monitoring due to bad channel quality, while the network still send LP-WUS to indicate UE to perform PDCCH monitoring. The mismatch between the UE and the network may cause the wastage of the Uu radio resource and gNB power consumption. Thus, we think Option 3 is not preferred.

Regarding Option 4 discussed in RAN1, it seems reasonable. However, since it is associated to other discussion, we prefer to delay the discussion on this option after we have some progress on the LP-WUS operation methods.

Proposal 12: In RRC_CONNECTED mode, in case LP-WUS monitoring is enabled, activation/deactivation of LP-WUS monitoring by L1/L2 signaling is introduced. Details FFS.

- Conclusion

In this contribution, we discuss the LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow the similar solution as DCP, i.e. not monitor LP-WUS in case the LP-WUS monitor occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Observation 2: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow similar solution as DCP if UE fails to detect the LP-WUS.

Observation 3: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow similar solution as DCP for CSI/L1 measurement report and SRS transmission during the time within drx-ondurationTimer.

Observation 4: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, no issues are identified to follow similar solution as DCP for LP-WUS impact on RRM /RLM /BFD /CSI measurement.

Observation 5: Option 1-2-1 could be beneficial only for latency, but no power saving gain is obtained for Option 1-2-1.

Observation 6:  The restriction of Option 1-2-1 configured together with Option 1-1 will bring extra complexity comparing to Option 1-2-2.

Observation 7: The LP-WUS procedures to trigger PDCCH monitoring Option 1-3 is useful for traffic with jitter, e.g. XR, as it can reduce unnecessary PDCCH monitoring due to jitter without loss of performance, e.g. latency.

Observation 8: For power saving gain, if the MR enters deep sleep during LP-WUS monitoring for Option 1-2-2, more power saving gain of around 30% could be obtained compared with Option 1-1.

Observation 9: For UPT gain, Option 1-2-2 could obtain up to more gain while Option 1-1 has some UPT performance loss.

Proposals:

Proposal 1: In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

RAN2 impacts on LP-WUS Options

RAN2 impacts on LP-WUS Option 1-1

Proposal 2: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, similar as DCP, the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

Proposal 3: For LP-WUS procedure to trigger PDCCH monitoring Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP are re-used by default, unless any critical issue is identified.

RAN2 impacts on LP-WUS Option 1-2

Proposal 4: RAN2 assumes Option 1-2-1 could excluded.

Proposal 5: For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, LP-WUS only impacts the PDCCH monitoring outside C-DRX Active Time and during legacy drx-onDurationTimer, i.e. UE should keep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running, even no LP-WUS has been detected.

Proposal 6: For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, the following solutions on how to stop the PDCCH monitoring could be considered:

- The UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication;

- Using a timer for PDCCH monitoring triggered by LP-WUS, and the UE performs PDCCH monitoring until the timer is expired. FFS whether a new timer or reuse legacy DRX timer, e.g. drx-InactivityTimer.

RAN2 impacts on LP-WUS Option 1-3

Proposal 7: For LP-WUS procedures to trigger PDCCH monitoring Option 1-3, UE monitors the LP-WUS within the drx-onDurationTimer duration and the UE starts to monitor PDCCH only if the detected LP-WUS indicates UE to monitor PDCCH.

Proposal 8: For LP-WUS procedure to trigger PDCCH monitoring Option 1-3, LP-WUS only impacts the PDCCH monitoring during legacy drx-onDurationTimer, i.e. UE should keep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running, even no LP-WUS has been detected.

Comparition between option 1-1 and option 1-2-2

Proposal 9: RAN2 assumes Option 1-2-2 should be supported for LP-WUS in RRC_CONNECTED mode.

Enable and disable for LP-WUS monitoring

Proposal 10: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 11: LP-WUS configuration is configured in RRCResume/RRCReconfiguration message.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 12: In RRC_CONNECTED mode, in case LP-WUS monitoring is enabled, activation/deactivation of LP-WUS monitoring by L1/L2 signaling is introduced. Details FFS.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- Chairman’s Notes of RAN1 #116bis.

- 3GPP TS 38.321: "NR Medium Access Control (MAC) protocol specification".

- TR 38.869 for LP-WUS WUR.

- Annex – Evaluation results from TR 38.869

Further details of simulations:

For option 1-1 (i.e. LP-WUS to replace DCP), the offset between LP-WUS monitoring occasion and the corresponding DRX onduration is 3ms and the window of LP-WUS monitoring is 1ms. Due to the short time offset, the MR has to ramp up from deep-sleep to light-sleep even before the LP-WUS monitoring in order to fulfill the 3ms wake-up latency if a positive LP-WUS is detected. However, if no positive LP-WUS is detected, MR can enter deep-sleep again but during this procedure, the MR sleep transition actually consumes additional power, as shown in the upper part of the figure below.

On the other hand, for LP-WUS to trigger MR PDCCH monitoring (i.e. option 1-2-2), MR does not have to have the above “sleep-state transition”, in other words, the MR is only wake-up from deep-sleep when triggered by positive LP-WUS detected, as shown in the lower part of the figure below.

SLS assumptions

| Parameter | Indoor Hotspot |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

|  | FTP traffic |
| --- | --- |
| Model | FTP model 3 |
| Packet size | 0.5 Mbytes |
| Mean inter-arrival time | 200 ms |
| DRX setting | Period = 160 ms, Inactivity timer = 100 ms |