3GPP TSG-RAN WG2 Meeting #126	R2-2405768

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0504 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - According to the LS R4-2406496 from RAN4, the capability of supporting single sample in UE Rx-Tx time difference measurement for single satellite based RTT should be added under the field nr-NTN-MeasAndReport. RAN4 has discussed the requirements for UE Rx-Tx time difference measurement for single satellite based RTT for NW verified location, and agreed that the measurement periods are based on single sample. RAN4 also discussed the UE capability related to single sample measurement, and concluded that supporting single sample in UE Rx-Tx time difference measurement for single satellite based RTT is a component FG 44-3, and it does not require UE to support reduced sample number for TN positioning measurement (FG 27-3-1). RAN4 respectfully asks RAN1 and RAN2 to take the above information into account and update the feature list and UE capability as necessary.. Furthermore, this capability has already been reflected in FG44-3 in the latest RAN1 feature list (R1-2405564), and thus should be captured in the specification accordingly. - As per latest RAN1 UE feature list (R1-2405564), LPP capability nr-NTN-MeasAndReport can also be supported for the FR2-NTN operating bands, and this should be added into the Spec. |
| Summary of change: |  | - Capture the UE capability on Rx-Tx time difference based on single sample in LPP capability nr-NTN-MeasAndReport, according to latest RAN1 feature list. - Clarify in the field description of nr-NTN-MeasAndReport that it can also be present for the FR2-NTN operating bands in Table 5.2.3-1 in TS 38.101-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NR NTN is not correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.12.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.5.12.6a	NR Multi-RTT Capability Information Elements

–	NR-Multi-RTT-MeasurementCapability

The IE NR-Multi-RTT-MeasurementCapability defines the Multi-RTT measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for Multi-RTT. Otherwise, the UE does not include this IE;

-- ASN1START

NR-Multi-RTT-MeasurementCapability-r16 ::= SEQUENCE {

maxNrOfRx-TX-MeasFR1-r16					INTEGER (1..4)						OPTIONAL,

maxNrOfRx-TX-MeasFR2-r16					INTEGER (1..4)						OPTIONAL,

supportOfRSRP-MeasFR1-r16					ENUMERATED { supported }			OPTIONAL,

supportOfRSRP-MeasFR2-r16					ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR1-r16				ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR2-r16				ENUMERATED { supported }			OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17					NR-UE-TEG-Capability-r17			OPTIONAL,

multi-RTT-MeasCapabilityBandList-r17		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

Multi-RTT-MeasCapabilityPerBand-r17

OPTIONAL

]]

}

Multi-RTT-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17					FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17		ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }			OPTIONAL,

...,

[[

supportOfDL-PRS-BWA-RRC-Connected-r18		ENUMERATED { supported }				OPTIONAL,

supportOfDL-PRS-BWA-RRC-Inactive-r18		ENUMERATED { supported }				OPTIONAL,

nr-NTN-MeasAndReport-r18					ENUMERATED { supported }				OPTIONAL,

nr-DL-PRS-RSCP-ReportingRRC-Connected-r18	ENUMERATED { supported }				OPTIONAL,

nr-DL-PRS-RSCP-ReportingRRC-Inactive-r18	ENUMERATED { supported }				OPTIONAL,

supportOfLegacyMeasurementInTimeWindow-r18	ENUMERATED { supported }				OPTIONAL,

assocSingleRx-Tx-WithUpToNsampleRSCP-r18	ENUMERATED { supported }				OPTIONAL,

supportOfRSCP-MeasurementInTimeWindow-r18	ENUMERATED { supported }				OPTIONAL,

supportOfSymbolTimeStampForRSCP-r18			ENUMERATED { supported }				OPTIONAL,

supportOfFinerTimingReportGranularityForPRS-Meas-r18	ENUMERATED { minus1, minus2,

minus3, minus4, minus5, minus6}		OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-MeasurementCapability field descriptions

maxNrOfRx-TX-MeasFR1 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS Resource/Resource Set on FR1.

maxNrOfRx-TX-MeasFR2 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS Resource/Resource Set on FR2.

srs-AssocPRS-MultiLayersFR1 Indicates whether the UE supports measurements derived on one or more DL-PRS Resource/Resource Sets which may be in different positioning frequency layers for SRS transmitted in a single CC. DL-PRS and SRS may be on different bands. This is for FR1 only.

srs-AssocPRS-MultiLayersFR2 Indicates whether the UE supports measurements derived on one or more DL-PRS Resource/Resource Sets which may be in different positioning frequency layers for SRS transmitted in a single CC. DL-PRS and SRS may be on different bands. This is for FR2 only.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for Multi-RTT. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: The capabilities NR-DL-PRS-ResourcesCapability, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1, supportOfRSRP-MeasFR2, srs-AssocPRS-MultiLayersFR1, srs-AssocPRS-MultiLayersFR2, simul-NR-DL-AoD-Multi-RTT are the same in RRC_INACTIVE state.

supportOfDL-PRS-BWA-RRC-Connected Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT. The target device can include this field only if the target device supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Inactive Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_INACTIVE for Multi-RTT. The target device can include this field only if the target device supports dl-PRS-MeasRRC-Inactive and prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive. Otherwise, the target device does not include this field.

nr-NTN-MeasAndReport This field, if present, indicates that the UE supports UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN with the following capabilities: - UE Rx-Tx time difference based on single sample and UE Rx-Tx time difference offset measurement and report for Multi-RTT positioning; - Reporting DL timing drift due to Doppler over the service link associated with the UE Rx-Tx time difference measurement period. NOTE 2: This field is only present, if freqBandIndicatorNR indicates the bands in Table 5.2.2-1 and Table 5.2.3-1 in TS 38.101-5 [54].

nr-DL-PRS-RSCP-ReportingRRC-Connected This field, if present, indicates that the target device supports reporting RSCP in RRC CONNECTED. The UE can include this field only if the UE supports maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field. NOTE 3: RSCP is reported together with UE Rx-Tx time difference measurement.

nr-DL-PRS-RSCP-ReportingRRC-Inactive This field, if present, indicates that the target device supports reporting RSCP in RRC INACTIVE. The UE can include this field only if the UE supports dl-PRS-MeasRRC-Inactive. Otherwise, the UE does not include this field. NOTE 4: RSCP is reported together with UE Rx-Tx time difference measurement.

supportOfLegacyMeasurementInTimeWindow This field, if present, indicates that the target device supports performing legacy measurements inside the indicated time window only for Multi-RTT. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

assocSingleRx-Tx-WithUpToNsampleRSCP This field, if present, indicates that the target device supports associating a single Rx-Tx measurement with up to N_sample RSCP measurement. The UE can include this field only if the UE supports one of nr-DL-PRS-RSCP-ReportingRRC-Connected and nr-DL-PRS-RSCP-ReportingRRC-Inactive. Otherwise, the UE does not include this field.

supportOfRSCP-MeasurementInTimeWindow This field, if present, indicates that the target device supports RSCP measurement on indicated DL-PRS resource sets within the indicated time window(s) for UE assisted positioning. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing, maxNumOfDL-PRS-ResProcessedPerSlot. Otherwise, the UE does not include this field.

supportOfSymbolTimeStampForRSCP This field, if present, indicates that the target device supports reporting timestamp with OFDM symbol index associated with RSCP measurement. The UE can include this field only if the UE supports one of nr-DL-PRS-RSCP-ReportingRRC-Connected and nr-DL-PRS-RSCP-ReportingRRC-Inactive. Otherwise, the UE does not include this field.

supportOfFinerTimingReportGranularityForPRS-Meas This field, if present, indicates that the target device supports finer timing reporting granularity for DL-PRS measurement.

END OF CHANGE