3GPP TSG RAN WG2#126	R2-2404113

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 Meeting #116bis	R1-2403646

Changsha, China, 15th – 19th April 2024

Title:	Reply LS on 2-step RACH for eRedCap

Response to:	R1-2401948 / R2-2401890

Release:	Rel-18

Work Item:	Enhanced support of reduced capability NR devices (NR_redcap_enh-Core)

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1#116bis discussed the following aspects of 2-step RACH for eRedCap raised in the LS from RAN2:

RAN2 had a discussion on how to configure 2-step RA resources for eRedCap UEs. RAN2 decided to recommend RAN1 to support 2-step RA for eRedCap UEs on 2-step eRedCap resources. If this is not agreeable RAN2 will assume that 2-step RA for eRedCap is not supported at all. It was also agreed that the following is not specified from RAN2 standpoint: the case where an eRedCap UE uses 2-step RedCap RA resources when 2-step eRedCap RA resources are not configured. If this is agreeable to RAN1, RAN2 will specify that an eRedCap UE that falls back from 2-step random access (using the 2-step eRedCap RA resources) shall use the 4-step eRedCap RA resources.

RAN1 has made the following earlier agreements:

Agreements: - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs. - Additional early indication in MsgA PRACH is not supported. - If MsgA PRACH early indication for Rel-17 RedCap UEs is configured, a Rel-18 eRedCap UE shall share the MsgA PRACH that is configured for Rel-17 RedCap UEs if the Rel-18 eRedCap UE performs 2-step RACH.

There was no consensus in RAN1 to agree with RAN2 recommendation and revert the RAN1 agreements above. Instead, RAN1 would like to make the following clarifications as a way forward:

First, for the case when eRedCap 4-step RA resources are not configured, but RedCap 4-step RA resources and RedCap 2-step RA resources are configured, both RedCap UEs and eRedCap UEs use RedCap 2-step RA resources for 2-step RA and use RedCap 4-step RA resources for 4-step RA including fallback from 2-step RA. RAN1’s understanding is that the LS from RAN2 does not concern this case, and that 2-step RACH will be supported for eRedCap UEs at least for this case.

Furthermore, from RAN1 point of view, fallback to RedCap 4-step RA resources can already be supported by existing RAN1 specifications for any case where an eRedCap UE uses RedCap 2-step RA resources for 2-step RA, under the assumption that gNB applies the eRedCap RAR timing relaxation for all RedCap UEs and eRedCap UEs when needed. To clarify this, RAN1#116bis made the following agreement:

Agreement: - On top of earlier RAN1 agreement - Additional early indication in MsgA PRACH is not supported. - If MsgA PRACH early indication for Rel-17 RedCap UEs is configured, a Rel-18 eRedCap UE shall share the MsgA PRACH that is configured for Rel-17 RedCap UEs if the Rel-18 eRedCap UE performs 2-step RACH, from RAN1 point of view, above can be supported with fallback to 4-step Rel-17 RedCap RACH resources by assuming that gNB will apply eRedCap RAR timing relaxation for RedCap UE when needed. - Send LS to RAN2 to inform the above agreement.

If this approach is acceptable from RAN2 point of view, this means that 2-step RA for eRedCap UEs can be handled using RedCap 2-step RA resources with fallback to RedCap 4-step RA resources.

# 2	Actions

To RAN2:

ACTION:	RAN1 respectfully asks RAN2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #117	20th – 24th May 2024	Fukuoka, JP

TSG-RAN WG1 Meeting #118	19th – 23rd August 2024	Maastricht, NL