3GPP TSG RAN WG2#126	R2-2404114

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 #116-bis	R1-2403650

Changsha, Hunan Province, China, April 15th – 19th, 2024

Title:	LS to RAN2 on CBSR for Rel-19 MIMO

Response to:

Release:	Rel-19

Work Item:	NR_MIMO_Ph5

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

RAN1 would like to inform RAN2 about the following two agreements pertaining to CBSR:

[116bis] Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, refine the legacy CBSR as follows:

- Only 1-bit hard restriction is supported (analogous to Rel-18 Type-II)

- Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured

  - Send LS to RAN2, and subject to RAN2 consent

- Group-based CBSR granularity where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction

  - FFS: Value(s) of X1 and X2 and detailed design/spec impact

FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/-II codebooks with a same (N1,N2).

[116bis] Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding CBSR design:

- 1-bit hard restriction is supported (analogous to Rel-15 Type-I)

- FFS: 3-bit scaling factor for soft restriction is supported with the scaling factor taken into account in CQI/PMI calculation

- Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured

  - Send LS to RAN2, and subject to RAN2 consent

- N1⋅O1⋅N2⋅O2X1⋅X2-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction

  - FFS: Value(s) of X1 and X2 and detailed design/spec impact

FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/II codebooks with a same (N1,N2).

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #117	26th February – 1st March, 2024	Fukuoka, Japan

TSG RAN WG1 Meeting #118	19th – 23rd August, 2024	Maastricht, NL