3GPP TSG RAN WG2#126	R2-2404115

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 #116bis	R1-2403683

Changsha, China, April 15th – 19th, 2024

# Title:	LS on the identification of the power control parameters after LTM cell switch

# Response to:

# Release:	Rel-18

# Work Item:	NR_mob_enh2-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Yosuke Akimoto

E-mail Address:	akimoto.yosuke at fujitsu.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

In RAN1#116bis, an issue was discussed on how to acquire the power control parameters associated with CandidateTCI-State/CandidateTCI-UL-State applied to the UL transmission after LTM cell switch till TCI-state/TCI-UL-State is indicated at the target cell. RAN1 considers that the following two approaches can be considered to solve this issue, but no consensus has been achieved:

- Approach 1

  - For UL transmission after cell switch and before the serving cell TCI state is indicated, UE applies power control parameter in the ul-powerControl-r17 of the TCI-State or the TCI-UL-State, if configured, corresponding to the CandidateTCI-State or the CandidateTCI-UL-State indicated in the LTM Cell Switch Command. Otherwise, ul-powerControl-r17 configured in BWP-UplinkDedicated of the target cell is applied.

- Approach 2

  - Introduce necessary RRC parameters for power control under LTM configurations.

If approach 1 is taken, it would be necessary to capture in RAN2 specifications the linkage between CandidateTCI-State/CandidateTCI-UL-State and TCI-state/TCI-UL-State for the same cell. The linkage has been agreed in RAN1#115 as following, but it is not captured in RAN1 specifications:

Agreement

UE may expect that:

- For a candidate cell, the configuration of an LTM TCI state in ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-ul-TCI-ToAddModList-r18 is same as its counterpart in dl-OrJointTCI-StateList-r17 and ul-TCI-ToAddModList-r17 of the first active BWP in ServingCellConfig, at least in terms of TCI state ID, the corresponding qcl-Type1 and qcl-Type2 for the DL or joint TCI state or referenceSignal for the UL TCI state.

- The LTM TCI state(s) in ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-ul-TCI-ToAddModList-r18 of a candidate cell is a subset of serving cell TCI state(s) in dl-OrJointTCI-StateList-r17 and ul-TCI-ToAddModList-r17 of the same cell.

If approach 2 is taken, the following parameters need to be newly added: it is noted that the final check/decision is up to RAN2.

- Under Uplink-powerControl-r18

  - Uplink-powerControlId-r18 ::= INTEGER(1.. maxNrofCandidateUL-TCI-r18)

  - p0AlphaSetforPUSCH-r18　　　 P0AlphaSet-r17　OPTIONAL, -- Need R

  - p0AlphaSetforPUCCH-r18　　　 P0AlphaSet-r17　OPTIONAL, -- Need R

  - p0AlphaSetforSRS-r18　　　　  P0AlphaSet-r17　  OPTIONAL　-- Need R

- Under CandidateTCI-State-r18,

  - ul-powerControl-r18 Uplink-powerControlId-r18 OPTIONAL,　 -- Need R

  - Field description: Configures power control parameters for PUCCH, PUSCH and SRS of the LTM candidate that includes this CandidateTCI-State

- Under CandidateTCI-UL-State-r18,

  - ul-powerControl-r18 Uplink-powerControlId-r18 OPTIONAL,　 -- Need R

  - Field description: Configures power control parameters for PUCCH, PUSCH and SRS of the LTM candidate that includes this CandidateTCI-UL-State

- Under LTM-TCI-Info-r18,

  - uplink-PowerControlToAddModList-r18　SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControl-r18　　　OPTIONAL,　 -- Need N

  - uplink-PowerControlToReleaseList-r18 SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControlId-r18　　OPTIONAL,　 -- Need N

  - Field description: Configures UL power control parameters for PUSCH, PUCCH and SRS when field unifiedTCI-StateType is configured for this serving cell.

Also, for approach 2, RAN1 sees the necessity to define a default behaviour when this ul-powerControl-r18 under LTM-TCI-Info-r18 for a candidate cell is not configured, e.g.

- UE is expected to be configured either ul-powerControl-r18 under LTM-TCI-Info-r18 in LTM-Candidate-r18 for a candidate cell or ul-powerControl-r18 under BWP-UplinkDedicated in ServingCellConfig for the candidate cell.

RAN1 respectfully asks RAN2 to take either option below:

Option 1: Capture the RAN1 agreement on the linkage between TCI states for candidate cell(s) and those for target cell(s) in RAN2 specification(s) for approach 1

Option 2: Introduce the new RRC parameters above for approach 2

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the above request into their work.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #117	20th May – 24th May 2024	Fukuoka, JP

TSG RAN WG1 Meeting #118	19th August – 23rd August 2024	Maastricht, NE