3GPP TSG-RAN WG2 Meeting #127	   R2-2406253

Maastricht, Netherlands, Aug 19~23, 2024

Title:	Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC

Response to:	LS R3-2403957

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	Qualcomm Incorporated (to be RAN2)

To:	RAN3

Cc: 	-

# Contact Person:

# Name:	Linhai He

# Email:		linhaihe AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN2 thank RAN3 for the questions on UL PSI based PDU discarding in NR-DC. The following are answers from RAN2.

On activation and deactivation of PSI-based PDU discard,

- UE activates PSI-based SDU discard on a UL split-bearer only after it has received PSI-Based SDU Discard De-/Activation MAC CE activating the discard from both MN and SN;

- UE deactivates PSI-based SDU discard on a UL split-bearer once it has received a PSI-Based SDU Discard De-/Activation MAC CE deactivating the discard from either MN or SN.

In addition, data volume reporting procedure for a split UL bearer also depends on the status of PSI-based SDU discard in a cell group. More specifically,

- After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the primary path that activates SDU discard, UE applies a smaller ul-DataSplitThreshold.

- After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the secondary path that activates SDU discard, UE applies a larger ul-DataSplitThreshold.

- UE applies the legacy ul-DataSplitThreshold if both primary and secondary paths are in congestion state.

The aforementioned UE behaviors are applicable to both Rel-18 and Rel-19. In RAN2’s understanding, these behaviors eliminate the need for coordination between MN and SN when de-/activating PSI-based SDU discard.

# 2. Actions:

# To RAN3:

ACTION: RAN2 kindly asks RAN3 to take the above answers into consideration in their studies.

# 3. Date of Next RAN2 Meetings:

RAN2#127	19th Aug – 23th Aug 2024				Maastricht, Netherland

RAN2#127-bis	14th Oct – 18th Oct 2024				Hefei, China