3GPP TSG RAN WG2 Meeting #127			R2-2406308

Maastricht, NL, Aug. 19th – 23rd, 2024

Agenda item:	8.3.2

Title:	Discussion on cluster based RRM measurement prediction

Source:	Beijing Jiaotong University (BJTU)

Document for:	Discussion

1. Introduction

In RAN 102 meeting, a new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved [1]. In the study item scope, study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

-	AI/ML based RRM measurement and event prediction,

- Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

- Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

- HO failure/RLF prediction (UE sided model) [RAN2]

- Measurement events prediction (UE sided model) [RAN2]

-	Study the need/benefits of any other UE assistance information for the network side model [RAN2]

-	The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

- NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

-	Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

- NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

-	Potential specification impacts of AI/ML aided mobility [RAN2]

-	Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

In the 3GPP TSG-RAN WG2 #126 meeting, [2] proposed cluster approach for RRM measurement prediction and introduced its relationship between model input and output in temporal and spatial prediction. According to the summary of the 126th meeting [3], there are two main discussion points for cluster approach.

- whether we should also report some information relevant to cluster approach. Apart from two company (Huawei, Apple), other companies are fine with proposal from NTT docomo. Since RAN2 agreed at last meeting that “To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain”. So rapporteur recommend to let company report as part of the “model input” and “model output”, if any.

- Huawei and ZTE raise the point that some of the parameters hidden in the Note 4/5/6 under table 1 should be listed in the template explicitly to align among company. Rapporteur recommend some of them on the reflector, where apart from Apple there is no negative comment is received.

In this paper, we propose an AI-based measurement prediction method based on cell-level/cluster-level measurement information for L3 mobility, including the inputs, outputs, data collection and training methods of the AI/ML model.

2. Motivation

Currently, there are two main prediction methods for network triggered L3-based handover. One is cell-specific/per-cell approach. The AI-based cell handover approach is to train a model for each cell to predict the future RSRP of that cell, and then deploy multiple trained models to the network side. This AI-based cell handover prediction method has a simple idea, but its shortcomings lie in the high deployment and management overhead of AI models, and it is difficult to inherit the trained models for different terminal locations, so it will encounter great obstacles in practical application. Another approach which we call cluster-based approach is to use an AI model to predict the future RSRP of multiple adjacent cells based on cell-level/cluster-level RSRP measurement information. In the last RAN2 meeting, the research on this work primarily focused on the basic idea of the cluster-based method. However, the detailed implementation plan for this method was lacking, such as the specifics of the input, output, label generation, and training scheme for the AI model. In this paper, we propose a cluster-based temporal prediction approach for AI-assisted L3 handover, and introduce model’s input, output, data collection, and training scheme.

3. Scenarios description

Fig. 1 The scenario of multi-cell mobility management

Without loss of generality, we consider a FR2 mmWave wireless communication scenario with a user equipment (UE) and seven base stations (BS) shown in Fig.1. Each BS is equipped with three half-wavelength spaced uniform plane arrays (UPAs) of 32 antennas with sector orientation of 30, 150 and -90 degrees, respectively, and we consider a downlink communication system with analog beamforming architecture for each UPA. Meanwhile, the UE is equipped with two half-wavelength spaced antennas without beamforming and moves according to the city street model. The distance between base stations is set to 200m, and the channel model is referenced to 3GPP TR 38.901 UMa for LOS propagation.

4 The cluster-based prediction approach

At the RAN2 125 meeting, the following three cases are agreed for cell level measurement prediction:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results

Different cases have different AI model’s input and output. In the following content, we will introduce three cluster-based temporal prediction approaches for L3 handover corresponding to the above three cases. The prediction mechanism of deep learning -based cell handover is shown in Fig.2. We use cell-level/cluster-level measurement information from the past h instances to predict the RSRP of all cells for the upcoming g instances which will be used to execute cell handover, and only one AI model needs to be trained to complete the RSRP prediction of all cells. This method leverages both the spatial correlation between RSRP measurements of different cells and the temporal correlation between RSRP measurements of the same cell at different times, significantly reducing the model training and deployment costs and ensuring the prediction accuracy.

Fig.2 Cluster-based cell handover prediction mechanism.

Model input/output: Here gives a detailed description of the AI model’s input and output of three cases. For case 1, we use L1-RSRP of all beams for all cells from the past h instances to predict the L1-RSRP of all beams for all cells for the upcoming g instances. The AI model output is then filtered to derive the L3-RSRP, which is used to execute cell handovers. For this option, the output L1-RSRP of AI model can be applied to both beam and mobility management. For case 2, the input option for AI models is the L3-RSRP of each cell from the past h instances, with the output being either the L3-RSRP of each cell for the upcoming g instances. For case 3, we use L1-RSRP of all beams for all cells from the past h instances to predict the L3-RSRP of all cells for the upcoming g instances, which we have detailed introduced in [4].

Data collection: In order to eliminate the effects of fast fading and shadow fading, the RSRP measurements are then filtered to obtain L1-RSRP and L3-RSRP for model training. In this way, L1-RSRP is obtained using the sliding average method with a window length of 5 based on historical RSRP measurements, and the average/maximum L1-RSRP of each cell is selected as the L3-RSRP for that cell.

Model training: In the end, we will introduce the network architecture and training methods of AI models for the above three input/output methods. For the first two input and output approaches, AI models based on LSTM networks combined with FC layers, or CNNs combined with FC layers, can be utilized to predict L1-RSRP from L1-RSRP and to predict L1-RSRP from L3-RSRP. Due to the relatively low data dimension of L3-RSRP, it is challenging to apply CNNs to predict L3-RSRP from L3-RSRP. Therefore, for the last case, LSTM networks combined with FC layers or standalone FC neural networks can be used for prediction. To enhance the prediction performance of the LSTM-based model, the output of this neural network is the predicted RSRP of all cells at a future moment, thus g networks need to be trained separately to predict g moments in the future. For CNN-based models or FC-based models, only one network needs to be trained to output RSRP of all cells in upcoming g moments. More detailed network architecture and simulation experiment results are introduced in [4]

Proposal #1: For cluster-based temporal prediction approach, consider using L1-RSRP of all beams of all adjacent cells or L3-RSRP of all adjacent cells in historical instances as input of the AI/ML model.

Proposal #2: For cluster-based temporal prediction approach, consider using L1-RSRP of all beams of all adjacent cells or L3-RSRP of all adjacent cells in future instances as output of the AI/ML model, and the selected cell set of model input and output is the same.

Proposal #3: Consider using LSTM-based and CNN-based AI models for cluster-based temporal prediction. For LSTM-based models, the output of this neural network is the predicted RSRP of all cells at a future moment, thus g networks need to be trained separately to predict g moments in the future. For CNN-based models, only one network needs to be trained to output RSRP of all cells in upcoming g moments.

6 Conclusion

In this paper, we introduce three cluster-based temporal prediction approaches for L3 handover corresponding to the three cases, focusing on the input, output, data collection and training methods of AI/ML models. Based on the discussion above, we propose the following proposals:

Proposal #1: For cluster-based temporal prediction approach, consider using L1-RSRP of all beams of all adjacent cells or L3-RSRP of all adjacent cells in historical instances as input of the AI/ML model.

Proposal #2: For cluster-based temporal prediction approach, consider using L1-RSRP of all beams of all adjacent cells or L3-RSRP of all adjacent cells in future instances as output of the AI/ML model, and the selected cell set of model input and output is the same.

Proposal #3: Consider using LSTM-based and CNN-based AI models for cluster-based temporal prediction. For LSTM-based models, the output of this neural network is the predicted RSRP of all adjacent cells at a future moment, thus g networks need to be trained separately to predict g moments in the future. For CNN-based models, only one network needs to be trained to output RSRP of all adjacent cells in upcoming g moments.

7. References

[1] RP-234055, ‘Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR’.

[2] R2-2404700, ‘Discussion on RRM measurement prediction’.

[3] R2-2406085, ‘Summary of [POST126][031][AIMob] Simulations (OPPO)’.

[4] W. Li, W. Chen, S. Wang, Y. Zhang, M. Matthaiou, and B. Ai, “Ai-based beam-level and cell-level mobility management for high speed railway communications,” 2024. [Online]. Available: https: //arxiv.org/abs/2407.04336