3GPP TSG-RAN WG2 #127	R2-2406312

Maastricht France August 19th – 23th, 2024

Agenda Item:	8.3.4.1

Source:	OPPO

Title:	Discussion on RLF use case

Document for:	Discussion, Decision

Introduction

About RLF use case, RAN2 concluded following agreement at RAN2#126 meeting:

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is down prioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this paper, left open issues of RLF use case are discussed.

Discussion

RLF prediction definition

The quantity of the L1 beam level measurement to detect RLF is equivalent to BLER of PDCCH. For the sake of simulation, RAN2 need align what quantity we will take to be equivalent to BLER of PDCCH. To us, it could be L1 SINR.

Proposal 1: The measurement quantity to detect RLF is L1 SINR

At last meeting the term direct RLF prediction and indirect RLF prediction is used but not well defined. Our understanding is that in direct RLF prediction the RLF event is predicted by model directly based on the input L1 SINR of the PCell. In indirect RLF prediction, L1 SINR of the PCell is predicted as RRM measurement intra-frequency temporal domain case A at first and then RLF event is derived based on configured RRC parameters.

Proposal 2: In direct RLF prediction, RLF event is directly predicted by model based on L1 SINR of the PCell

Proposal 3: In indirect RLF prediction, L1 SINR of the PCell is predicted as RRM measurement intra-frequency temporal domain case A at first and then RLF event is derived based on configured RRC parameters.

At RAN2#126 meeting, for the outcome of the RLF prediction, it is FFS for indirect prediction case. For direct prediction case it is also not crystal clear about the timing information of the result.

The methodology to predict L1 SINR is similar to RRM sub case 1 and should be done in scenario intra-frequency FR2 temporal domain case A. It means a set of L1 SINR results within one prediction window are available based on a set of L1 SINR results within one observation window. The assessment of RLF event is derived based on predicted L1 SINRs if the prediction window is longer than T310 or both measured L1 SINRs and predicted SINRs if prediction window is shorter than T310.

Figure 2.1-1

Figure 2.1-1 gives an example of potential indirect prediction case. In this example it is assumed:

- T310 e.g. 200ms is shorter than prediction window e.g. 400ms

- Sample period is 20ms

UE can realize that T310 may start at t0. At t1 it can already predicted that T310 expires at t1+prediction window. Before t3 (which is equals to t1+ prediction window), the same RLF event is kept predicted within prediction window e.g. at t2. And after t3 UE will know this predicted event really happens at t3 so further prediction doesn’t matter at all.

When the channel is stable i.e. less than N311 Qin is reported during t1 and t3, the time instance when RLF may occur i.e. t3 is stable too. The relative distance between t3 and beginning of the prediction window (current time instance from model perspective) changes but the absolute time instance i.e. t3 in this example is not changed. And t3 should be the time instances where L1 SINR, either measured or predicted, can be available, which granularity is sample period i.e. 20ms. If the prediction accuracy of L1 SINR is acceptable then there is no ambiguity on the possibility of RLF event. Or it is impossible to predict RLF at all. And obviously the length of the prediction window is closely coupled with prediction accuracy of L1 SINR. The indirect RLF event can be predicted with following characteristics:

1, The predicted RLF event is claimed at one specific time instance. It can be expressed as absolute timing information

2, The claimed time instance should be the time instances of availability of predicted L1 SINR and hence it is not a window

3, Once the RLF event is predicted, the same predicted RLF event will appear within whole prediction window till it really happens

4, The length of the prediction window should be set as such that prediction accuracy is acceptable high

Observation: The indirect RLF event can be predicted with following characteristics:

1, The predicted RLF event is claimed at one specific time instance. It can be expressed as absolute timing information

2, The claimed time instance should be the time instances of availability of predicted L1 SINR and hence it is not a window

3, Once the RLF event is predicted, the same predicted RLF event will appear within whole prediction window till it really happens

4, The length of the prediction window should be set as such that prediction accuracy is acceptable high

The same observation can be also applied to direct prediction. The main difference between direct and indirect prediction of RLF event is the algorithm of the model while the algorithm itself doesn’t change the way UE performance measurement on L1 SINR and the channel itself. If the definition of the prediction result and its corresponding metrics are different between direct and indirect prediction, it will be very difficult to compare among them.

Proposal 4: For both direct and indirect RLF prediction, RLF prediction result is the RLF event at time instance when predicted L1 SINR is available instead of probability of RLF event.

Proposal 5: The predicted RLF event with same timing information is taken as same RLF event

Metrics of RLF

The way to assess the performance of RLF could be similar to what is introduced for measurement event prediction as introduced in [3].

| Genie event\predicted event | Negative | Positive |
| --- | --- | --- |
| Negative | n0 | n1 |
| Positive | n2 | n3 |

Table 2.2-1 Illustration of the prediction results

In table 2.2-1, there are 4 counters defined as follows:

- Counter n0: number of cases, where no event is predicted and no event occurs in baseline

- Counter n1: number of cases, where an event is predicted but no event occurs in baseline

- Counter n2: number of cases, where no event is predicted but an event occurs in baseline

- Counter n3: number of cases, where an event is predicted and an event occurs in baseline

The accuracy formulas are:

- Precision = n3/(n1+n3)

- Recall = n3/(n2+n3)

- F1 score = 2*Precision*Recall/ (Precision+ Recall)

For the definition of counter n3 should be clarified. The time instance when a RLF event is predicted and the time instance of genie RLF event (i.e. real RLF event for monitoring) could be the same. But it should not be precisely exactly the same because:

1, The availability time instance of predicted L1 SINR and measured L1 SINR could be different

2, As long as the gap between these two time instances is close enough, such uncertainty doesn’t hurt the value of the predicted RLF event

Figure 2.2-1 counter n3           Figure 2.2-2 counter n2			Figure 2.2-3 counter n1

In Figure 2.2-1 when the time gap between genie RLF event and predicted RLF event is lower than allowed maximum allowed time gap (called MATG), counter n3 increases i.e., in this case a RLF is predicted accurately. In Figure 2.2-2, if a genie RLF event occurs and there is no predicted RLF event within MATG before and after this event, counter n2 increases. In Figure 2.2-3, if an RLF event is predicted and there is no genie RLF event within MATG before and after this predicted RLF event, counter n1 increases.

Proposal 6: RLF prediction considers both precision and recall as representative metrics, i.e. through F1 score.

Proposal 7: RLF event is accurately predicted if the relative time difference between the time instances of predicted RLF event and genie RLF event is less than maximum allowed time gap (MATG). FFS on MATG value.

Methodology of RLF

The RLF event shall be predicted in PCell in this SID. But the change of PCell is obviously can’t be predicted by model for RLF event prediction during the simulation. In order to make such decision, a hypothetical HO procedure should run in parallel so that UE know the information about serving cell. The hypothetical HO procedure could be a process to decide on measurement event A3. Since this is hypothetical HO procedure, the next question is whether we should also align key parameters of measurement event A3. Since the change of PCell is critical for the assessment of RLF event, we prefer to align among companies. To make it simple objective offset and cell specific offset could be 0 db. And the parameter need be aligned are event offset, hysteresis and TTT timer.

Proposal 8: A hypothetical HO procedure should run in parallel with model for RLF prediction to change PCell.

Proposal 9: The handover parameter i.e. event A3 offset, hysteresis and TTT timer are aligned for simulation

At RAN2#126 meeting, it was agreed that

“For the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead”

The prediction of RLF event is obviously to improve handover performance and hence it is for 2nd study goal. It means no measurement reduction should be applied for measurement of input L1 SINR. And similar to RRM measurement sub case A, a sliding window scheme should be applied for both observation window and prediction window.

Proposal 10: No measurement reduction is necessary for measurement of input L1 SINR.

Proposal 11: A sliding window scheme is adopted for both observation window and prediction window

When a RLF event is predicted and eventually at some point of time a real RLF occurs, what should UE do in the simulation? There are two options:

- Option 1: the simulation stops for this UE and this trajectory

- Option 2: the simulation continues after a fixed duration similar to T311. The serving cell could be changed to cell with best measurement result.

Proposal 12: RAN2 is kindly requested to discuss the two alternatives when a real RLF occurs during the simulation. We slightly prefer option 1 due to its simplicity

- Option 1: the simulation stops for this UE and this trajectory

- Option 2: the simulation continues after a fixed duration (to simulate RRC re-established). The serving cell may be changed and count on the best measurement result of recovery time point.

Simulation assumptions of RLF

The concluded simulation assumptions for RRM measurement use case are collected in Table 2.2.1-1 of status report [4]. At RAN2#126 meeting it is agreed that:

“Simulation assumptions discussed in section 2.3.1~2.3.3 is taken as baseline also for use cases other than RRM measurement prediction. Any update is subject to further discussion on other use cases”

“Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible)”

In general, we don’t see any adjustment is needed for RLF event prediction i.e. all the simulation assumptions can be reused.

Proposal 13: The simulation assumptions agreed for RRM measurement use case can be reused for RLF event prediction simulation

On top of that the RRC parameters to assess RLF event should be aligned for both indirect and direct prediction method. For direct prediction it is at least needed for baseline case, where label of RLF event is generated.

There is recommended value in TR 36.839 as following:

| Parameters | values |
| --- | --- |
| Qout | -8db |
| Qin | -6db |
| T310 | 1s (the default value in 36.331) |
| N310 | 1 |
| T311 | Not used for calibration (since RLF recovery is not simulated in the calibration) |
| N311 | 1 |

Table 2.4-1

After initial simulation we find it is difficult to trigger RLF event based on parameter in table 2.4-1. The key parameter is the length T310 i.e. it is too long to trigger RLF. For evaluation purpose it would be desirable to set T310 short enough make RLF bit easy to happen to facilitate training and inference. Due to the same reason the parameter N311 should be large enough so that T310 is not easily stopped.

In email discussion [POST125bis][021], the sampling period of FR2 is 20ms. To facilitate the simulation, the value of T310 could be an integer multiple of 20ms and be short as much as possible. In TS 38.331, the available values for t310 is ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000}. Therefore,100ms seems to be a good choice.

The value of N311 should be less than T310/(L1 sampling period) and large enough to get more RLF. If we set t310 to be 100ms, then N311 should <100ms/20ms=5. As TS 38.331 states N311 to be ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}, we can set N331 to be 2.

Proposal 14: the assumption of T310 and N311 should be set as such that RLF could be a bit easy to happen and a bit hard to cancel to facilitate training and inference

Proposal 15: RAN2 is kindly asked to discuss the detailed values of T310 and N331 so that they can be aligned among companies. Our recommendation: T310 = 100ms, N311=2.

As for the handover parameter i.e. event A3 offset, hysteresis and TTT timer, the recommended value from us are: 5 db, 0db, 480ms. A large A3-offset can result in a poor serving signal during handover, thus easier to get more RLF.

Other than these 3 parameters, handover preparation and execution time (HPET) also matter. In TR 36.839, handover preparation time and handover execution time are discussed separately. In measurement event prediction, that is not necessary and we can combine them into one parameter (i.e., HPET) for example 80ms.

Proposal 16: The recommended value for handover parameters i.e. event A3 offset, hysteresis, TTT timer, and handover preparation and execution time (HPET), the recommended value are: 5db, 0db, 480ms and 80ms

Conclusion

Proposal 1: The measurement quantity to detect RLF is L1 SINR

Proposal 2: In direct RLF prediction, RLF event is directly predicted by model based on L1 SINR of the PCell

Proposal 3: In indirect RLF prediction, L1 SINR of the PCell is predicted as RRM measurement intra-frequency temporal domain case A at first and then RLF event is derived based on configured RRC parameters.

Observation: The indirect RLF event can be predicted with following characteristics:

1, The predicted RLF event is claimed at one specific time instance. It can be expressed as absolute timing information

2, The claimed time instance should be the time instances of availability of predicted L1 SINR and hence it is not a window

3, Once the RLF event is predicted, the same predicted RLF event will appear within whole prediction window till it really happens

4, The length of the prediction window should be set as such that prediction accuracy is acceptable high

Proposal 4: For both direct and indirect RLF prediction, RLF prediction result is the RLF event at time instance when predicted L1 SINR is available instead of probability of RLF event.

Proposal 5: The predicted RLF event with same timing information is taken as same RLF event

Proposal 6: RLF prediction considers both precision and recall as representative metrics, i.e. through F1 score.

Proposal 7: RLF event is accurately predicted if the relative time difference between the time instances of predicted RLF event and genie RLF event is less than maximum allowed time gap (MATG). FFS on MATG value.

Proposal 8: A hypothetical HO procedure should run in parallel with model for RLF prediction to change PCell.

Proposal 9: The handover parameter i.e. event A3 offset, hysteresis and TTT timer are aligned for simulation

Proposal 10: No measurement reduction is necessary for measurement of input L1 SINR.

Proposal 11: A sliding window scheme is adopted for both observation window and prediction window

Proposal 12: RAN2 is kindly requested to discuss the two alternatives when a real RLF occurs during the simulation. We slightly prefer option 1 due to its simplicity

- Option 1: the simulation stops for this UE and this trajectory

- Option 2: the simulation continues after a fixed duration (to simulate RRC re-established). The serving cell may be changed and count on the best measurement result of recovery time point.

Proposal 13: The simulation assumptions agreed for RRM measurement use case can be reused for RLF event prediction simulation

Proposal 14: the assumption of T310 and N311 should be set as such that RLF could be a bit easy to happen and a bit hard to cancel to facilitate training and inference

Proposal 15: RAN2 is kindly asked to discuss the detailed values of T310 and N331 so that they can be aligned among companies. Our recommendation: T310 = 100ms, N311=2.

Proposal 16: The recommended value for handover parameters i.e. event A3 offset, hysteresis , TTT timer, and handover preparation and execution time (HPET), the recommended value are: 5db, 0db, 480ms and 80ms

- Reference

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

TR 36.839 Mobility enhancements in heterogeneous networks

R2-2402169 Discussion on measurement event prediction

RP-240926 Status report of AI mobility NR SID