3GPP TSG-RAN WG2 Meeting #127	R2-2406410

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.11.3

Source:	Xiaomi

Title:	SBFD resource indication and CLI handling

Document for:		Discussion and Decision

Introduction

The WID of SBFD [1] has objectives related to SBFD resource indication and CLI handling. Recent RAN1 progresses facilitate related RAN2 discussion.

In this contribution, we discuss RAN2 aspects of SBFD resource indication and CLI handling.

Discussion

SBFD resource indication

In previous RAN1 meetings, following was agreed regarding resource indication of SBFD subbands:

For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

Above RAN1 agreements imply that cell-specific configuration for SBFD cannot be reverted by any UE-specific configuration or group-common signalling. Therefore from RAN2 perspective, RAN2 can focus on signalling design of cell-specific configuration on time and frequency location of subbands.

Proposal 1: For resource indication of SBFD subbands, RAN2 focuses on signalling design of cell-specific configuration.

In current NR signalling design, cell-specific configuration can be provided in SIB (e.g. IE ServingCellConfigCommonSIB in SIB1), or in dedicated RRC signalling (e.g. in IE ServingCellConfigCommon). For SCells, cell-specific configuration are provided in dedicated RRC signalling. For PCell, cell-specific configuration can be provided in SIB1 or in dedicated RRC signalling for RRC_CONNECTED UEs. For SBFD resource indication, the question is whether cell-specific configuration can be provided in SIB1. Considering that RAN1 agreed to “Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode”, it is helpful to signal the resource indication in SIB1 to facilitate random access in RRC_IDLE/INACTIVE mode.

Proposal 2: Resource indication of SBFD subbands can be provisioned in SIB1 (e.g. IE ServingCellConfigCommonSIB) and dedicated RRC signalling (e.g. in IE ServingCellConfigCommon).

In previous RAN1 meetings, agreements were reached regarding time domain resource indication of SBFD subbands:

For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details

as well as frequency domain resource indication of SBFD subbands:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier. For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS

There are still FFS points regarding the signaling details, and RAN2 can wait for further RAN1 progress.

Proposal 3: RAN2 waits for further RAN1 progress regarding signalling details on resource indication of SBFD subbands.

CLI handling

UL resource muting for PUSCH

In RAN1#117 meeting, following was agreed regarding UL resource muting for PUSCH:

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - Note: No new DCI field / MAC CE is introduced - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: This feature is subject to UE capability

Based on above agreement, it can be seen that RAN2 needs to define: 1) RRC signalling to indicate UL resource muting for PUSCH; 2) UE capability for UL resource muting for PUSCH.

Proposal 4: RRC signalling is introduced to indicate UL resource muting for PUSCH.

Proposal 5: UE capability is introduced for the support of UL resource muting for PUSCH.

Since this is a RAN1 feature, it is expected that RAN1 will send RAN2 LS for detailed RRC parameter as well as UE feature list. Therefore RAN2 can wait for RAN1 progress before discussing details for RRC signalling and UE capability.

Proposal 6: RAN2 postpones the discussion on details for RRC signalling and UE capability for UL resource muting for PUSCH until receiving related RRC parameter list and UE feature list from RAN1.

L1 based UE-to-UE CLI measurement and reporting

In RAN1#117 meeting, following was agreed regarding L1 based UE-to-UE CLI measurement and reporting:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - Note: At least wideband reporting is supported - UCI bits generation - Priority rules for multiple CSI reporting

As the feature is L1 based measurement and reporting, the main RAN2 impacts are related to measurement resource configuration and potentially UE capability reporting. It is expected that RAN1 will send RAN2 LS for detailed RRC parameter as well as UE feature list. Therefore RAN2 can wait for RAN1 progress before discussing details for RRC signalling and UE capability.

Proposal 7: RAN2 postpones the discussion on details for RRC signalling for measurement resource configuration and UE capability for L1 based UE-to-UE CLI measurement and reporting until receiving related RRC parameter list and UE feature list from RAN1.

Conclusion

In this contribution, we discuss RAN2 aspects of SBFD resource indication and CLI handling, and propose the following:

Proposal 1: For resource indication of SBFD subbands, RAN2 focuses on signalling design of cell-specific configuration.

Proposal 2: Resource indication of SBFD subbands can be provisioned in SIB1 (e.g. IE ServingCellConfigCommonSIB) and dedicated RRC signalling (e.g. in IE ServingCellConfigCommon).

Proposal 3: RAN2 waits for further RAN1 progress regarding signalling details on resource indication of SBFD subbands.

Proposal 4: RRC signalling is introduced to indicate UL resource muting for PUSCH.

Proposal 5: UE capability is introduced for the support of UL resource muting for PUSCH.

Proposal 6: RAN2 postpones the discussion on details for RRC signalling and UE capability for UL resource muting for PUSCH until receiving related RRC parameter list and UE feature list from RAN1.

Proposal 7: RAN2 postpones the discussion on details for RRC signalling for measurement resource configuration and UE capability for L1 based UE-to-UE CLI measurement and reporting until receiving related RRC parameter list and UE feature list from RAN1.

- References

[1] RP-241614, Huawei, "Revised WID: Evolution of NR duplex operation: Subband full duplex (SBFD)"