3GPP TSG-RAN WG2 Meeting #127		R2-2406432

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:		Discussion on event-triggered L1 measurement reporting

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the remaining open issues and further details of event-triggered L1 measurement reporting procedure, including:

- Whether to use MAC CE or UCI for the measurement report;

- Whether parameters related to reporting procedure need to be supported, e.g. the number of reporting times, the configuration of whether to report when leaving condition is met;

- What information to be included in the measurement report.

- Discussion

  - MAC CE or UCI

In Rel-18 intra-CU LTM, we agreed with MAC CE to serve as the cell switch command. Therefore, it seems natural to make MAC layer responsible to determine whether the measurement report should be triggered. Also, if we are going to support conditional LTM, where the measurement results of multiple beams are considered by the UE itself, MAC layer is also more appropriate to make the decision for the final LTM triggering as the Rel-18 LTM. Therefore, the L1 measurement report should firstly be known by the MAC layer with cross-layer interaction with PHY layer, and then MAC layer can trigger the measurement report or trigger LTM execution in conditional LTM case. As the conditional LTM would be further evaluated after RAN #105, for now we only focus on the measurement report. Moreover, in Rel-19, L1 measurement events are discussed in RAN2 and are supposed to be captured in RAN2 specification. Therefore, we propose:

Observation 1: In Rel-18 LTM, MAC layer on UE side is responsible to trigger the cell switch based on LTM Cell Switch Command MAC CE from NW.

Observation 2: In Rel-19, L1 measurement events are discussed in RAN2 and are supposed to be captured in RAN2 specification.

Proposal 1: MAC layer is responsible to determine whether the measurement report should be triggered based on L1 measurement results.

Naturally, we think MAC CE should also be the UL signalling to convey the measurement report. Generally speaking, as the physical-layer uplink resource to carry the MAC CE is not pre-allocated or fixed in current design, MAC CE-based reporting would not cause any wastage of resource and can have more/flexible payload than L1 signaling, especially considering the size of measurement results can vary in a large range.

RAN1 has also discussed the basic steps for using MAC CE[1]:

Proposal 3.1(new): On beam report transmission procedure for UE-initiated/event-driven beam reporting, further study at least of the following aspects for beam report transmission: - Option-1 (MAC-CE): - Step 1: UE transmits a SR for requesting UL-SCH resources, if trigger event occurs. - Step 2: UE detects the DCI format for UL grant. - Step 3: The beam report is carried by MAC CE in a new transmission of PUSCH. - Note: Step-1 and Step-2 can be skipped if UL-SCH resource is available for new transmission, and above do NOT imply to update the legacy procedure of MAC-CE. - Note: The MAC-CE can be carried in dynamically scheduled or semi-static configured resource.

Although RAN1 agreed in RAN1 #116bis that Mode A (dynamically scheduling UCI by gNB) and Mode B (UCI in pre-configured resource(s) for second UL channel) are supported for UE-initiated/event-driven beam reporting, considering the objective of Rel-19 Mobility targeting for mobility between cells, it is still preferred to use MAC CE as the MAC layer can have more information. Therefore, we have the observation and proposal as follows:

Observation 3: MAC CE is more efficient for reporting L1 measurement results which may vary widely in size.

Proposal 2: MAC CE is used to transmit L1 measurement report for Rel-19 mobility. If this is agreed, send an LS to RAN1.

  - Parameters related to reporting

reportOnLeave

In last RAN2 #126 meeting, it was agreed:

Agreements on measurement enhancements for LTM:

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In legacy L3 event-triggered measurement reporting, there is a parameter related to leaving condition which is reportOnLeave, to take L3 A4 event as the example[2]:

EventTriggerConfig::= SEQUENCE { eventId CHOICE { eventA4 SEQUENCE { a4-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useAllowedCellList BOOLEAN },

If reportOnLeave is set to true, then the measurement reporting procedure will be initiated by the UE when the leaving condition applicable for this event is fulfilled. In this way, the network can know timely when the channel quality deteriorates. For L1 measurement report, the motivation for reportOnLeave should be further discussed as the measured signalling strength can change more rapidly without L3 filtering, and L1 reporting may be triggered more frequently. To be simple, as this parameter is configurable in legacy L3, we can also make it configurable so the network can decide whether to enable this reporting to have more information on L1 measurement results. Therefore,

Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

reportAmount

In the legacy L3 measurement, reportAmount and reportInterval are supported. A UE can repeat to send the measurement several times based on the configured reporting amount after the event is triggered.

In our understanding, by reporting multiple times, the network can know whether a triggered beam consecutively has a good/bad quality. Even the signalling strength can change more rapidly, it can be up to network implementation whether to configure and require UE for multiple beam measurement results associated with a report. Therefore,

Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration).

  - Information in the report

In RAN1 MIMO session, they have discussed what needs to be included in L1 event-triggered measurement report and almost all companies agreed that L1-RSRP and beam ID should be included in the measurement report[1]. In our understanding, this is reasonable and also aligned with legacy reporting framework.

Meanwhile, as in RAN2 mobility discussion, the LTM L1 measurement and reporting are mainly for mobility between cells and the UE may measure beams from multiple cells, we understand that cell ID is also needed. In Rel-18, the cell ID is signaled in an implicit way, as following:

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

That is, LTM candidate cell IDs will be associated to the SSBs in the ltm-CSI-SSB-ResourceList. We understand whether the cell ID in L1 measurement report is explicit or implicit can be FFS for Rel-19 LTM.

At last, according to the agreements from RAN2 #125bis meeting[3]:

Agreements on measurements: 1. L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

We understand that one report configuration includes one event configuration (while the resource configuration ID may be associated to different events), so the report configuration should also be informed to network so that the network can know e.g. this measurement reporting is for which event, what is the related configured threshold, etc.

Therefore, we propose:

Proposal 5: The contents in measurement report for event triggered L1 measurement reporting at least include:

-	L1-RSRP;

-	Beam ID;

-	Cell ID, FFS whether explicit or implicit way;

-	Reporting configuration ID (indicates e.g. event type, threshold, etc);

Another issue is how to decide the number of beams of measurement results to be included in the measurement report. Actually, RAN1 has also discussed about this issue and various proposals were raised, e.g. whether variable number of reported beams are supported, whether the reported beams should all satisfy the triggering condition, etc[4].

In Rel-18, the number of cells and RSs per cell are reported within a single L1 measurement report instance are configured by network as follows:

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

So, we could first decide the maximum number of beams(s) to be reported. It would be simple to take Rel-18 as the baseline, so the total number of beams to be included can be restricted to 16. As for the actual number, we could wait for more RAN1 progress, as they already have some kinds of proposals as below[5]:

RAN1 #117 Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

The agreement is based on UCI to be the UL signalling to carry L1 measurement report. If we agree with MAC CE, the UE can simply report all beams that satisfies the reporting condition, so we could come back to this one after we decide MAC CE or UCI. On the other hand, this is anyway left to RAN1, so no proposal is given here.

Proposal 6: Taking R18 LTM as the baseline, the maximum number of beam(s) to be reported in the L1 measurement reporting is 16.

Moreover, it is not clear whether measurement result of serving cell should be included. In L3 measurement reporting, as servingCellMO is configured, the UE would report serving cell measurements result to gNB. For L1 measurement reporting, we understand it is also beneficial to report the measurement results of current beam to gNB for better decision to trigger LTM. Here the current beam, according to RAN1’s agreement, can be understood as a beam corresponding to the indicated TCI state.

In Rel-18, spCellInclusion is introduced to control whether the UE shall include a L1 measurement report associated to the current SpCell. Similarly, network can also control whether L1 measurement results of current beam (i.e. a beam corresponding to the indicated TCI state) in SpCell should be included in the measurement report.

Furthermore, RAN1 also agreed in RAN1 #117 meeting that:

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

Therefore, we propose:

Proposal 7: For event triggered L1 measurement reporting, RAN2 confirm that network can configure whether the L1 measurement result of current beam (i.e. a beam corresponding to the indicated TCI state) in SpCell should be always included in the measurement report.

- Conclusion

In this contribution, we discussed the issues related to event-triggered L1 measurement reporting. We have the following observations and proposals:

UCI or MAC CE

Observation 1: In Rel-18 LTM, MAC layer on UE side is responsible to trigger the cell switch based on LTM Cell Switch Command MAC CE from NW.

Observation 2: In Rel-19, L1 measurement events are discussed in RAN2 and are supposed to be captured in RAN2 specification.

Observation 3: MAC CE is more efficient for reporting L1 measurement results which may vary widely in size.

Proposal 1: MAC layer is responsible to determine whether the measurement report should be triggered based on L1 measurement results.

Proposal 2: MAC CE is used to transmit L1 measurement report for Rel-19 mobility. If this is agreed, send an LS to RAN1.

Parameters related to reporting

Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration).

Information in the report

Proposal 5: The contents in measurement report for event triggered L1 measurement reporting at least include:

-	L1-RSRP;

-	Beam ID;

-	Cell ID, FFS whether explicit or implicit way;

-	Reporting configuration ID (indicates e.g. event type, threshold, etc);

Proposal 6: Taking R18 LTM as the baseline, the maximum number of beam(s) to be reported in the L1 measurement reporting is 16.

Proposal 7: For event triggered L1 measurement reporting, RAN2 confirm that network can configure whether the L1 measurement result of current beam (i.e. a beam corresponding to the indicated TCI state) in SpCell should be always included in the measurement report.

- References

- RAN1 #116 chairman notes;

- 3GPP TS 38.331 V18.2.0 (2024-06);

- RAN2#125bis meeting;

- R1-240xxxx, Moderator Summary #4 on UE-initiated/event-driven beam management;

- RAN1 #117 chairman notes;