3GPP TSG-RAN WG2#127	R2-2406494

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda Item:	8.13.3

Source: 	MediaTek Inc.

Title:  	Discussion on control plane procedures for multi-hop relays

Document for:	Discussion, decision

# 1	Introduction

This document discusses control plane procedures to support layer 2 multi-hop relays.

# 2	Discussion

In RAN#104 meeting, a work item for NR sidelink multi-hop relay is agreed as follows:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

1.	Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

A.	Relay discovery and (re)selection [RAN2]

B.	Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

C.	Impact on SRAP and QoS handling for multi-hop [RAN2]

D.	Control plane procedures [RAN2, RAN3]

According to the work item description, the impact on SRAP and QoS handling needs to be discussed to support multi-hop Layer-2 UE-to-Network relay. And the specified mechanisms should be forward compatible for future extension for additional relays.

The QoS handling impact on multi-hop relay is discussed in this contribution.

2.1 QoS handling for Layer-2 UE-to-network multi-hop relay

In single-hop U2N relay, the QoS profile for end-to-end flow is splitted into 2 parts. The first QoS profile is applied to Uu interface and the second QoS profile is applied to PC5 interface. By reusing the same concept for QoS management of Rel-17 U2N relay, if an additional hop is added onto single-hop U2N relay, the QoS profile is broken down into 3 parts. For future extension on QoS handling toward N-hop relays, the QoS profile is broken down into N+1 parts. Hence, we have following proposal:

Proposal 1: For a two-hop relay (i.e., gNB -> relay UE 1 -> relay UE 2 -> remote UE), the QoS for E2E flow is splitted into three parts; for an N-hop relay, the QoS for E2E flow is splitted into N+1 parts.

In single-hop U2N relay, it is up to gNB implementation to handle QoS breakdown over Uu and PC5 interface for end-to-end QoS enforcement. The gNB decides QoS value (i.e., PDB) for Uu and PC5 interfaces and the gNB issues the QoS configuration through RRC message over PC5/Uu to remote UE and U2N relay.

On the other hand, for layer-2 U2U relay, it is up to U2U relay UE implementation to split QoS value (i.e., PDB) for the first and second hop PC5 links. The remote UE sends to U2U relay UE all the QoS profiles for the end-to-end QoS flows via PC5-RRC message. Then the U2U relay UE performs QoS split for PDB value. The U2U relay UE sends the splitted QoS value back to remote UE via PC5-RRC message. The remote UE or the serving gNB issues the QoS configuration through RRC message over PC5 to the peer remote UE.

The architecture of a multi-hop U2N relay can be seen as a U2U relay as additional hop extension on top of single-hop U2N relay and U2U relay as additional hops extension. In this case, two options for QoS split are illustrated as shown in Figure 1 and Figure 2:

- Option 1: gNB implements QoS split for layer-2 U2N multi-hop relay

It is up to gNB implementation to handle QoS breakdown over multiple interfaces of multiple hops for end-to-end QoS enforcement. The gNB decides split QoS values and issues the QoS configuration through RRC message to remote UE and relay UEs

- Option 2: relay UE of each hop implements QoS split for layer-2 U2N multi-hop relay

It is up to U2N relay UE implementation to split QoS value on the link connecting to the relay. The first hop relay UE performs QoS split to get a QoS profile for the first interface and sends the rest of QoS value to the second hop relay UE. The second hop relay UE performs QoS split to get QoS profiles for the second and last interface. The gNB gets the split QoS values and issues the QoS configuration through RRC message to remote UE and relay UEs.

Figure 1: gNB implements QoS split for layer-2 mult-hop U2N relay

Figure 2: relay UEs implements QoS split for layer-2 mult-hop U2N relay

Proposal 2: RAN2 to discuss whether gNB (as in R-17 U2N relay) or relay UE of each hop (as in R-18 U2U relay) to perform QoS split.

In layer-2 U2N single hop relay, gNB performing QoS split is aware of Uu link’s condition and overall end-to-end transmission path’s condition. Therefore, the gNB can derive PC5 link’s condition and set reasonable QoS split value (i.e., PDB) by parsing latency differences between end-to-end path and Uu link. Similarly, for layer-2 U2U single hop relay, the relay UE performing QoS split is aware of first hop PC5 link’s condition and second hop PC5 link’s condition. In this case, the relay UE can set reasonable QoS split value by its own implementation.

However, if one or more hops are added into U2N relay path, the gNB would be lack of information to determine PDB value for QoS split after first hop. As shown in figure 3, the gNB is unable to know the link qualities individually for PC5_1 and PC5_2 interfaces, and thus the gNB cannot set appropriate PDB values for PC5_1 and PC5_2. Likewise, relay UE 1 is unable to know the link quality for PC5_2 interface and overall end-to-end transmission path’s condition. Relay UE 2 is unable to know the link quality for Uu interface and overall end-to-end transmission path’s condition.

Figure 3: Unknown link quality for QoS split

With information lacking on partial of links, the E2E QoS would not be fulfilled if one of the links within multi-hop relay is assigned with a PDB value that is more stringent than its link serving capability.

Proposal 3: The node that performs QoS split should be aware of latencies and/or link qualities between every hop from a multi-hop relay, otherwise, the E2E QoS would not be achieved.

In order to perform end-to-end QoS enforcement for mutli-hop relay, the link qualities and/or latencies between each hop of the multi-hop relay should be reported to the node that performs QoS split. Hence, it is suggested to embed latency or link quality reporting framework for providing assistance information from each hop to gNB or relay UE.

Proposal 4: Report latency and/or link quality to the node that performs the QoS split. Which quantities are reported can be further discussed.

# 3	Conclusion

This document promulgated the following long list of proposals:

Proposal 1: For a two-hop relay (i.e., gNB -> relay UE 1 -> relay UE 2 -> remote UE), the QoS for E2E flow is splitted into three parts; for an N-hop relay, the QoS for E2E flow is splitted into N+1 parts.

Proposal 2: RAN2 to discuss whether gNB (as in R-17 U2N relay) or relay UE of each hop (as in R-18 U2U relay) to perform QoS split.

Proposal 3: The node that performs QoS split should be aware of latencies and/or link qualities between every hop from a multi-hop relay, otherwise, the E2E QoS would not be achieved.

Proposal 4: Report latency and/or link quality to the node that performs the QoS split. Which quantities are reported can be further discussed.

# 4	References

[1]	TS 38.331

[2]	TR 38.836