3GPP TSG-RAN WG2 Meeting #127	R2-2406526

Maastricht, Netherlands, Aug 19th – 23rd, 2024	Revision of R2-2405429

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on information for Store & Forward

Document for: 	Discussion and Decision

- Introduction

In RAN2 #125bis meeting [1], we have made the following progress on Store & Forward operation:

RAN2 assumptions:

- S&F implies that at least the full eNB will be onboard

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

3.	The S&F satellite operation is common for NB-IoT and eMTC.

4.	The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

And in RAN2 #126 meeting [2], we have made the following agreements:

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In this contribution, we would like to further discuss what assistance information should be provided to the UE for S&F operation.

- Discussion

Store and Forward (S&F) operation is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. As specified in TR 22.865 [3], S&F could be considered as another operation mode (as shown in Figure 1 below) in addition to the “normal/default satellite operation” mode (as shown in Figure 2 below).

Figure 1: “S&F satellite operation” mode [2]

Figure 2:“Normal/default satellite operation” mode [2]

Since the UE behaviour when the UE is in S&F operation mode should be different from UE behaviour when the UE is in normal/default operation mode, the UE needs to know whether the current serving network is operating in which mode. In the previous meetings, it was agreed that a S&F indication can be provided by SIB that an IoT NTN network shall be able to inform UE(s) whether S&F satellite operation is applied.

The IoT NTN network may switch between these two different operation modes. For example, when feeder link is available for a satellite, the IoT NTN network could operate in normal/default operation mode. And when feeder link becomes unavailable, the IoT NTN network could switch to S&F operation mode. Since when to switch operation mode may depend on satellite movement and should be predictable, it is beneficial to indicate the information about when to switch operation mode between S&F operation mode and normal/default operation mode to the UE.

With the information, the UE could know whether the S&F operation is applied. Moreover, if the UE could know when the IoT NTN network switch to/from S&F operation mode, it could estimate how long the data transmission would be interrupted. The UE could also use the information to determine whether to select the IoT NTN network.

Proposal 1:  The S&F indication to the UE should include information about when to switch to/from S&F operation mode.

- Conclusion

We have the following proposal for S&F indication:

Proposal 1:  The S&F indication to the UE should include information about when to switch to/from S&F operation mode .

- Reference

[1] Chairman Notes, RAN2#125bis, 15th – 19th April 2024, Changsha

[2] Chairman Notes, RAN2#126, 22nd – 26th May 2024, Fukuoka

[3] 3GPP TR 22.865 v19.2.0, “Study on satellite access Phase 3”