3GPP TSG-RAN2 Meeting #127	R2-2406552

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	7.4.2

Source:	Google

Title:	Triggering LTM Cell Switch without L1 Measurement and Reports

Document for:	Discussion & Decision

- Introduction

This paper discusses how to trigger an LTM cell switch execution without L1 measurement and reports.

- Discussion

In RAN1 UE feature list (R1-2405566), it is clear that the features of L1 measurement and reports for LTM are optional with capability signalling. In the last meeting, the following was concluded based on the observation 2 of R2-2404299.

As the LTM L1 measurement capability and LTM capabilities are decoupled: - 1. A UE which reports LTM capability without 45-1 may not perform L1 measurement reporting, and it is up to network implementation how to trigger the LTM execution.

Here is observation 2 in R2-2404299:

Observation 2: If the capabilities of L1 measurement and LTM are decoupled; a UE which only reports MCG LTM capability may perform L3 measurement report, and it is up to network implementation how to trigger the LTM execution.

Observation 2 in R2-2404299 highlights that the network can trigger an LTM cell switch execution based on the UE's L3 measurement report. More specifically, when the CU decides to trigger an LTM cell switch due to the UE's L3 measurements, it needs to instruct the serving DU to initiate this LTM cell switch for the UE. However, RAN3 currently lacks a defined F1AP procedure for the CU to convey this instruction to the serving DU (refer to Figure 8.2.1.4-1 Intra-gNB-DU LTM and Figure 8.2.1.5-1 Inter-gNB-DU LTM in TS 38.401).

Observation 1: The CU cannot instruct the serving DU to initiate an LTM cell switch for the UE.

With neither the UE’s L1 measurement reports nor a CU indication, the DU can only initiate an LTM cell switch for the UE in a "blind handover" manner. From the specification and implementation perspectives, blind handover is infeasible. Therefore, blindly triggering LTM cell switch execution is also not a viable solution.

Observation 2: Blindly triggering a LTM cell switch is infeasible for UEs that do not support L1 measurement and reports for LTM.

To enable LTM cell switch execution without the need for UE L1 measurement and reports, RAN3 should define an F1AP mechanism that allows the CU to instruct the serving DU to initiate an LTM cell switch for the UE. Therefore, we propose the following:

Proposal: RAN2 sends a LS to RAN3 to request RAN3 to support the scenario where the CU instructs the serving DU to initiate an LTM cell switch for the UE.

- Conclusion

We propose the following observations and proposal for RAN2’s discussion and decision.

Observation 1: The CU cannot instruct the serving DU to initiate an LTM cell switch for the UE.

Observation 2: Blindly triggering an LTM cell switch is infeasible for UEs that do not support L1 measurement and reports for LTM.

Proposal: RAN2 sends a LS to RAN3 to request RAN3 to support the scenario where the CU instructs the serving DU to initiate an LTM cell switch for the UE.

- Annex Draft LS to RAN3

Title:	Draft LS on Support of CU triggered LTM cell switch execution

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	Google (to be RAN2)

To:	RAN3

Cc:

Contact person:	Frank Wu

Email Address:	frankwu@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN2 agrees to support the scenario where the gNB-CU can instruct the serving gNB-DU to initiate an LTM cell switch to a LTM candidate cell that has been prepared for a UE.

# 2	Actions

To RAN3

ACTION: 	RAN2 kindly requests RAN3 to define necessary signalling in F1AP to support the scenario where the gNB-CU can instruct the serving gNB-DU to initiate an LTM cell switch to a LTM candidate cell that has been prepared for a UE.

# 3	Dates of next RAN2 meetings

RAN2#127-bis		October 14th - 18th, 2024			HeFei, China

RAN2#128			November 18th – 22nd, 2024		Orlando, USA