3GPP TSG-RAN WG2 Meeting #127	R2-2406577

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED Mode

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In RAN1#116bis meeting [1], 3 main options for LP-WUS monitoring in RRC_CONNECTED mode were agreed and in RAN2#126 meeting [2], various agreements were achieved for LP-WUS in RRC_CONNECTED mode. In this contribution, we continue to analyze the procedure for PDCCH monitor for LP-WUS and activation/deactivation of LP-WUS in RRC_CONNECTED, and study the potential issues and solutions. Finally, proposals are provided from our perspective.

Discussion

2.1	LP-WUS monitoring procedure

In this section, we will continue to discuss the impacts of LP-WUS operation methods identified in RAN1from RAN2 perspective.

    - Option 1-1

In RAN1#116bis, option 1-1 was agreed as one option for LP-WUS monitoring:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality

And in RAN2#126 meeting, the agreements for option 1-1 were made as following:

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

Along with the progress made in RAN1 and RAN2, it can be seen that solutions/operations introduced for DCP mechanism is taken as baseline. In the following, we continue to analyze other issues for option 1-1.

2.1.1.1	PDCCH monitoring

In Rel-16 power saving, when the DCP indicates the UE to wake up, the UE will start drx-onDurationTimer. Otherwise, the drx-onDurationTimer will not be started. And the UE will monitor PDCCH when drx-onDurationTimer is running. The UE behaviors related other timers, drx-InactivityTimer, drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL will not impacted by DCP.

As RAN2 has agreed that DCP mechanism is taken as baseline for option 1-1, the following UE behaviors can be agreed for option 1-1, i.e., when LP-WUS indicates the UE to wake up, the UE will start drx-onDurationTimer. Otherwise, the UE will not start drx-onDurationTimer. The UE will monitor PDCCH, when drx-onDurationTimer is running. The other DRX timers, e.g., drx-InactivityTimer, drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

Proposal 1a: For option 1-1, the UE will start drx-onDurationTimer when it is indicated to wake up by LP-WUS; otherwise, the UE will not start drx-onDurationTimer. When drx-onDurationTimer is running, the UE will monitor PDCCH.

Proposal 1b: For option 1-1, other DRX timers, e.g. drx-InactivityTimer, drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

2.1.1.2	Dual DRX group

In order to reduce the power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation, a secondary DRX group was introduced with a separate drx-InactivityTimer and drx-onDurationTimer configured for the FR2 cells enabling FR2 to go to sleep more quickly. However, in order to reduce the impacts to RAN1 and keep the simplicity of DRX group mechanis, dual DRX group is not configured simutanously with DCP.

As RAN2 has assumed that the solutions/ operations introduced for DCP mechanism is taken as baseline, the LP-WUS should not configured simultaneously with dual DRX group to reduce the specification complexity.

Proposal 2: In option 1-1, LP-WUS should not be configured with dual DRX groups simultaneously.

2.1.1.3	Short DRX cycle

For DCP, if short DRX cycle is supported, it may require a huge overhead in the DL which network vendors may have concerned. In RAN1 replied LS [3], it indicated RAN1 assumed that DCP was not applicable for the short DRX cycle if both long and short DRX cycles are configured for the UE. Finally, it was agreed that DCP was only applicable to long DRX cycle.

For LP-WUS, it can simply follow the same mechanism of DCP as above.

Proposal 3: Short DRX cycle is not applicable for LP-WUS.

2.1.1.4	CSI reporting

In DCP, the drx-onDurationTimer may not be started if the UE is not indicated by DCP to wake up. Regarding how to report periodic CSI reports, RAN1 agreed to introduce two parameters ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI, wherein ps-TransmitPeriodicL1-RSRP is used to indicates whether the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start and ps-TransmitOtherPeriodicCSI is used to indicate whether the UE to transmit periodic CSI report(s) other than L1-RSRP reports  when the drx-onDurationTimer does not start.

Periodic CSI report(s) mechanism in DCP[4]

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start.

ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

For option 1-1, whether the UE needs to report periodic CSI reports when drx-onDurationTimer is not started based on LP-WUS can also be studied. But this greatly depends on RAN1 discuss. Hence, we can leave this to RAN1.

Proposal 4: In option 1-1, leave RAN1 to discuss whether the UE transmit periodic CSI report(s) when drx-onDurationTimer is not started according to LP-WUS indication.

2.1.1.5	Miss-detection

In Rel-16, DCP is located at one offset before drx-onDurationTimer. It may happen that the UE does not detect DCP and can’t figure out whether to start drx-onDurationTimer. Hence, one flag ps-WakeUp was defined to indicate whether the UE to wake-up if DCP is not detected outside active time.

DCP mechanism[4]

ps-WakeUp Indicates the UE to wake-up if DCI format 2-6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

As RAN2 has agreed that the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer, the similar miss-detection issue may also happen. From RAN2 perspective, we think this is in RAN1 scope and should be left to RAN1 discussion.

Proposal 5: Leave RAN1 to discuss whether the UE to start drx-onDurationTimer if LP-WUS is not detected outside active time in option 1-1.

2.1.1.6	Pros/Cons

Apparently, option 1-1 is simple. But the transmission latency may be impacted considering the UE can only be waked up during drx-onDurationTimer.

    - Option 1-2-1

Based on the agreement in RAN1, option 1-2-1 is copied below:

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

The following aspects are analyzed for option 1-2-1.

2.1.2.1	PDCCH monitoring

Normally, the PDCCH monitoring for option 1-2-1 can be divided into 3 parts:

- the time outside legacy DRX active time;

- the time during legacy drx-onDurationTimer;

- the time inside legacy DRX active time other than drx-onDurationTimer.

According to RAN1 agreement, the UE may be triggered to perform PDCCH monitor outside at least legacy C-DRX active time according to LP-WUS monitoring configuration. Hence, it can be concluded that the UE may be triggered to perform PDCCH monitoring based on LP-WUS indication outside legacy DRX active time.

According to the current spec, the DRX Active includes the following cases [5]:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

For the time during drx-onDurationTimer, it was agreed that PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. That is the UE may start drx-onDurationTimer and monitor PDCCH based on RRC configured DRX cycle and start offset, regardless LP-WUS indication. If so, the power consumption in option 1-2-1 will be quite high and the gain on power saving will totally vanish considering the UE has to monitor PDCCH not only during entire drx-onDurationTimer but also during the time outside legacy DRX active time. Taking this into account, RAN1 further agrees that if this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX. Hence, whether the UE starts drx-onDurationTimer in option 1-2-1, the UE should follow LP-WUS indication.

For legacy DRX Active time other than drx-onDurationTimer, it may be simple that the UE follows legacy DRX behavior. For example, when drx-HARQ-RTT-TimerDL expires, the UE may start drx-RetransmissionTimerDL. And the UE remains in DRX active Time when drx-RetransmissionTimerDL is running. However, for drx-InactivityTimer which may be started by LP-WUS, it can be studied when more progress of LP-WUS has been made.

The whole procedure is illustrated in Figure 1.

Figure 1: LP-WUS monitor procedure in Option 1-2-1

According to the analysis above, it is proposed that:

Proposal 6: RAN2 confirms the following understanding for Option 1-2-1:

- The UE is triggered to perform PDCCH monitor based on LP-WUS indication during the time outside legacy DRX active time;

- UE behaviors related to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

2.1.2.2	CSI reporting

Considering the mechanism of drx-onDurationTimer is similar to DCP, we think whether CSI is reported can also be left to RAN1 if option 1-2-1 is supported.

2.1.2.3	Pros/Cons

In option 1-2-1, although the latency will be reduced for the data transmission, but the power saving gain may be impacted or even vanish, as the drx-onDurationTimer can be started by LP-WUS. It requires the UE to keep monitoring the PDCCH once the drx-onDurationTimer is started, which is not friendly to the power saving. Furthermore, two different UE mechanisms are defined to monitor PDCCH, which inevitably brings additional complexities to UE behavior and huge impacts on the spec. For example, the UE can monitor LP-WUS continuous during the time outside legacy DRX active time, but the UE may not monitor LP-WUS during legacy drx-onDurationTimer. And correspondingly, the LP-WUS resource configuration should be considered to align with the UE behavior. Thus, this also brings complexities to network implementation.

Hence, it is proposed that:

Proposal 7: Option 1-2-1 is not supported for LP-WUS.

    - Option 1-2-2

Based on the agreement in RAN1, option 1-2-2 is copied below

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

The following aspects are analyzed for option 1-2-2.

2.1.3.1	PDCCH monitoring

For option 1-2-2, similar to option 1-2-1, the following time duration can be studied:

- the time outside legacy DRX active time;

- the time during legacy drx-onDurationTimer;

- the legacy DRX active time other than drx-onDurationTimer.

In the following, we analyze the UE behaviors for these time durations.

The time outside legacy DRX active time

Based on RAN1 agreement, LP-WUS monitoring outside at least legacy C-DRX active time is according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Hence, for the time outside legacy DRX active time, the UE starts to monitor PDCCH based on the LP-WUS indication.

The time during legacy drx-onDurationTimer

Based on RAN1 agreements, PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS in option 1-2-2. In order to trigger UE to monitor PDCCH during legacy drx-onDurationTimer, the LP-WUS can be used. Like the UE behavior outside legacy DRX active time in option 1-2-2, the UE can be triggered to monitor PDCCH during legacy drx-onDurationTimer.

The legacy DRX active time other than drx-onDurationTimer

For legacy DRX active time other than drx-onDurationTimer, the UE can monitor PDCCH following current DRX principle. Similar to Option 1-2-1, timers including drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS. For drx-InactivityTimer, it can be studied when more progress of LP-WUS has been made.

The above procedure is illustrated in Figure 2:

Figure 2: The UE procedure in option 1-2-2

Based on the analysis above, we propose that:

Proposal 8a: The UE is triggered to perform PDCCH monitor based on LP-WUS indication during the time outside legacy DRX active time and the time during legacy drx-onDuration in option 1-2-2.

Proposal 8b: For option 1-2-2, UE behaviors related to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

When the UE is triggered to monitor PDCCH based on LP-WUS, RAN1 has no conclusion when the UE stops PDCCH monitoring. One of the candidate solutions is that the network can control the UE to stop PDCCH monitoring. And for legacy DRX procedure, (long) DRX command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer. For simplicity, these MAC CEs can be reused for LP-WUS. Regarding other network indications, e.g. DCI or LP-WUS, can be left for RAN1 further study.

Considering network based indication may bring significant impacts on the downlink overhead, timer based solution can also be adopted as candidate solution. For instance, when the UE is triggered by LP-WUS to monitor PDCCH, the UE starts one timer which can be newly defined or the legacy drx-inactivityTimer. Considering the MR ramping up time can be 0ms, 6ms and 20ms, the timer can be started after a configured time offset after LP-WUS. The range of time offset can be determined by RAN1.

When the timer is running, the UE performs PDCCH monitoring. When the UE receives PDCCH scheduling new transmission for DL/UL, the UE can re-start this timer or start drx-inactivityTimer depending on further study. It is straightforward that the (long) DRX command MAC CE can be used to stop this timer.

Proposal 9a: For option 1-2-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. The timer can be new defined timer or legacy drx-inactivityTimer.

Proposal 9b: For option 1-2-2, the timer is started at a configured time offset after LP-WUS. The range of time offset is left for RAN1.

Proposal 9c: For option 1-2-2, (long) DRX command MAC CE can be used to stop the timer. The other indications, e.g. DCI or LP-WUS, are dependent on RAN1.

2.1.3.2	CSI reporting

CSI mask is an efficient way for the network to configure physical resources for CSI reporting regardless of the UE activities. It is mainly used together with the long DRX cycle. By configuring CSI mask and the long DRX cycle, the network knows that the UE only reports CSI during the drx-onDurationTime and will not report CSI outside of drx-onDurationTimer, and then the network can reuse the physical resources for CSI reporting out of drx-onDurationTimer.

For option 1-2-2, the active time for a UE is irregular if the UE follows LP-WUS indication to monitor PDCCH. If the UE reports CSI following DRX active time, it is complex for the network to utilize the physical resource among UEs. However, if CSI mask is supported, it may consume power especially there is no traffic during legacy drx-onDurationTimer. Considering CSI reporting is highly dependent on RAN1. RAN2 can leave this for RAN1 for further study.

Proposal 10:  For option 1-2-2, whether to enhance CSI reporting during legacy drx-onDurationTimer depends on RAN1.

Since whether to start the legacy drx-onDurationTimer depends on RAN1 further study, RAN2 can defer the discussion on whether drx-onDurationTimer is started or not when further inputs from RAN1 are achieved.

Proposal 11: For option 1-2-2, whether the legacy drx-onDurationTimer is started or not can wait for further inputs from RAN1, especially progress on CSI reporting during drx-onDurationTimer.

2.1.3.3	Pros/Cons

This option shows high power saving gain, since the UE can be waked up when necessary. Meanwhile, it is also beneficial to latency reduction considering the UE can be waked up when necessary. But it may have impacts on legacy drx-onDurationTimer and DRX active time. Thus, it shows higher spec impacts compared with option 1-1.

    - Option 1-3

Based on the agreement in RAN1, option 1-3 is copied below:

- Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

2.1.4.1	PDCCP monitoring

In this option, the UE monitors the LP-WUS within the legacy drx-onDurationTimer. When the UE detects the LP-WUS indicates the UE to monitor PDCCH, the UE starts to monitor PDCCH as shown in Figure 3.

Figure 3: LP-WUS monitor procedure in Option 1-3

2.1.4.2	Pros/Cons

This option is beneficial to traffic with jitter, e.g., XR, to save some power consumption. But the scenario is only limited to traffic with jitter, which can’t be extended to other scenario. And in option 1-3 the LP-WUS is monitored inside C-DRX active time to reduce the unnecessary PDCCH monitoring which is to have the LP-WUS in place of search space for the PDCCH monitoring indication. Therefore, the power saving gain is marginal. Considering the analysis above, option 1-3 should NOT be supported.

Proposal 12: Option 1-3 is not supported for LP-WUS.

2.2	LP-WUS activation/Deactivation

It was agreed in RAN1 116bis meeting that:

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Based on RAN1 progress, RAN1 identified that the activation/deactivation may depend which option is adopted for PDCCH monitoring. Hence, RAN2 can wait for RAN1 for LP-WUS activation/deactivation.

Proposal 13: Waiting for RAN1 for activation/deactivation of LP-WUS monitoring.

Conclusion

In this document, we analyse proceudre and activation/deactivation of LP-WUS in RRC_CONNECTED state, and we have the following proposls:

Option 1-1:

Proposal 1a: For option 1-1, the UE will start drx-onDurationTimer when it is indicated to wake up by LP-WUS; otherwise, the UE will not start drx-onDurationTimer. When drx-onDurationTimer is running, the UE will monitor PDCCH.

Proposal 1b: For option 1-1, other DRX timers, e.g. drx-InactivityTimer, drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

Proposal 2: In option 1-1, LP-WUS should not be configured with dual DRX groups simultaneously.

Proposal 3: Short DRX cycle is not applicable for LP-WUS.

Proposal 4: In option 1-1, leave RAN1 to discuss whether the UE transmit periodic CSI report(s) when drx-onDurationTimer is not started according to LP-WUS indication.

Proposal 5: Leave RAN1 to discuss whether the UE to start drx-onDurationTimer if LP-WUS is not detected outside active time in option 1-1.

Option 1-2-1:

Proposal 6: RAN2 confirms the following understanding for Option 1-2-1:

- The UE is triggered to perform PDCCH monitor based on LP-WUS indication during the time outside legacy DRX active time;

- UE behaviors related to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

Proposal 7: Option 1-2-1 is not supported for LP-WUS.

Option 1-2-2:

Proposal 8a: The UE is triggered to perform PDCCH monitor based on LP-WUS indication during the time outside legacy DRX active time and the time during legacy drx-onDuration in option 1-2-2.

Proposal 8b: For option 1-2-2, UE behaviors related to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

Proposal 9a: For option 1-2-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. The timer can be new defined timer or legacy drx-inactivityTimer.

Proposal 9b: For option 1-2-2, the timer is started at a configured time offset after LP-WUS. The range of time offset is left for RAN1.

Proposal 9c: For option 1-2-2, (long) DRX command MAC CE can be used to stop the timer. The other indications, e.g. DCI or LP-WUS, are dependent on RAN1.

Proposal 10: For option 1-2-2, whether to enhance CSI reporting during legacy drx-onDurationTimer depends on RAN1.

Proposal 11: For option 1-2-2, whether the legacy drx-onDurationTimer is started or not can wait for further inputs from RAN1, especially progress on CSI reporting during drx-onDurationTimer.

Option 1-3:

Proposal 12: Option 1-3 is not supported for LP-WUS.

LP-WUS monitoring activation/deactivation:

Proposal 13: Waiting for RAN1 for activation/deactivation of LP-WUS monitoring.

Reference

- R1-2403821, RAN1#116bis meeting report;

- R2-2406202, RAN2#126 meeting report;

- R1-1911475, LS on PDCCH-based Power Saving Signal/Channel carrying indication of UE wakeup before DRX ON, CATT;

- TS 38.331, V18.2.0, Radio Resource Control (RRC) protocol specification, Release 18;

- TS 38.321, V18.2.0, Medium Access Control (MAC) protocol specification, Release 18.