3GPP TSG-RAN WG2 Meeting #127	R2-2406582

MAASTRICHT 19.08.2024-23.08.2024

Source: 	Vodafone

Title: 		Inventory procedure without permanent device ID

Document for: Decision

Agenda Item: 8.2.2 - Functionality aspects

## Introduction

During RAN WG2#126, 3gpp reached the following agreements on inventory and command:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

At the same time, RAN WG2 took the following decisions regarding paging:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

## Discussion

As it can be seen, the inventory procedures foresees that a single device, group of devices or all devices can be paged and as an answer they return their device IDs. The device ID is returned in the msg 3, which is large in size, as only device ID will include about 100 bits + CRC of 16 bits and other information. In contrast to msg 1 which is smaller and includes a random number of e.g. 16 bits and a CRC of 5 bits. The communication flows are described in Figure 1 below:

Figure 1 – Paging + Inventory + Command procedures.

The inventory procedure as described above is thus expected to be completed with a particular latency, and also power consumption on the devices to transmit Msg1, Msg3, which are even more increased in the case of command procedure following after with subsequent messages.

In order to tackle part of this latency and power consumption in certain scenarios related to the inventory procedure, a leaner protocol design is needed, where the inventory procedure may not always include the whole device ID.

The scenarios in particular are for the cases where a customer is inventorying groups of devices or all devices and would only like to know the number of the devices which have arrived or have left, are present or are missing under a particular location/locations.

For example, if furniture items are delivered to a factory and the customer executed the full inventory and knows that there are e.g. 100 items, it might like to know how many of the furniture items are now under the coverage of a reader, as this reader  covers  areas within a production line and in the case of more than 50 (or any other number) items are there, the production line will be initiated.

In such cases, it is not necessary for the device to provide the whole device ID and consume energy, but also for the system, it is not necessary to receive the whole device IDs. Also, it is not necessary for the reader to send msg 2 as this message would also be needed to be processed, which again consume energy on the device site. In other words, for certain scenarios, the inventory procedure may not necessarily need to be completed, as the relevant information for a customer is just the number of devices within its network. With only this information, it can enable multiple use cases to the customer already, whilst reducing the overall latency and power consumption of the devices within the network.

It is therefore suggested to agree that the inventory procedure could also be performed without receiving the whole device ID, but instead to base the procedure on the reception of random IDs (Msg1) only as seen in Figure 2.

Figure 2 – Lean protocol for inventory procedure in Ambient IoT.

This procedure would thus be based only on paging/initial message transmitted by the core network and the Msg1 transmitted by the devices. Moreover, Msg2 would only be sent in case of contention resolution

Proposal 1: It is proposed to call the A-IOT inventory without delivery of msg 3 comprising the full device ID a lean inventory procedure.

It is proposed to support A-IOT lean inventory procedure within the A-IOT study item and include it into  TP 38.769.

Proposal 2: It is proposed to liaise with SA2 and SA3 informing these groups about RAN WG2 decision and ask if this procedure would have any impact on the architecture and/or security aspects.

## Conclusions

In this contribution we propose the following:

Proposal 1: It is proposed to call the A-IOT inventory without delivery of msg 3 comprising the full device ID a lean inventory procedure.

Proposal 2: It is proposed to liaise with SA2 and SA3 informing these groups about RAN WG2 decision and ask if this procedure would have any impact on the architecture and/or security aspects.