3GPP TSG-RAN2 Meeting #127	R2-2406605

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the RAN#104 meeting, the revised Work Item for network energy saving enhancement has been approved in RP-241650 including the following objective on on-demand SIB1 operation for NES [1]:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several agreements have been achieved [2]:

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

In this contribution, we provide our further considerations on on-demand SIB1 operation.

# Discussion

## 2.1 Scenarios and use cases

RAN1 has discussed the following scenarios and achieved the following agreement [3] and RAN2 has agreed to support Case 2 as the starting point.

Agreement For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A

For case 3, RAN2 had an initial discussion [2]:

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism. - Postponed. [Fraunhofer]: Ok with the proposal. In addition, we need to consider how to reduce the signalling overhead. [Nokia, Ericsson]: Case 3 requires quite close coordination between two cells in order to guarantee the latest SIB1. [Ericsson]: Once the UE gets SIB1 from cell A and camps on NES cell, if the UE requires OD-SIB1 again for the camped NES cell, will the UE back to cell A and request it again? [Samsung]: Once camped on the NES cell, the SIB1 will be maintained by SIB change notification, so the scenario Ericsson mentioned will not happen. - Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

During previous meetings, we have discussed the applicable scenarios for on-demand SIB1 operation. We have some conclusions about Case 2 and Case 1. Case 1 is decided to be left up to RAN1 and Case 2 is already decided to be supported by RAN1 and RAN2. However, whether Case 3 should be supported has no conclusion. Since this is the last meeting before the decision to conclude the WI scope of the OD-SIB1 operation, we need to discuss Case 3.

From the energy saving perspective, the biggest energy saving gain comes from Case 3, where the SIB1 is provided by Cell A. In this scenario, the NES Cell does not need to send SIB1. When SIB1 is changed, the NES Cell can follow the legacy procedure to send a SIB change notification and send an updated SIB1 on the NES cell.

Some companies are concerned about the frequent SIB1 update when the SIB1 is provided by Cell A. In our understanding, the SIB1 update is not so often. And since the WUS config needs to be sent from the NES Cell to cell A, SIB1 can also be included in this information exchange. Furthermore, there were comments that there is a large overhead on Cell A to provide different cell’s SIB1. To solve this, the NES cell’s SIB1 can be provided on-demand in another SIB by Cell A, which is triggered only by interested UEs capable of R19 OD-SIB1, in which case the overhead issue is not a big problem.

Then the UE can camp or access the NES Cell based on the SIB1 information obtained from Cell A. When the SIB1 is changed, the NES Cell could send a SIB change notification to all the NES UEs and broadcast the updated SIB1 in a specified SI window. In such way, case 3 becomes an optimised version of Case 2. The spec impact of such an enhancement is low. A simple solution to have an option for the NW to deliver NES cell’s SIB1 in Cell A in a new container is valuable. It reduces the procedures and power consumption on the NES cell and is compatible with case 2 once the UE camps on NES cell.

Proposal 1: RAN2 to support case 3 by allowing to deliver NES cell’s SIB1 in Cell A as a container in a new SIB; how the UE requests for the new SIB is FFS.

## 2.2 How to indicate NES Cell state

Another essential issue is how to indicate the NES Cell state. In RAN1’s discussion, the identification of NES Cell was studied with the following agreement [3]:

Agreement RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options: - Option 1: By WUS configuration - Option 2: By PBCH payload of NES cell

For the case 2 where the WUS configuration provided from Cell A, the WUS configuration can naturally be the identification of the NES Cell. When Cell A has provided the WUS configuration, it should be clear for NES UEs which cell(s) is/are the NES Cell and that the NES Cell is in on-demand SIB1 operation. As for the indication from PBCH payload of the NES Cell, it is challenging to put the indication in the PBCH of the NES Cell and RAN1 is studying several methods of doing so.

Since the WUS configuration from Cell A is essential for the NES UE, it is a natural candidate to indicate the NES Cell’s status, i.e. that such cell is in on-demand SIB1 operation via the WUS configuration.

Proposal 2: The NES Cell identification is derived at least from the WUS configuration received from Cell A.

## 2.3 The OD-SIB1 request method

In the RAN2#125bis meeting, the Msg 1 based procedure was agreed. There is no conclusion on the Msg 3 based solution.

5.	Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

In the legacy on-demand other SI procedure, Msg 3 can be utilized for requesting the specific SI when the configuration of RACH for on-demand SI is not provided. Msg 3 is supported in the legacy mechanism because the PRACH resource reserved for requesting different kind of SIBs could occupy valuable resources, which would reduce the resources used for cell access. However, the discussed on-demand SIB1 mechanism is not the same as on-demand other SI. The request is only sent for requesting SIB1 transmission. Thus, the reserved resource for requesting on-demand SIB1 can be small. On the other hand, in the on-demand SIB1 case, if Cell A does not provide the WUS configuration and only indicates that the NES Cell is supporting on-demand SIB1 operation, Msg 3 based triggering method might be applicable. But the PRACH resource for accessing to the NES Cell should be included in the Cell A’s SIB. Since the PRACH configuration will be provided and there is only one possible request on that resource (on-demand SIB1) we do not see a benefit in using the Msg 3 based triggering method in case 2 (for requesting to the NES Cell). On the other hand, if case 3 is supported the UE might need to differentiate which cell’s OD-SIB1 it is interested in. In that case it should be left open whether to support msg1 and msg3 requesting methods.

Proposal 3: Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

## 2.4 The transmission behavior of OD-SIB1

After the UE has sent the WUS to trigger the OD-SIB1 and has received the RAR from the NES Cell to indicate that the NES Cell will broadcast OD-SIB1, the next issue is how to broadcast the OD-SIB1. To save the NES Cell’s energy, the periodical broadcast behaviour of OD-SIB1 needs to be avoided. The triggered OD-SIB1 broadcast should last for a specific time. In legacy OD-SI procedure, the si-WindowLength is provided in SIB1 to indicate the transmission of OD-SI. Following such mechanism, the si-Window based design can be reused for OD-SIB1. The SI window for legacy is broadcasted in SIB1. Since we don’t have SIB1 we would need to broadcast it e.g. in Cell A with WUS config or in msg2 in the NES cell, it can be left FFS for the WI phase how to realize it.

Proposal 4: Reuse the SI-window mechanism for the OD-SIB1 procedure. How to convey the SI-window if FFS.

## 2.5 Legacy UE impact

For the legacy UE impact, two different scenarios should be considered. One is cell reselection from Cell A to NES Cell and another is cell selection on the NES Cell. Some solutions were discussed during the last meeting:

Proposal 4: NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism). - Agreed. [CATT]: Ok with the proposal, however to use ssb-SubcarrierOffset needs RAN1 confirmation. Proposal 5 (modified): How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that). - Agreed. [OPPO]: Clarified that intention is to set different cell reselection priority to legacy UE and R19 NES UE. [Xiaomi]: First we need to understand whether NES cell dedicated carrier is realistic scenario or not. [Huawei]: Agree with Xiaomi. Understand NES cell is per cell turn-on and off. [OPPO]: There is no restriction for NES cell dedicated carrier. [Apple]: Even with mixed carrier case, assume the number of macro cell is small on that carrier, so still to deprioritize the carrier makes a sense. [Samsung]: For the NES cell dedicated carrier, why not the existing UE dedicated cell reselection priority work? For the mixed carrier case, we can utilize the black cell list for the legacy UE. [Vivo]: Similar discussion happened for the dedicated cell reselection priority in R18, it was not pursued. [Rakuten]: If NES cell turns on, question is whether we consider it normal cell for legacy UE or not. [Session chair]: We’ve just agreed NW need to bar the legacy UE. [OPPO]: Regardless of barring, the intention is to deprioritize the NES carrier, which doesn’t mean the legacy UE will never attempt that carrier. [Nokia]: Clarified intention is not aligned with P5 itself. Think P5 itself is ok to companies.

Cell reselection

For the former case, the intention of avoiding/deprioritizing legacy UEs from camping on NES Cells is to avoid legacy UEs initiating the procedure of searching for the NES Cell without SIB1 transmission to reduce the overhead of finding a suitable cell. As mentioned in the discussion, there are several options to avoid legacy UEs to search for the NES Cell.

Option 1: Set lower priority level to the NES Cell frequency for legacy UEs. The NES UEs would need a new priority to be allowed to camp on the NES Cell. For this option we would like to note that the OD-SIB1 feature is controlled per cell (not per carrier) therefore any modifications to cell reselection need to be per cell. Since the priority is control by per carrier, the priority based solution is not suitable.

Option 2: Use excluded cell list for the legacy UEs not to consider the NES Cell as candidate. At the same time a new excluded cell list without the NES Cells included in it should be provided to the NES UEs to allow the NES UEs to consider camping on the NES Cells. For this purpose a new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs in SIB3 can be added, in addition to the legacy IntraFreqExcludedCellList / InterFreqExcludedCellList, to give the NW a possibility to configure NES cell camping restrictions separately.

Proposal 5: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

Initial cell selection/cell barring

As a current principle if a legacy UE cannot obtain SIB1 from a given cell it considers it as barred. We can identify several legacy options to indicate to legacy UEs that this cell should not be camped on. In our view any option that is chosen should not be dependent on setting the cellBarred in MIB to barred. One option would be to use the ssb-SubcarrierOffset, if the field indicates that the cell does not provide SIB1 and there is no CORESET#0 configured, the legacy UE will treat the cell as barred. This option also means that the cell operates on NCD-SSB. Currently the cell selection and reselection rely on the CD-SSB on the sync raster. If the NES Cell does not operate on the CD-SSB, legacy UEs will not consider it as a candidate cell for cell selection and cell reselection. Relevant text from the specifications is highlighted below.

Another option is to use a new sync raster only for NES Cell operation. The legacy UEs would not be able to detect the NES Cell following the current sync raster, therefore they will not be impacted by NES Cells. The NES UE could search on a newly specified sync raster. The new sync raster information could be provided e.g. in Cell A. After camping on Cell A and obtaining the WUS configuration, the NES UE could search on the new sync raster and detect the NES Cell.

### --Cited from TS38.300 v18.2.0 ### 9.2.1 Mobility in RRC_IDLE #### 9.2.1.1 Cell Selection The principles of PLMN selection in NR are based on the 3GPP PLMN selection principles. Cell selection is required on transition from RM-DEREGISTERED to RM-REGISTERED, from CM-IDLE to CM-CONNECTED and from CM-CONNECTED to CM-IDLE and is based on the following principles: - The UE NAS layer identifies a selected PLMN and equivalent PLMNs; - Cell selection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4): >>Omit some unrelated part<< #### 9.2.1.2 Cell Reselection A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following: - Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

Cited from TS 38.331 v18.2.0 ssb-SubcarrierOffset Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See TS 38.211 [16], clause 7.4.3.1). For operation with shared spectrum channel access in FR1 (see 37.213 [48]), this field corresponds to kSSB, and kSSB is obtained from (see TS 38.211 [16], clause 7.4.3.1); the LSB of this field is used also for deriving the QCL relation between SS/PBCH blocks as specified in TS 38.213 [13], clause 4.1. The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [13]. This field may indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB (see TS 38.213 [13], clause 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see TS 38.213 [13], clause 13).

Cited from TS 38.331 v18.2.0 #### 5.2.2.5 Essential system information missing The UE shall: 1> if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running: 2> if the UE is unable to acquire the MIB: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs or intraFreqReselection2RxXR for 2Rx XR UEs, is set to allowed; 2> else if the UE is unable to acquire the SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> if the UE is a RedCap UE: 4> perform barring as if intraFreqReselectionRedCap is set to allowed; 3> else if the UE is an eRedCap UE: 4> perform barring as if intraFreqReselection-eRedCap is set to allowed; 3> else if the UE is a 2Rx XR UE: 4> perform barring as if intraFreqReselection2RxXR is set to allowed; 3> else: 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20]. NOTE 1: The SIB19 is essential for NTN access. If UE is unable to acquire the SIB19 for NTN access, the action is up to UE implementation (e.g., cell re-selection to other cells). NOTE 2: The SIB22 is essential for ATG access. If UE is unable to acquire the SIB22 for ATG access, the action is up to UE implementation (e.g., cell re-selection to other cells).

Observation 1: Legacy UEs can be barred through the legacy cell selection and reselection criteria. Detailed mechanism can be left to the WI phase.

# Conclusion

In this contribution, we discussed the on-demand SIB1 operation, and made the following observations and proposals:

Proposal 1: RAN2 to support case 3 by allowing to deliver NES cell’s SIB1 in Cell A as a container in a new SIB; how the UE requests for the new SIB is FFS.

Proposal 2: The NES Cell identification is derived at least from the WUS configuration received from Cell A.

Proposal 3: Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Proposal 4: Reuse the SI-window mechanism for the OD-SIB1 procedure. How to convey the SI-window if FFS.

Proposal 5: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

Observation 1: Legacy UEs can be barred through the legacy cell selection and reselection criteria. Detailed mechanism can be left to the WI phase.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R2-2405701 Report from session on V2X/SL, R19 NES and MOB, Samsung

- Chair notes RAN1#116bis eom2.doc